3GPP TSG-SA WG2 Meeting #168	S2-2503788

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	KI#2: New Solution on SM policy control based on EIF notification

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on SM policy control that the PCF may utilize the notification including Energy Consumption information from the EIF and update the SM policy for a UE/PDU Session/QoS flow based on the received information.

# 1.	Discussion

One of the key issues for Rel-20 EnergySys Phase2 Study includes the discussion on solutions that can support policy update based on the notification information from the EIF related to the network energy consumption for a UE/PDU Session per UE/QoS flow per UE. A new solution is proposed to support SM policy control by the PCF based on the energy consumption information received from the EIF.

# 2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change (all new text) * * * *

## 6.X	Solution #X: Session control based on energy information from EIF

### 6.X.1	Key Issue mapping

This solution is to address session control mechanism that the PCF updates SM policy for a UE/PDU Session/QoS Flow based on energy related information (per UE, PDU Session, QoS Flow) received from the EIF.

### 6.X.2	Functional Description

The solution assumes the energy saving indication for per-UE/Network Slice/DNN is stored in the subscription data of a UE. The PCF and/or the SMF can retrieve the information during the SM Policy Association or PDU session establishment. In case of the dynamic policy control, the PCF retrieves from the UDR and stores the energy saving indication information for UE/Network Slice/DNN and provides a policy to control the energy use by a UE during the policy association establishment or modification with the SMF. The PCF can subscribe to the EIF and receive the energy consumption information in a required granularity to control the energy consumed in the network to serve the UE/PDU Session/QoS flow. The PCF may decide, based on the energy related information received from the EIF, operator policy, energy saving indication, and local configuration, if network energy saving is required. The PCF may provide an updated SM policy for UE/PDU Session/QoS flow to the SMF to apply a changed policy for energy saving.

### 6.X.3	Procedures

Figure 6.x.3-1 illustrates session management procedure considering energy related information from the EIF.

Figure 6.36.3-1: Session management procedure considering energy related information

1.	UE sends a PDU Session Establishment Request message including S-NSSAI and DNN to set up a communication path for an application(s).

2.	The AMF sends an Nsmf_PDUSession_CreateSMContext Request message to the SMF to request a PDU session for the UE.

3.	The SMF responds to the AMF by sending an Nsmf_PDUSession_CreateSMContext Response message.

4.	The SMF performs Subscription Retrieval procedure with UDM and receives subscription data for the UE. The subscription data may include energy saving indication information for the UE (e.g. energy control per UE/Network Slice/PDU Session). The SMF may use this information together with operator policy and local configurations to decide control parameters for the PDU Session, e. g. inactivity timer, buffering size, etc.

5.	The SMF performs Policy Association Establishment procedure with PCF if dynamic policy is supported. The PCF may provide the SM policy for the PDU session of the UE (e.g. energy control per UE/PDU Session/QoS flow) based on the energy saving indication and energy related information from the EIF or OAM. The SMF may use the information to set the control parameters for the PDU Session, e. g. inactivity timer, buffering size, etc.

6.	The SMF sends an N4 Session Establishment Request message to the UPF to request a user plane resource setup at the UPF for the PDU session of the UE. The message includes the control parameters for the PDU session that was decided by the SMF based on the energy related policy information in the SM policy.

7.	The UPF responds to the SMF by sending an N4 Session Establishment Response message.

8.	The remaining steps for PDU Session Establishment procedure as defined in steps 11 to 21 in Figure 4.3.2.2.1-1 of TS 23.502 [3] proceeds.

9.  The PCF may subscribe to the EIF for notification of the energy-related information related to the UE/PDU Session/QoS Flow.

10. The EIF may send the updated energy related information as requested by the PCF in the step 9.

11.	The PCF may decide to change the SM policy for energy saving, e.g., to trigger the aggressive saving mode when the energy consumption in the network increases beyond the defined threshold happens, based on the energy related information included in the notification from the EIF, operator policy, local configuration, etc.

12.	The PCF performs SM Policy Modification procedure with SMF and provides the updated policy for the PDU session of the UE including energy related policy information.

13.	The SMF sends an N4 Session Modification Request to provide updated control parameters (e.g., inactivity timer, buffering size, etc. to control initiation of subsequent paging and data transmission procedures) for the PDU session of the UE, based on the energy policy information in the SM policy received from the PCF. Or, the SMF can send an N4 Session Release Request message to initiate the PDU session deactivation procedure based on the energy policy information in the updated SM policy received from the PCF.

14.	The remaining steps for PDU Session Modification or CN-initiated deactivation procedure proceeds.

### 6.X.4	Impacts on existing services, entities and interfaces

SMF:

-	Sets session control parameters for a PDU session(s) of a UE based on the subscription data from the UDM or SM policy from the PCF that can be updated based on energy related information received from the EIF.

UDM:

-	Stores energy saving indication information for UE/Network Slice/DNN (PDU Session) in the subscription data.

PCF:

-	Stores energy saving indication information for UE/Network Slice/DNN (PDU Session) and provides to the SMF with the policy to control the energy use by a UE/PDU Session/QoS Flow.

* * * End of Change * * * *