3GPP SA WG2 Meeting #168	S2-2504241

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-2503958)

Source: 	LG Electronics

Title: 	KI#1, New Sol#X: Generalized UE data collection over user plane

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes to add a new solution for AIML UE data collection over UP in TR 23.700-04.

1	Discussion

From FS_AIML_CN_Ph2 SID (SP-250399)

-	WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-04.

* * * * Start of 1st Change * * * *

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[x]	3GPP TS 23.273: " 5G System (5GS) Location Services (LCS); Stage 2".

[y]	3GPP TS 24.572: " 5G System (5GS); User plane Location Services (LCS) protocols and procedures; Stage 3".

* * * * Start of 2nd Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #X | O |  |

* * * * Start of 3rd Change * * * *

!! All New Texts !!

6.X	Solution #X: Generalized UE data collection over user plane

6.X.1	High level principles

Editor's Note:	This clause will describe the high-level principles of the proposed solution.

6.X.2	Description

This solution addresses the key issue#1

.

It proposes a solution for Generalized UE data collection over user plane by applying the existing user plane-based LCS, in similar to a user plane connection between UE and LMF specified in clause 6.18 of TS 23.273 [x].

It assumes the followings

-	Data collection function (DCF) in core network

Decides to use user plane for UE data collection based on UE subscription data, UE capability (if needed) to perform data collection and local configuration;

Provides the UE the information to establish a secure connection (e.g. DCF address);

Sends the event reporting (if the event data collection is allowed such as Deferred MT-LR of TS 23.273 [x]) related request to UE;

Expose the collected data to UE side data collection server/OTT server, etc.

-	UE subscription data in UDM

DCF ID;

Data collection over user plane between UE and DCF is allowed/not allowed, etc.

- 	URSP and/or UE Local configuration information includes

DCF address;

PDU session information for UE data collection (e.g. associated DNN and S-NSSAI), etc.

6.X.3	Procedures

Figure 6.X.2-1: High-level procedure for UE data collection over user plane

Editor's note:	In this solution, which steps are related to UE Data Collection and which steps are related to UE Data Transfer is FFS.

Editor’s Note: 	How to support MNO’s controllability and visibility in solution is FFS.

- UE side data collection server/OTT server sends UE data collection request to 5GC. In case the UE side da collection server/OTT server is located outside MNO, the NEF authorizes it to accept the request.

- The DCF determines to use user plane for UE data collection based on UE subscription data (e.g. Data collection over user plane between UE and DCF is allowed/not allowed), etc. It may be triggered by AMF request.

Editor's note:	It is FFS whether DCF can be newly defined or Location Management Function (LMF) specified in TS 23.273 [x] can be reused/collocated.

- In similar to user plane connection establishment procedure specified in clause 6.2.1 of TS 24.572 [y], the DCF invokes an Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request transfer of the USER PLANE CONNECTION ESTABLISHMENT COMMAND message. The AMF fowards it to the UE via DL NAS TRANSPORT message.

- The UE establishes a PDU session if there is no established PDU session for UE to DCF PDU connectivity service, and establishes a TLS connection with the DCF. Finally, the secured user plane connection is established between UE and DCF.

NOTE:	The URSP rules or UE Local configuration is used to enable to routing of collected data via a specific PDU session (e.g. associated DNN and S-NSSAI).

Editor’s note:	How to separate “UE data collection” traffic over user plane from end user generated user plane traffic is FFS

- The UE sends USER PLANE CONNECTION COMPLETE message via UL NAS TRANSPORT message the to the AMF, and the AMF forwards it to the DCF by invoking Namf_Communication_N1MessageNotify service operation.

- The UE performs UE data collection. The NG-RAN may provide the configuration for the UE data measurement.

Editor’s note:	How the UE/NG-RAN knows the data to be collected is FFS.

- The collected data and/or event reports (if the event data collection is allowed such as Deferred MT-LR of TS 23.273 [x]) are notified to UE side data collection server/OTT server via the DCF based on the request of Step 1.

6.X.4	Impacts on services, entities and interfaces

DCF

-	Decides to use user plane for UE data collection, and provides the UE the information to establish a secure connection (e.g. DCF address, etc.).

-	Exposes collected data and/or event report from UE to UE side data collection server/OTT server.

UE

-	Establishes a secured user plane for UE data collection with DCF.

-	Performs UE data collection and provides it to the DCF.

* * * * End of Changes * * * *