SA WG2 Meeting ##168	S2-2502804

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-CT WG1 Meeting #153	C1-250819

# Athens, Greece, 17-21 February 2025

# Title:	Reply LS on request for IMS Data Channel related clarifications

# Response to:	LS on request for IMS Data Channel related clarifications (C1-246142/S3i240707)

# Release:	Rel-18

# Work Item:	NG_RTC

Source:	CT1

To:	SA3-LI

Cc:	SA2, SA4, CT4

Contact Person:

Name:	Xu Chen

Tel. Number:

E-mail Address:	chenxu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT1 thanks SA3-LI for the LS on request for IMS Data Channel related clarifications.

CT1 would like to reply to the questions in the LS as below:

4: 	In the SessionEvenNotifictions, how are the Calling Identity and Called Identity populated by the IMS AS (i.e. which SIP headers from the initial INVITE, re-INVITE are used)?

CT1 answers: From CT1 view, the calling identity can be populated from P-Asserted-Identity header field and the called identity can be populated from Request URI of the initial INVITE request.

5: 	If call forwarding occurs at the terminating side (clause 10.7, TS 24.186), the two users present on the established IMS session (i.e. after the successful redirection) are:

  - Original calling party.

  - Redirected-to party.

5-1: 	Assuming Calling Identity is the original calling party, what would the Called Identity be that IMS AS sends in the session Event Notification to the DCSF, i.e.redirecting party identity or the redirected-to party identity?

CT1 answers:

For the IMS AS of the user originating the call(i.e. Original calling party), the Called Identity in the session Event Notification upon receiving the INVITE request would be the redirecting party identity.

For the IMS AS of the diverting user(i.e. redirecting party), not all the Communication Forwarding cases trigger the session Event Notification, in the cases the session Event Notification upon receiving the INVITE request, the Called Identity would be the redirecting party identity. After communication forwarding, the IMS AS will not send the session Event Notification upon receiving the re-INVITE request.

For the IMS AS of the diverted user (i.e. redirected to party), the Called Identity in the session Event Notification upon receiving the INVITE request would be the redirected-to party identity .

5-3:	If it is the later, does the IMS-AS in the originating network know about the redirected-to party identity?

CT1 answers: The IMS-AS in the originating network knows about the redirected-to party identity via the presence of the History-Info header field in the 181 and 200 OK responses to the initial INVITE request, i.e. from the last hi-targeted-to-uri entry of History-Info header field.

2. Actions:

To SA3-LI

ACTION: 	CT1 kindly asks SA3-LI to take the above answers into account.

3. Date of Next CT1 Meetings:

CT1#154                           7th April- 11th April 2025...............................China

CT1#155                           19th May- 23rd  May 2025.............................Bratislava, Slovakia

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India