3GPP TSG-WG SA2 Meeting # 168	S2-2502981

# Goteborg, SE, 7 - 11 April 2025												(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6171 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PWS handling with Additional ULI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PWS handling linkage with Additional ULI is missing in this spec. As indicated by LS S2-2502805/C1-250820 from CT1, PWS handling with MWAB and MBSR shall be clarified in CT1 spec TS 23.041. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the usage of Additional ULI for PWS in clause 5.35A.6 for MBSR with reference to TS 23.041. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing linkage between PWS handling and Additional ULI in this spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.6, 5.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.35A.6	Providing cell ID/TAC of MBSR for services

The TAC and cell ID broadcasted by the MBSR cell(s) are configured and reconfigured e.g. upon mobility as specified in TS 38.401 [42].

After the MBSR is authorized as defined in 5.35A.4, when a UE is served by a cell of this MBSR, the IAB-donor-CU may provide 'Additional ULI' in addition to User Location Information, in N2 messages. The 'Additional ULI' is the ULI of the IAB-UE. The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions).

When the AMF provides user location information to the LMF as specified in clause 5.9 of TS 23.273 [87] for a UE connected via MBSR, the AMF may also send the Additional ULI received via N2 messages.

The AMF and IAB-donor-CU use Additional ULI to support the PWS function as described in clause 5.16.1.

# * * * * Next change * * * *

### 5.16.1	Public Warning System

The functional description for supporting Public Warning System (including the deployment scenarios with MBSR, e.g. AMF delivers the warning message to IAB-donor gNB if the TAI of the Additional ULI is included in the “List of NG-RAN TAIs” and  IAB-donor-CU distributes the warning message if the TAI/CGI of the Additional ULI is included in the “Warning Area List of NG-RAN”) for 5G System can be found in TS 23.041 [46].

# * * * * End of changes * * * *