SA WG2 Meeting #168	S2-2502993

07 - 11 April, 2025, Goteborg

Source:	SK Telecom, China Mobile, China Telecom, Deutsche Telekom, KPN N.V., LG Uplus, Verizon

Title:	WT#2 use case: Analytics of detection and control of UPF traffic bursts to support efficient performance of User Plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a use case to cover Work Task#2 of the FS_AIML_CN_Ph2 SID.

1	Discussion

This paper discusses a use case for FS_AIML_CN_Ph2 (SP-250413) Work Task #2 (WT#2), as described below.

Whether and how to enhance the 5GC analytics to support efficient performance user plane. Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework. Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall have neither RAN or UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

A related use case for the WT#2 is provided in accordance with the SA2 guidance (SP-250418).

2	Proposal

It is proposed to include the following changes in TR 23.700-04 V.0.0.

* * * * Start of Changes – All New Text * * * *

5.X	Use cases #x: Analytics of detection and control of UPF traffic bursts to support efficient performance of User Plane

Depending on the time of day, configurations, UE applications, and UPF deployments, certain data burst packets from UEs/Sessions/Applications may cause unexpected UPF overloads and irregular behaviour in network functions, which can result in overall network performance degradation. In such cases, NWDAF for example can provide a monitoring and analysis of to solve UPF related performance issues. Based on these analytics, consumer NFs can take corrective actions, such as reconfiguring UPF parameters related to queues, buffering, jitter, and latency for those UEs/Sessions/Applications to ensure optimal performance.

* * * * End of Changes * * * *