3GPP TSG-WG SA2 Meeting #168	S2-2503004

7th – 11th April 2025, Goteborg, Sweden	(revision of 25XXXXX)

Source:	Lenovo, China Telecom

Title:	Update for Command Procedure: Command failure handling

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions of how to handle the command failure in the clause 6.2.3 of the TS 23.369 for Ambient IoT.

- Introduction

This paper is intended to address the scenario when the command failure happens, e.g., the command (read, wirte and disable) towards a specific device cannot be performed successfully and no command responses have been received from the AIoT device. The command failure situation can happen due to various reasons, such as radio link failure, or the poor radio link conditions between the BS reader and AIoT device, or device moving to a different reader and cannot be reached via the current reader, or device lack of energy and cannot respond to the BS reader and AIOTF.

From the AIOTF point of view, when realizing that the command towards a device has failed, it can take several actions to address the command failure, instead of directly sending the command failure result to the AF.

Option 1: AIOTF sends the command request to other readers that have reported the device ID to the AIOTF before during the inventory procedure, e.g, other last known readers for the same device. It is possible that the inventory result of the device (device ID) has been reported to the AIOTF via multiple readers. Therefore, AIOTF can send the command request to other serving readers towards the failed AIoT device(s).

Option 2: AIOTF buffers the command request, and retries sending the command via the same reader later, which is based on local configuration.

Option 3: Given that the command failure may be due to the poor radio connection between the current reader and the device, or device out of the reader coverage area, it is reasonable to find the new serving reader for the device that has a better radio connection with the device. Therefore, when command failure happens, the AIOTF can then start a new round of inventory procedure for the AIoT device (that the command was supposed to send), e.g., to all the readers within its supported area, and when the device ID is reported after the inventory procedure, AIOTF can send the command request to the device via the latest corresponding reader. The inventory request can be based on the local configuration, or based on the previously received service request from AF.

Option 4: Inform the AF (via NEF) that the command cannot be performed successfully, associated with the correlation identifier and device ID. This is a default way of handling the command failure.

Based on the discussion above, we propose to add the descriptive texts into the TS command procedure clause.

2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

13.	The AIoT RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the AIoT RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

16. After receiving the command response message from the AIoT RAN either directly or indirectly, in case of command delivery failure, the AIOTF may conduct any of the following procedure:

a)	If other readers have reported the device ID to the AIOTF during the inventory procedure, the AIOTF may send the command request message (correlation identifier, NAS Command Request)) to these readers.

b)	Based on local configuration, the AIOTF may determine to perform the inventory procedure for the AIoT device again in the target area as described in step 7 to step 11 of procedure for Inventory specified in clause 6.2.2. When obtaining the inventory result from the corresponding reader (s), the AIOTF can then send the command request to the AIoT device via that reader (s).

c)	Buffers the command and retry sending the command request to the AIoT device either to the same reader or perform the Inventory procedure before sending the command request. This is based on local configuration in the AIOTF.

d)	Informs the AF (via NEF) that the command cannot be performed successfully as in step 17 with an appropriate failure cause.

17.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

18.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

# * * * * End of changes * * * *