3GPP SA WG2 Meeting #168	S2-2503053

07 - 11 April, 2025, Gothenburg, Sweden

# Title:	[DRAFT] Reply LS on support for multiple non-3GPP device identifiers

# Response to:	LS on support for multiple non-3GPP device identifiers (C1-250822/S2-2502806)

# Release:	Rel-19

# Work Item:	UIA_ARC

Source:	Samsung (TO BE SA2)

To:	CT1

Cc:	CT3

Contact Person:

Name:	Naman Gupta

E-mail Address:	nmn.gupta@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 would like to thank CT1 for the LS on support for multiple non-3GPP device identifiers.

CT1 asked:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

SA2 Answer: As per current agreement in SA2, only a single non-3gpp device identifier case for a single PDU SESSION MODIFICATION REQUEST is supported. In case there is any agreement in SA2 regarding support of multiple non-3GPP device identifiers in a single request, SA2 would update the CT1 accordingly.

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

SA2 Answer:  In case UE sent a PDU SESSION MODIFICATION REQUEST for QoS differentiation of a Non-3GPP Device Identifier but UE’s request could not be fulfilled by the network since Non-3GPP Device Identifier is not present, then SMF rejects the whole PDU SESSION MODIFICATION REQUEST by sending PDU SESSION MODIFICATION REJECT message as UE’s particular request for PDU Session modification was not fulfilled and the procedure needs to be halted on the UE side once it receives the reject message.

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

SA2 Answer: In case removal of multiple Non-3GPP device identifiers is notified to the PCF, PCF will modify the PCC rules taking into account all the removed device identifiers and provide the updated PCC rules to SMF (as described in clause 4.3.3.2 of TS 23.502), and SMF in turn can notify that (e.g. with updated QoS rules) to UE in a single PDU SESSION MODIFICATION COMMAND message.

2. Actions:

To CT1 group.

ACTION: 	SA2 requests CT1 to take above into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#169      	May 19 – 23, 2025                                     Fukuoka, Japan

TSG-SA2 Meeting#170                      August 25 -29, 2025                                  Gothenburg, Sweden