SA WG2 Meeting #168	S2-2503189

7 – 11 April 2025, Goteborg, Sweden

Title:	KI#1 solution using the generic architecture for collecting and exposing the collected UE data

Source:	Ericsson

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#1 for FS_AIML_CN_Ph2.

# 1.		Introduction

This solution describes how to address KI#1: Data collection over user plane for UE side model training, reusing the generic data collection procedure from the UE application using an intermediate Application Function that is described in TS 26.531 and TS 23.288 clause 6.2.8.

In order to use the Data collection architecture, new EventIDs to allow collection of UE data that is needed for training ML Models at the UE training center are defined.

Based on the RAN requirements and the WT#1 description the following solution is proposed:

- The intermediate Application Function part of the generic architecture for data collection is only located in the MNO domain.

  - This is to comply with requirements for “the MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer”,

- Given that DCAF in the MNO domain initiates and terminates the UE data collection, the MNO has full visibility for standardized data.

- The information security model defined in TS 26.531, under SA4 scope, is understood to cover the following, may need validation with SA4:

  - The data collected is secured and data integrity and confidentiality for that data is ensured.

The solution proposed in this contribution is on high level, while additional aspects need to be covered, as such those are documented in the clause “Aspects for further consideration”.

# 2. Text Proposal

It is proposed to agree the following solution to be part of TR 23.700-04.

************ 1st change (all text is new) **************

## 6.x	Solution #N: User Plane solution for Data collection for UE-side model training

### 6.x.1	Description

This solution is addressing KI#X: Data collection over user plane for UE side model training.

According to RAN LS, RP-243316. the following is requested: “TSG RAN kindly asks TSG SA to start studying the transfer of data over UP for solution 2 in WG SA2”.

Then RAN selected Solution 2 and the requirements to be fulfilled for that solution are described in RAN LS RP-242389 as follows:

Solution 2: UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. The requirements are:

- The data collected is secured and data integrity and confidentiality for that data is ensured.

- User data privacy, anonymity and user consent is respected.

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

- The design is futureproof and extendable.

Note that there are additional requirements in R2-2411152 that states:

- ”Data collection initiation and configuration for data collection is under network control”.

  - FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server)),

- “SA2 can assume that the gNB is involved in providing radio measurement configuration (if needed) for beam management use case and LMF is involved in providing PRS measurement configuration (if needed).

  - However, RAN2 has not agreed that the gNB/LMF is in charge of “initiating, terminating and fully managing data transfer”

- “Standardized data refers to data whose format/content is explicitly defined in 3GPP specifications, allowing the network to understand its content and meaning.”

### 6.x.2	High-level solution principles

This solution reuses the generic architecture for collecting and exposing the collected UE data, via an intermediate Application Function that provides events to the event consumer using the event exposure service, defined in TS 23.288 [y] 6.2.8 and in SA4 TS 26.531 with the following considerations:

- The intermediate Application Function, i.e. DCAF:

  - is always located in the MNO to fulfil the requirements on visibility and controllability of the data collection procedure.

  - Receives configuration data from the UE training center per each of the EventIDs.

  - Receives request to collect input data from the UE for model training, via existing Naf_EventExposure service.

  - Decides whether and when to initiate and terminate the data collection from the UE, e.g. the DCAF may use e.g. PDTQ to determine when to start or whether to terminate data collection or the RAN, or take decisions when requests from multiple UE training centres are received.

- The UE training center:

  - may be located in the trusted MNO domain or in an untrusted MNO domain.

  - provides configuration information to DCAF.

  - Triggers the request for data collection towards the DCAF.

  - Checks user consent before requesting data from DCAF.

- The UE:

  - request configuration information from DCAF.

  - performs data collection and provides it to the DCAF as indicated in the configuration information.

- How the data collected is secured and how data integrity and confidentiality for that data is ensured is described in TS 26.531 [x], clause 4.5 Information security model, whether additional requirements are foreseen will be discussed by SA3.

- It is assumed that the UE receives configuration information from DCAF, then the UE requests resources and configuration information from gNB, as such no requirements exists in SA2, it is rather between RAN and the UE when it comes to “SA2 can assume that the gNB is involved in providing radio measurement configuration (if needed) for beam management use case and LMF is involved in providing PRS measurement configuration (if needed).

The figure below schematically shows the solution for data collection from the UE via user plane, reusing the reference architecture defined in TS 26.531 [x] clause 4.2. Note that the interface between the UE Application and the Direct Data collection is internal to the UE, as such this solution considers the UE only with no distinction between both. Interactions at R8 reference point are out the scope of 3GPP, according to 26.531 clause 4.3.

Figure 6.x.2_1. Data Collection from the UE Applications and transfer to the UE training centre for Model training

### 6.x.3	Procedure for collecting UE data and reporting to the UE training center

This procedure follows TS 26.531 [x] clause 5 and 23.288 [y] clause 6.2.8 for data collection from the UE and reporting the data collected to the UE training center. Figure 6.x.3_1 shows how the procedure is applied for collecting input data for training a ML Model.

Figure 6.x.3_1: UE training center triggers data collection from the UE

- The UE training center discovers and selects the DCAF that provides data collection (based on the DCAF profiles registered in NRF). The DCAF profile is extended to indicate the new EventIDs that DCAF supports include CSI compression data, CSI prediction data, Beam management, or UE Positioning or combinations of those.

- The UE training center provisioning per Event ID information to be configured to the UE, possibility, to enable the UE to report the required input data to the UE training center.

- The UE training center subscribes to the DCAF for UE data collection, possibly via NEF, using Naf_EventExposure Subscribe including the Target for Event Reporting (i.e. GPSI or SUPI, Internal or External Group Identifier or any UE), both Event ID(s), Event Filters such as UE location (to enable to collect data in certain areas only) listed in clause 5.2.19.2.1 in TS 23.502 [3].

- At the time that the UE needs to train a ML Model, it establishes a PDU Session according to the provisioned URSP Rules that enables the establishment of a PDU Session and a user plane connection to retrieve the provisioning information and report collected measurements to the DCAF.  The DCAF provides the provisioning information to the UE if both steps 2 and 3 happened and DCAF knows what provisioning information to provide and the Events that the UE training center subscribed to.

- The DCAF decides whether to initiate the data transfer, as such if the data collection and reporting should not be started at the time the UE requested, the DCAF may either a) provide an response to the provisioning request indicating that the configuration and reporting instructions are not available yet or b) the DCAF can include a timer in the provisioning information to indicate to the UE to report measurements when the timer expires.

- When the UE receives the provisioning information from DCAF and if the timer is included in the provisioning information the time expires, the UE starts data collection and reporting to DCAF.

NOTE: 	The UE requests the gNB for radio resources, then gNB provides reference signals and necessary configuration to the UE for data collection. When the UE gets radio resources and configuration information then the UE can start measurement. The UE reports to DCAF according to the instruction from DCAF.

6.	The DCAF may perform aggregation of the UE data depending on the Target for Event reporting, i.e. when the Target for Event Reporting is an Internal-Group-ID provided in step 2.

7.	The UE training center trains the ML Model with the input data provided by DCAF.

### 6.x.4	Aspects for further consideration

- What information is sent in the provisioning request to the UE.

- RAN requirements on User data privacy, anonymity and user consent is respected, whether SA4 TSs can cover this requirement or part of it.

- Relation with the LCS privacy profile when requesting positioning data.

- Support from the UE or from the RAN for collecting training data.

### 6.x.5	Impacts on services, entities and interfaces

DCAF:

- Extensions to the AF exposure service with new EventIDs and configuration parameters from the UE training center, including registration in NRF, and subscription and notify service operations.

- Determines when to start and terminate the data collection, notifies the UE when the data collection and reporting to the UE training center.

- Determines what information to configure to the UE based on the provisioning request from the UE training center.

UE:

- Receive provisioning information for the new EventIDs.

- May receive a timer to indicate that reporting the data collected should start when the timer expires.

- May receive an indication that the provisioning of configuration an reporting information is not available yet, as such the UE will receive it when the DCAF decides that the collecting of training data should start.

UE training center:

- Provide provisioning information for the new EventIDs

- Subscribe to the reporting of measurements for training a model.

************ End of changes **************