SA WG2 Meeting #168	S2-2503254

7 - 11 April, 2025, Gothenburg, Sweden

Source: 	Google

Title: 	WT#1: KI#X for UE policy of UE data collection

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes the KI description for WT#1 on UE policy of UE data collection for UE-side model training.

Proposal :

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

## 5.x	Key Issue #x: UE policy for UE data collection over UP

### 5.x.1	Description

This key issue aims to study whether and how to define the policies by the HPLMN to support UE data collection for UE-side model training, e.g. for the AI/ML enabled features of AI/ML NR air interface operation, the following aspects are to be studied:

-	Whether and what information in policies need to be provided by the HPLMN for the UE to determine whether and how to establish a PDU Session or use an existing PDU Session for UP data collection of AI/ML enabled features;

-	Whether and what information in policies need to be provided by the HPLMN for the UE to determine whether and how to setup a UP connection for a UP data collection of AI/ML enabled feature over an established PDU Session for UP data collection;

-	Whether and how the policy for UE data collection of AI/ML enabled features have impacts on existing UE policies.

NOTE 1: The UE privacy and user consent aspects will be in coordination with SA WG3.

* * * End of Change * * * *