3GPP TSG-SA2 Meeting #168	S2-2503410

07 - 11 April, 2025, Goteborg                                                           (revision of S2-2501699)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1386 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements on the input and output of QoS and Policy Assistance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Some applications are sensitive to bandwidth and packet loss rate, such as cloud gaming, where RAN status may also affect the QoE value even under the same RAN signal power and quality. For these applications, the RAN status values need to be analyzed by the NWDAF, and the perfomance data collected from the OAM reused in this paper enables the NWDAF to more accurately predict the QoE. - In the ouput predictions of QoS and Policy Assistance Analytics, the "Applicable duration of QoS and Policy Assistance information" is redundant since the "Validity period" was already defined to indicate the applicable time duration in a time slot for the assistance information. the "Applicable duration of QoS and Policy Assistance information" is proposed to be removed from Table 6.23.3-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RAN performance data collected from the OAM defined in clause 6.6.2 in TS 23.288 is added as input. - Delete "Applicable duration of QoS and Policy Assistance information" in the output table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete input data may lead to unreliable QoE prediction. Duplicated information may lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.23.2, 6.23.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.23.2	Input Data

The NWDAF supporting QoS and policy assistance analytics shall be able to collect information from AF, OAM and 5GC NFs.

The following specified input data might be reused by QoS and policy assistance analytics:

-	The input data defined for Observed Service Experience related network data analytics as defined in clause 6.4.2, e.g. the information of the application, Service Experience and QoE metrics.

-	The input data collected from the OAM for Network Performance analytics as defined in clause 6.6.2, e.g. the RAN status, load and performance per Cell Id for the traffic type of interest and in the Area of Interest.

In addition, more information collected by the NWDAF from relevant 5GC NFs (i.e. UPF, SMF, AMF, PCF) is defined in Table 6.23.2-1.

NOTE:	The NWDAF may also collect the Observed Service Experience Analytics from other NWDAFs, based on NWDAF implementation.

Table 6.23.2-1: Input data from 5GC NF related to QoS and policy assistance information

| Information | Source | Description |
| --- | --- | --- |
| QoS binding events (1..max) | SMF | SMF provides such event when specific QoS parameters are applied to some QoS flows. |
| > Used QoS parameter set(s) | SMF | The QoS parameter set(s) that have been already applied by SMF. As detailed in clause 6.23.1, the QoS parameter set includes QoS parameters and QoS characteristics attributes (as defined in TS 23.501 [2]). |
| > Used QoS profile | SMF | The QoS profile associated to the used QoS parameter set(s). |
| > Traffic descriptor | SMF | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set |
| ..> S-NSSAI | SMF | Slice used to transport the QoS flow |
| > DNN | SMF | DNN associated with the QoS flows. |
| > Application ID | SMF | Identifies the application service provider and application for the QoS Flow(s) |
| > DNAI | SMF | Identifies the access to the DN to which the PDU Session connects |
| > PDU Session Type | SMF | Type of PDU Session |
| > SSC Mode | SMF | SSC Mode of the PDU Session(s) |
| UE identifier | SMF, AMF | The identifier of UE, e.g. SUPI, UE IP address, etc. |
| UE Location | AMF | The UE location information, e.g. cell ID or TAI. |
| Time stamp | SMF, AMF | The time stamp associated to the collected data. |

# * * * * 2nd change * * * *

6.23.3	Output Analytics

The NWDAF shall be able to provide prediction on QoS and Policy Assistance to consumer NFs, i.e. PCF, as defined in Table 6.23.3-1.

Table 6.23.3-1: QoS and Policy Assistance predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > QoS and Policy Assistance information (1…max) (NOTE 1) | List of QoS and Policy Assistance information. Max. is the number of the candidate QoS parameter set(s), if applicable. |
| >> QoS parameter set identifier | Identifies the QoS set for which the entry applies. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> Application ID(s) | Identifies the application(s) that associated to the QoE or QoS parameter set. |
| >> Predicted QoE (NOTE 7) | The predicted QoE or the service experience (e.g. QoE, MOS) of the corresponding QoS parameter set, e.g. average, variance, maximum, minimum of QoE. |
| >> QoS parameter values (NOTE 5) | The parameters and corresponding values for the QoS parameter set associated to the predicted QoE, |
| >>> 5QI | The reference to 5G QoS characteristics and QoS parameters. |
| >>> ARP | The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability, as defined in TS 23.501 [2]. |
| >>> Resource type | The resource type of the corresponding QoS flow, e.g. GBR QoS flow, non-GBR QoS flow, delay-critical QoS flow. |
| >>> Packet Delay Budget | Packet Delay Budget (PDB) indicates the upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [2]. |
| >>> Packet Error Rate | Packet Error Rate (PER) defines an upper bound for a rate of non-congestion related packet losses, as defined in TS 23.501 [2]. |
| >>> Flow Bit Rates (NOTE 2) | The flow bit rates only applies to GBR QoS Flow. |
| >>>> GFBR | Guaranteed Flow Bit Rate (GFBR) for UL and/or DL. |
| >>>> MFBR | Maximum Flow Bit Rate (MFBR) for UL and/or DL. |
| >>> Packet Loss Rate (NOTE 2) | The Maximum Packet Loss Rate (UL, DL) indicates the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. This is provided to the QoS Flow if it is compliant to the GFBR. |
| >>> Averaging Window (NOTE 2) | The Averaging window is applied when the resource type is GBR QoS. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE), as defined in TS 23.501 [2]. |
| >>> Maximum Data Burst Volume (NOTE 3) | The Maximum Data Burst Volume (MDBV) applies to GBR QoS Flow with Delay-critical resource type. The MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| >> Validity period | The validity period within the time slot for the analytics on service experience associated to QoS in clause 6.1.3. |
| >> Spatial validity | Area where the analytics on service experience associated to QoS applies. |
| >> Traffic descriptor | One of Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set. |
| >> Usage Duration information (NOTE 6) | Maximum/Minimum/Average usage duration of QoS Flows associated to Candidate QoS parameter set. |
| >> Number of usage (NOTE 6) | The number of times that the QoS Flows associated to Candidate QoS parameter set to be used. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Reporting Thresholds". NOTE 2: The output analytics only applies to GBR QoS Flow. NOTE 3: The output analytics only applies to GBR QoS Flow with Delay-critical resource type. NOTE 4: Only the parameters that have more than one candidate values provided by the consumer shall be included in the NWDAF output and only the values provided by the consumer are allowed. NOTE 5: The usage duration and number of usage of QoS Flow is determined by NWDAF using the SMF Events QoS Flow establishment (i.e. QFI Change) or QoS Flow termination (i.e. QFI deallocation) or Traffic binding of QoS Flow events. For example, the duration equals to the time period between the timestamp of QoS Flow establishment and QoS Flow termination events. NOTE 6: The predicted QoE is reported for the entire QoS set if only one parameter value has been provided for each parameter and otherwise separately for each combination of parameter values. |  |

Editor's note:	Clarification on Usage Duration information and Number of usage is needed.

# * * * * End of changes * * * *