3GPP TSG-WG SA2 Meeting #168	S2-2503424

Stor-Göteborg, Sweden, 7th - 11th Apr 2025	(revision of S2-2502143)

Title:	Reply LS to "OSAppID usage by AppToken use case"

Response to:	LS from GSMA TSGNS: OSAppID usage by AppToken use case (S2-2500077)

Release:	Rel-19

Work Item:	TEI19

Source:	SA2

To:	SA

Cc:	SA3

Contact Person:

Name:	Krisztian Kiss

E-mail:	krisztian@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 provides the following updates to SA and asks to consolidate the responses (SA2 and SA3) and respond to GSMA TSGNS LS that was received in TSGNS44_003v6 (S2-2501377).

- Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

Answers from SA2:

SA2 has not defined how the application identities are generated. However, based on the OSAppId definition in TS 23.503 (“An identifier associated with a given application and uniquely identifying the application within the UE for a given operating system”) it is unclear how the application backend could guarantee the uniqueness of the identifier within the UE for the given operating system without e.g. standardizing the format of the identifier or making sure the application backend has access to some kind of centralized database on OSAppIds.

SA2 believes such uniqueness can only be guaranteed if values for the OSAppId are reserved and managed by the operating system of the UE.

- Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

Answers from SA2:

The user privacy aspect is SA3’s responsibility, however it is unclear to SA2 how end user privacy is achieved by the proposed mechanism. The usage of App Token achieves anonymization of the OSAppId, however such anonymization looks redundant. Since the Operator Entitlement Server belongs to the MNO and is aware of the mapping between the App token and the OSAppId, the MNO has clear visibility on the applications installed on the UE, hence no user privacy is guaranteed.

Considering the MNO also learns information about when the user launches the application on the UE, end user privacy is considered even degraded compared to when no App Token mechanism is used.

- Consider the architectural implications of the proposed mechanism and provide feedback.

Answers from SA2:

SA2 specifications provide a number of traffic descriptors as defined in table 6.6.2.1-2 of TS 23.503 that allow applications to request for specific network slices and DNNs. Moreover, in Release 18, SA2 allows applications to indicate traffic categories, defined by GSMA NG.141, to accommodate network requirements, and in Rel-19, SA2 has defined mechanisms that allow application servers to request to replacement of specific slices without the need to provide application ID’s to the network as defined in subclause 5.15.5.2.2a of TS 23.501.

Additionally, SA2 would like to ask GSMA to clarify the validity of the AppToken and whether there are any associated timers, in order to identify if there are any impacts on the implementation on the UE side.

SA2 would also like to point out that the proposed mechanism is dependent on third party applications being able to pass the App Token to the OS in the UE which adds additional burden for third party application developers.

2. Actions:

To SA.

ACTION: 	SA2 kindly asks SA to take above SA2 input and respond to the GSMA TSGNSA LS that was received in TSGNS44_003v6 (S2-2501377).

3. Date of Next SA2 Meetings:

SA WG2#169	19 – 23, May 2025	Fukuoka, Japan

SA WG2#170	25 – 29, August 2025	Goteborg, Sweden