SA WG2 Meeting #168	S2-2503491

# April 7– 11, 2025; Goteborg, Sweden

Source: 	China Mobile

Title: 	Key issue for WT#2.2

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_ARCH_Ph4 / Rel-20

Abstract of the contribution: this paper proposes a key issue for the WT#2.2 of FS_5GSAT_ARCH_Ph4.

1	Discussion

In FS_5GSAT_ARCH_Ph4, the WT#2.2 proposes to study the UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available as following:

WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

Based on the WT, it is supposed to study how to support 5G-LAN service with IP and Ethernet PDU session type for UE-SAT-UE communications via UPF deployed on LEO or MEO satellite with NG-RAN, which should take the architectural enhancement and service continuity into account.

Therefore, we proposed to define the KI as followed:

-	Architectural enhancement to support UE-Satellite-UE communication via UPF onboard NGSO satellite(s) for 5G-LAN service with IP and Ethernet PDU Session type;

-	Service continuity of 5G-LAN service for UE-Satellite-UE communication when the serving NGSO satellite(s) changes.

2 Proposal

It is proposed to capture the following updates in TR 23.700-19.

5	Key Issues

5.x	Key Issue #x Support of 5G- LAN service for UE-satellite-UE communication under NR NTN NGSO constellation

5.X.1	Description

The 5G VN group communication for UE-Satellite-UE communication mainly includes local switch and N19-based forwarding. For 5G LAN-type service using N19 tunnel, there may be two UPFs located on different LEO satellites. Due to the characteristics of NGSO, the UPF on the satellite(s) will change frequently, which may lead to the interruption of the 5G-LAN service. Therefore, it is proposed to study the following issues:

-	Architectural enhancement to support UE-Satellite-UE communication via UPF onboard NGSO satellite(s) for 5G-LAN service with IP and Ethernet PDU Session type.

-	Service continuity of 5G-LAN service for UE-Satellite-UE communication when the serving NGSO satellite(s) changes.