3GPP TSG-WG SA2 Meeting #168	S2-2503519

# Gothenburg, Sweden, 7th Apr – 11th Apr, 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6226 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for available bitrate exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The conditions for the reporting of the Available bitrate are not fully clear. While the bitrate that NG-RAN is able to provide can change often (due to RAN internal decisions) a reporting should only occur when threshold(s) are matched or exceeded. Furthermore, NG-RAN shall only send a new report when the available bitrate information is different from the previously reported value(s) for at least one direction as subsequent reports of the same value are of no use to the AF. It is also necessary to describe the handling of the handover situation. It is proposed to require from the source RAN node a final report to avoid relying on the feature support at the target RAN node as well as SMF awareness of the handover (and extensions for SMF to UPF interaction for triggering such a final report). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of details about the conditions for the reporting. Addition of behavior during handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conditions for the reporting and the behavior during handover are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.45.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.45.6	Available bitrate monitoring

The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles by the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide in the respective direction for a QoS Flow within a duration. The duration is based on RAN implementation.

NOTE 1:	How the NG-RAN measures the available bitrate is NG-RAN implementation specific.

NOTE 2:	The available bitrate can be larger than the GFBR but can never exceed the MFBR. The available bitrate can also be smaller than the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only event triggered reporting applies.

Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting thresholds per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the thresholds. Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide changes and matches or exceeds indicated threshold(s), the NG-RAN reports the available bitrate information of the highest fulfilled threshold(s) for the UL and/or DL direction to the PSA UPF via the GTP-U header of UL packets (or dummy UL packets). The NG-RAN shall only send a report when the available bitrate information is different from the previously reported value(s) for at least one direction. If thresholds exist for both directions, the NG-RAN shall always indicate the available bitrate information for both directions when sending a report.

NOTE 3:	The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value than the one from the previous report. The report when the UL and/or DL available bitrate matches or exceeds a higher threshold value than the one from the previous report can occur with some hysteresis.

The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed.

During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

Before the handover is completed and if the previously reported bitrate value was different from the GFBR, the Source NG-RAN shall report the bitrate value of the GFBR to the PSA UPF.

NOTE 4:	The report of the Source NG-RAN during handover and the subsequent exposure of the GFBR value as Available Bitrate to the AF ensures that the AF is notified about the return to the GFBR which is expected to happen during the handover.

# * * * * End of changes * * * *