TSG SA WG2 Meeting #168	S2-2503543

07 - 11 April, 2025, Goteborg, SE

Source:	Lenovo

Title:	New KI: sensing data and associated information collection and transmission

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes KI description for sensing data and associated information collection and transmission

1	Background

According to the SP-250401[1], the study on Integrated Sensing and Communciation targeted at Rel-20 supports 5GS architectural and functional extensions so that sensing service can be supported. One of the objectives that this study attempts to address is:

-	WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation..

In this paper, sensing data and associated information collection and transmission are discussed.

2	Discussion

2.1 Sensing data collection and transmission

Generally, it is assumed that a new functionality in the 5GC (e.g., a new Sensing Function or an enhanced existing NF) is responsible to collect sensing data and sensing associated information in order to perform sensing result calculation. It is proposed that the UE (in case if UE-based sensing) and RAN node (in case of gNB-based sensing) are referred as sensing radio measurement entities. The 5GC selects one or more sensing radio measurement entities for a sensing tast and sends the sensing request to them. For gNB-based sensing, the new functionality in the 5GC collects sensing data from the sensing capable gNB directly or via AMF. For UE-based sensing, SF collects sensing data from sensing UE via control plane or user plane. At this stage, common aspect for both gNB-based sensing and UE-based sensing should be studied.

Proposal 1: it is proposed to study whether and how to enhance the existing 5GC architecture to support sensing data collection from the sensing radio measurement entities.

2.2 Sensing associated information collection and transmission

In TS 22.137 [2] , Sensing assistance information and sensing contextual information are defined as follows,

Sensing assistance information: information that is provided to the 5G system from a trusted third-party and can be used to support the derivation of a sensing result. This information does not contain 3GPP sensing data. NOTE 1: Examples of sensing assistance information are map information, area information, a UE ID attached to or in the proximity of the sensing target, UE position information, UE velocity information etc. Sensing contextual information: information that is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. This information does not contain 3GPP sensing data. NOTE 2: Examples includes map information, area information, time of capture, UE location and ID. This contextual information can be required in scenarios where the sensing result is to be combined with data from other sources outside the 5GS.

Meanwhile, WT-4 of Sensing SI is described in S2-250401[1] as follows,

-	WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation.

Based on the definition in TS 22.137 [2], “sensing assistance information” is provided to 5G system from a trusted third-party and can be used to derive sensing result. And “sensing associated information” in addition to sensing data (e.g. from the sensing radio measurement entities) in WT-4 is collected by 5GC for sensing result calculation.

Observation 1: “Sensing associated inforamtion” in WT-4 and “Sensing assistane information” in TS 22.137 are all used for sensing result calculation.

Regarding the examples of “sensing assistance information”, map information, area information and UE ID attached to or in the proximity of the sensing target are provided by trusted third-party. However, it is FFS whether other parameters are provided by trusted third-party or collected by 5GC internally? That is, sensing associated information may contain more parameters than the sensing assistance information provided by the trusted third-party.

Proposal 2: it is proposed to identify the sensing associated information which is useful for sensing result calculation and study how 5GC collects sensing associated information.

3 Reference

[1] SP-250401, New SID on Study on Stage 2 for Integrated Sensing and Communication, Xiaomi

[2] TS 22.137, 3rd Generation Partnership Project; Technical Specification Group TSG SA; Service requirements for Integrated Sensing and Communication; Stage 1

4 Proposal

It is proposed to capture the following changes to TR 23.700-14.

* * * * Start of Change * * * *

## 5.X	Key Issue X: < Sensing data and associated information collection and transmission >

### 5.X.1	Description

This key issue will address sensing data and assistance information collection and transmission. The key issue will study the following aspects:

-	Whether and how to enhance the existing 5GC architecture to support sensing data collection from the sensing radio measurement entities?

-	Identify the sensing associated information which is useful for sensing result calculation and study how 5GC collects sensing associated information?

* * * * End of Change * * * *