3GPP TSG-SA WG2#168	S2-2503663

Gothenburg, Sweden, 7-11 April 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated, Xiaomi

Title:	Discussion of the user consent and UE LCS privacy check for AI/ML based positioning

Document for:	Discussion

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This discussion paper analyses the user consent procedure and the UE LCS privacy check procedure for AI/ML based positioning.

# 1.	Discussion

## 1.1	User Consent supported in 3GPP

There were several discussions about user consent and UE LCS privacy procedure for AI/ML based positioning in Rel-19, the user consent procedure is already approved for Rel-19, but whether the UE LCS privacy is also applied for AI/ML based positioning is not determined yet. The following table 1.1-1 list the comparation for these two features that defined in the current specs.

Table 1.1-1: Comparation of User Consent and UE LCS Privacy

|  | User Consent | UE LCS privacy |
| --- | --- | --- |
| Definition | Indicates whether the user has given consent for collecting, distributing and analysing UE related data. User consent is provided per purpose (e.g. analytics, model training). (Table 5.2.3.3.1-1: UE Subscription data types in TS 23.502) | UE LCS privacy is a feature which allows a UE and/or AF to control which LCS clients and AFs are and are not allowed access to UE location information. (clause 5.4.1 in TS 23.273) |
| Storage | UDM | UDM |
| UE Subscription data types key | Data Key: SUPI Data Sub Key: Purpose (TS 23.502) | Data Key: SUPI (TS 23.502) |
| Profile | Enumeration: UserConsent: "CONSENT_NOT_GIVEN" or "CONSENT_GIVEN" Enumeration: UcPurpose: "ANALYTICS" "MODEL_TRAINING" "NW_CAP_EXPOSURE" "EDGEAPP_UE_LOCATION" (Clause 6.1.6.3.20 and 6.1.6.3.21 in TS 29.503) | Enumeration: LocationPrivacyInd: "LOCATION_DISALLOWED" or "LOCATION_ALLOWED" PrivacyCheckRelatedAction "LOCATION_NOT_ALLOWED", "LOCATION_ALLOWED_WITH_NOTIFICATION", "LOCATION_ALLOWED_WITHOUT_NOTIFICATION", "LOCATION_ALLOWED_WITHOUT_RESPONSE" "LOCATION_RESTRICTED_WITHOUT_RESPONSE" Enumeration: LcsClientClass "BROADCAST_SERVICE" "OM_IN_HPLMN" "OM_IN_VPLMN" "ANONYMOUS_LOCATION_SERVICE" "SPECIFIC_SERVICE" "NWDAF_IN_HPLMN" "NWDAF_IN_VPLMN" (clause 6.1.6.3.8, 6.1.6.3.9, 6.1.6.3.10 in TS 29.503) |

Based on the comparison in Table 1.1-1, although both User consent and UE LCS profile are stored as UE subscription, these two mechanisms are defined for different purposes. The user consent is applied to all the UE related data and may have different consents for different UE data collection purposes, while UE LCS privacy is only related to UE location information regardless the usage of UE location information.

Observation 1: The user consent is applied to all the UE related data and may have different consents for different UE data collection purposes, while UE LCS privacy is only related to UE location information regardless the usage of UE location information.

## 1.2	User consent and UE LCS privacy check LCS assisted NWDAF analytics

The Network data analytics assisted by LCS was the feature supported in Rel-18, NWDAF may collect location information for a target UE or a group of target UEs using LCS as described in clause 6.2.12 in TS 23.288. NWDAF performs user consent before the location request, if the user consent is granted, NWDAF works as a LCS client requests the location information from GMLC. GMLC interacts with LCS to obtain the UE’s location information. If privacy verification is required, GMLC will interact with UE via AMF before sending the location information to NWDAF as described in clause 6.2.12.2 in TS 23.288.

Observation 2: Rel-18 defined the NWDAF collects UE location information from LCS as input data to assist the NWDAF analytics, NWDAF performs the user consent check before UE location request, GMLC performs UE LCS privacy check before UE location collection.

## 1.3	UE LCS privacy check procedure for LCS assisted NWDAF analytics in Rel-18

The UE LCS privacy profile is stores as part of UE subscription data in the UDM, for following content are defined as part of UE LCS privacy profile:

- Location Privacy Indication as described in clause 5.4.2.3 in TS 23.273;

- Privacy Classes in clause 5.4.2.2 in TS 23.273;

- PrivacyCheckRelatedAction in clause 5.4.2.2 in TS 23.273 for each privacy classes

There is also a LCS privacy selection flow rule in Annex B.1 in TS 23.273 for information.

Clause 5.4.5 and clause 5.4.6 also clearly described the LCS service authorization for an Immediate UE location and a Deferred UE location.

As a summary, the authorization is determined by first verifying whether the location request is allowed by LPI (Location Privacy Indication) as described in Clause 5.4.2.3 in TS 23.273

The LPI at least includes one of the following global settings (for all LCS clients and AFs):

-	Location for UE is disallowed (location for UE not allowed to any LCS client except where POI applies).

-	Location for UE is allowed (default setting, and LCS requests for UE from LCS clients are authorized based on their associated privacy classes as defined in clause 5.4.2.2).

NOTE:	Additional LPI values may be supported for additional differentiation of location request types.

The LPI also allows the following optional settings:

-	Valid time period for LPI, including start time and end time.

The LPI takes precedence on the subscribed privacy classes as defined in clause 5.4.2.2. The LPI allows a UE to override the location preference of the subscribed privacy classes.

Only if LPI sets to location for UE is allowed in the valid time period, the GMLC will further check the Privacy Classes and the corresponding PrivacyCheckRelatedAction for the specific Privacy Classes.

Different privacy classes are defined in clause 5.4.2.2 in TS 23.273, Table 1.1-1 also shows the stage 3 defined details about the different privacy classes and the privacy check related actions. Based on different privacy classes and the corresponding privacy check related actions defined in LCS privacy profile, the GMLC will determine if the location Notification and verification request is required to send to UE or not.

However, For the PLMN operator Class client types that are permitted to receive UE location information, the privacy check related actions may always set as "location allowed without notification". Which means, if the LPI sets to “allowed”, and the LCS client is set as PLMN operator Class, GMLC will continue the UE location request procedure without notification / verification request to UE.

Based on the definition in clause 5.4.2.2.4 in TS 23.273, NWDAF is defined as PLMN operator class.

Besides the types of client specified in clause 9.5.3.4 of TS 23.271 [4], the PLMN Operator Class also supports positioning by the following types of client:

-	NWDAF in the HPLMN (when the UE is currently being served by the HPLMN);

-	NWDAF in the VPLMN.

Observation 3: During the LCS assisted NWDAF analytics procedure, NWDAF is worked as the LCS client. GMLC authorizes the LCS services by verifying the LPI in UE LCS privacy profile fist. If the LPI sets as “Location for UE is disallowed”, an error response will return to the NWDAF. If the LPI sets as “Location for UE is allowed”, the GMLC will further check the privacy classes of the LCS client. Since the NWDAF is allowed as PLMN Operator Class, GMLC will continue the UE location request procedure without notification / verification request to UE.

## 1.4	 UE LCS privacy check for AI/ML based positioning for Rel-19

The Data Collection for LMF-based AI/ML positioning based on NG RAN measurements is defined in clause 6.22.3 in TS 23.273, since there is no involvement of GMLC for the AI/ML based positioning, NWDAF sends the exposure subscription to LMF directly but not go through the GMLC.

In order to support the user consent check, it is already approved to support the interface between LMF and UDM in Rel-19. LMF requests the UE subscription from UDM in step 4 for user consent check. The only change to support UE privacy check is in step 4 as described in Figure 1.4-1. LMF checks not only user consent, but also UE LCS privacy with UDM before the data collection from UE / NG-RAN is performed.

Figure 1.4-1: Updated data collection procedure with UE LCS privacy check

Proposal: LMF subscribes the UE subscription for both User Consent check and UE LCS privacy check. The data collection procedure will be performed after both user consent and UE LCS privacy authorized.

# 2.	Text proposal

It is proposed to agree the CR S2-2503665 and S2-2503668.