3GPP TSG-SA2 Meeting #168	S2-2503700

# Stor-Göteborg, Sweden, 7th  - 11th Apr 2025			                            revision of S2-25xxxxx

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0310 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on I-SMF operation for N6 Delay Measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The feature for N6 delay measurement can be used by I-SMF for both LOM scenario and ETSUN not for LOM. Accordingly, the text needs to be revised. For LOM scenario, I-SMF does not need to perfrom N6 delay measurement for applications not matched with LOM information because there will be no local path estabalished for those application and L-PSA managed by I-SMF for LOM scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification on I-SMF’s use of N6 delay measurement for both LOM and non LOM scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N6 delay measurement for I-SMF operation is not clarified. Misoperation of I-SMF for LOM scenario is expected when performing N6 delay measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

## 6.9	Use of N6 Delay Measurement

To enhance EAS and local UPF (re)selection by considering N6 delay measurements per pair of 5GC N6 termination point (i.e. UPF/L-PSA) and the measurement endpoint at application side, as described in clause 4.3.3 of TS 23.502 [3].

The N6 delay measurement results received from the UPF may be used by the SMF in the following way:

-	Before a UE sends a DNS query, the SMF may create or update the DNS message handling rule on EASDF, e.g. to build/replace the EDNS Client Subnet (ECS) option or replace Local DNS Server to be used for a given FQDN.

-	For an already established connection to an EAS, the SMF may perform L-PSA UPF re-selection and/or an edge relocation considering N6 delay measurement and user plane latency requirement (clause 6.3.6) and then it may also trigger an EAS re-discovery procedure (clause 6.2.3.3).

In the Local Offloading Management scenario, the I-SMF may perform the N6 delay measurement for L-PSA UPF (re)selection and EAS (re)discovery. The I-SMF may receive information required for N6 delay measurement (i.e. indication of N6 delay consideration from the AF via SMF and retrieve N6 delay measurement assistance information from NEF). Based on these information, I-SMF may perform N6 delay measurements for pair of its candidate UPF(s) (i.e. L-PSA UPF) and measurement endpoint in the DN for the application, which is matched with Local Offloading Management Information.

In the scenario where I-SMF is inserted not for Local Offloading Management scenario, the I-SMF may perform the N6 delay measurement for L-PSA UPF (re)selection if I-SMF needs to insert or relocate UL CL/BP as described in clause 5.34.6.1 of TS 23.501 [1].

>>>> End of Changes <<<<