SA WG2 Meeting #168	S2-2503728

07 - 11 April, 2025, Goteborg, Sweden	(Revision of S2-xxx)

Source:	Samsung

Title:	KI#1 New Sol: Support of AIML data transfer via UP with AMF involvement

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes a new solution for AIML_CN_Ph2 WTI#1.

# Discussion

New SID on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2 was approved in SP-250413 with the following objective:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

This contribution provides solutions to address how to support standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

# Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** Start of the change ***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #x | x |  |

*** Next change (all new text) ***

## 6.X	Solution #X: Support of standardized data over UP for NR air interface operation with UE-side model training

### 6.X.1	Description

This solution aims to address the issues in WT#1: whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

As requested by RAN, SA2 is expected to study the transfer of data over UP for Option 2. The data transfer path of Option 2: UE-> CN -> Server for data collection for UE-side model training/OTT server. In general, the UP data is transferred from the UE to the target DN or application via UPF. To support Option 2, this solution proposes to transfer AIML data from UE to the 5GC NF that hosts the AIML data collection management functionality (e.g. DCF, Data Collection Function) via UP connection; then the DCF notifies/transfers the data to the AIML server for model training. This solution can provide AIML data visibility and controllability as required by RAN.

Editor's Note:	It is FFS whether to introduce a new 5GC NF or enhance the existing 5GC NF to support the functionality of AIML data collection management functionality.

NOTE:	further coordination with RAN WGs is needed to check the required visibility level of AIML data, e.g. fully or partial visibility.

### 6.X.2	Procedures

#### 6.X.2.1	Procedures of DCF initiated UP connection for data transfer

Figure 6.x.2.1-1: UE side AIML date transfer via a User Plane Connection between UE and DCF initiated by DCF

The DCF (data collection function) may trigger the user plane connection establishment for UE side AIML data transfer, e.g. after receiving AIML data collection request from AIML (training) server or based its internal logic, if target the UE does not have user plane connection with the corresponding DCF. The DCF may subscribe to the AMF for the status of the user plane connection for the target UE.

Figure 6.x.2.1-1 shows the procedures of DCF initiated UP connection for AIML UE side data transfer for AIML model training.

0.	[Optional] The AIML (model training) server may determine to request UE side data for AIML model training by sending request to or subscribe to DCF. The DCF may check the user consent to determine whether the data collection request can be complied or not.

1.	The DCF determines to request the UE side data for AIML model training purpose. This step might be triggered by the request from AIML server or DCF internal logic.

[Conditional] The DCF may also decide whether to transfer the AIML data via UP or CP. In this case, step 4 will be skipped.

2.	DCF sends data collection request to the target UE, e.g. via AMF or NG-RAN.

3.	UE performs data collection for AIML model training based on the request from network.

4.	[Conditional] DCF makes determination on AIML UE side transfer via UP. If the DCF make the decision in step 1, step 4 will be skipped.

The DCF may determine the AIML data transfer via UP based on the AIML data volume, the duration of the data collection and/or data transfer, UE user plane AIML data transfer capability, control plane and user plane congestion status, etc.

5.	[Conditional] If the DCF decides to utilize user plane connection for AIML data transfer but there is no established secure user plane connection between the UE and DCF, the DCF invokes Namf_communication_N1N2MessageTransfer service operation to send its user plane information to AMF in a NAS container to indicate UE to utilize user plane for AIML data transfer.

The user plane information includes the user plane address of DCF (e.g. IP address or FQDN).

DCF may also may allocate an AIML data collection binding/correlation ID to associate the user plane connection to be established with the target UE identify (SUPI and/or GPSI). The DCF may include this ID in the user plane information.

The target UE identify (SUPI and/or GPSI) might be indicated to the AMF by DCF in step 2

6.	[Conditional] When AMF receives the user plane information, AMF sends it to the target UE via a DL NAS TRANSPORT message by including the information in step 5.

7.	[Conditional] If there is no established applicable PDU session for the user plane AIML data transfer, the UE uses the URSP as defined in TS 23.503 which includes user plane AIML data transfer related PDU session parameters, e.g. a dedicated DNN, time window, S-NSSAI, location, etc., to establish the PDU session for user plane AIML data transfer.

8-9.	[Conditional] UE sends an acknowledgement to the DCF through AMF to indicate a success of user plane connection establishment for AIML data transfer via UP or a failure to utilize the user plane connection.

10.	[Conditional] The AMF stores the context of the UP connection for user plane AIML data transfer as part of UE context.

11.	AIML UE side data transfer for AIML model training from UE to DCF via the established UP connection with the assigned AIML data collection binding/correlation ID.

12.	DCF notifies the AIML UE side data for AIML model training to the AIML server.

The DCF may filtered the data to be transferred to the AIML server based on user consent and subject to operators’ policies.

13.	Data collection and/or transfer termination. The termination might be decided by the AIML server, the target UE or DCF.

#### 6.X.2.2	Procedures of UE initiated UP connection for data transfer

Figure 6.x.2.2-1: UE side AIML date transfer via a User Plane Connection between UE and DCF initiated by UE

UE may trigger the user plane connection establishment for UE side AIML data transfer, if the UE does not have user plane connection with DCF. Figure 6.x.2.2-1 shows the procedure of UE side AIML date transfer via a User Plane Connection between UE and DCF initiated by UE.

0.	[Optional] The AIML (model training) server may determine to request UE side data for AIML model training by sending request to or subscribe to DCF. The DCF may check the user consent to determine whether the data collection request can be complied or not.

1.	The DCF determines to request the UE side data for AIML model training purpose. This step might be triggered by the request from AIML server or DCF internal logic.

2.	DCF sends data collection request to the target UE, e.g. via AMF or NG-RAN. The DCF may optionally indicate its user plane information or NF ID to the UE.

3.	The target UE performs data collection for AIML model training based on the request from network.

4.	The UE makes determination on AIML UE side transfer via UP, e.g. based on AIML data volume, UE user plane AIML data transfer capability or other UE internal logic.

5.	UE sends a user plane connection establishment request to AMF via NAS Message with the indication of AIML data transfer via UP, if there is no established secure user plane connection between the UE and DCF.

6.	[Conditional] if the UE is authorized based on UE Subscription to use the user plane AIML data transfer, AMF to establish a user plane session for the UE.

If the DCF indicate its address or ID in step 2, the UE may indicate the DCF address to AMF in step 6. Therefore the AMF will directly establish the UP connection with corresponding DCF. Otherwise, the AMF may either query the NRF or based on local configuration to discover and select a proper DCF.

7.	The AMF sends request to DCF to request set up of UP connection for AIML data transfer, e.g. by invoking Ndcf_DataTrans_UPConfig request or other service operation.

8.	[Conditional] If DCF accepts to utilize user plane for AIML data transfer and there is no established secure user plane connection between the UE and DCF, DCF sends a user plane information to AMF to indicate the UE request is accepted.

The user plane information includes the user plane address of DCF (e.g. IP address or FQDN).

DCF may also may allocate an AIML data collection binding/correlation ID to associate the user plane connection to be established with the target UE identify (SUPI and/or GPSI). The DCF may include this ID in the user plane information.

9.	[Conditional] When AMF receives the user plane information from DCF, AMF forwards it to UE via a DL NAS TRANSPORT message by including the information in step 8.

10.	[Conditional] If there is no established applicable PDU session for the user plane AIML data transfer, the UE uses the URSP as defined in TS 23.503 which includes user plane AIML data transfer related PDU session parameters, e.g. a dedicated DNN, time window, S-NSSAI, location, etc., to establish the PDU session for user plane AIML data transfer.

11 - 12.	UE sends an acknowledgement to the DCF through AMF to indicate a success of user plane connection establishment for AIML data transfer via UP or a failure to utilize the user plane connection.

13.	DCF responds to AMF that user plane connection between the UE and DCF has been established.

Step 14 – 17 are the same as step 10 -13 in Figure 6.x.2.1-1.

### 6.X.3	Impacts on services, entities and interfaces

DCF (data collection function):

-	Support UE side data collection for AIML model training, including initiating and termination the data collection and transfer, establishing and configuring the UP connection, notifying the AIML data to AIML server.

AMF:

-	Establish UP connection between UE and DCF for AIML data transfer.

UPF:

-	Route UE side AIML data for model training to DCF.

*** End of the change ***