3GPP TSG-WG SA2 Meeting #168	S2-2503857

07 – 11 April 2025, Goteborg, Sweden	(revision of S2-250XXXX)

Source:	Lenovo

Title:	pCR to TS 23.369: AIoT Reader selection

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to enhance the description on AIoT Reader Selection.

# Introduction

According to the RAN3 LS in R3-250905, RAN3 concluded that the AIOTF is configured with AIoT RAN information via OAM mechanism. The AIoT RAN information may include: supported Area, served reader ID list, location of each reader.

It is proposed that the AIOTF selects the AIoT Reader based on 1) the configured AIoT RAN information and 2) the information from the Inventory Request which includes the requested Service Area Information.

# 2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

## 5.3	AIoT Reader Selection

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on configured AIoT RAN information and the information from the Inventory Request.

Editor’s note:	How AIOTF selects the AMF and the AIoT RAN in the indirect path is FFS.

The AF may provide information about the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal target area information, to the AIOTF.

Editor’s note:	How the NEF converts target area information to internal target area information and the format of the internal area information is FFS.

The AIOTF is locally configured (e.g. via OAM) with AIoT RAN information. The AIoT RAN information is specific for an AIoT RAN node and may include one or more of the parameters supported AIoT area of the AIoT RAN node, served reader ID list and location of each reader.

The AIOTF selects the AIoT RAN by mapping the internal target area for the AIoT service operation to the AIoT area of the AIoT RAN node(s) or AIoT Reader(s). If there is no the target area for the AIoT service operation, the AIOTF may also consider the last known location of the target AIoT Device(s) or based on operator policies and implementation means.

The AIOTF sends the Inventory Request to the selected AIoT RAN node(s) and includes the internal target area information. The internal target area information may be used by the AIoT RAN node for further down select an appropriate AIoT RAN reader ID. If the AIOTF is able to select one or more AIoT RAN readers, the Inventory Request contains the selected AIoT RAN reader ID list and does not include the internal target area information.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE:	AIoT RAN reader information is not exposed to the AF.

When the selected AIoT RAN node for the Inventory procedure responds to the AIOTF, the Inventory Report may include the AIoT RAN reader ID which received the response from the AIoT device. The AIOTF uses the AIoT RAN reader ID to update the last known AIoT RAN reader information in the AIoT device context.

If the AIOTF cannot select any AIoT RAN node or AIoT Reader for AIoT service request, then the AIOTF rejects the AIoT service request.

# * * * * End of changes * * * *