3GPP TSG-SA WG2#168	S2-2503893

Gothenburg, Sweden, 7th – 11th April, 2025	(was S2-2503829)

Source:	Huawei, HiSilicon, China Mobile, SK Telecom

Title:	Discussion on Rel-20 FS_XRM Ph3

Document for:	Discussion

Agenda Item:	30.6

Work Item / Release:	Rel-20

# 1	Discussion

## 1.1	General

This paper intends to further justify the necessities for study on XRM Ph3 and also give the potential objectives in the end.

# 2	Justification

With the higher bandwidth, lower latency, and larger capacity provided by 5GS, various emerging multimedia services are expected to be more and more popular in the mobile network, such as short video, live streaming and on-demand high resolution video, Augment Reality (AR) / Virtual Reality (VR) and cloud gaming services. In order to better support these services, the following aspects need to be considered:

1. Coordination between Application and Network

Since the beginning of 5G era, plenty of features have been introduced to enhance the coordination between the application and networks via N33/N5 APIs, e.g., application function requested QoS handling, the PDU Set based handling, the network information exposure and ECN marking for L4S.

However, the commercialization progress of such AF/AS based coordination features is not very well due to the difficulty for the 3rd party applications on the understanding and real deployments of the Application Function and also the API tests with hundreds of MNOs all over the world. Furthermore, the MNOs also need to negotiate with thousands of 3rd party applications on the potential beneficial APIs to be deployed. Hence, it is necessary to further introduce other mechanism for the real time coordination between the application and the 5G system as alternative to the existing AF/AS based coordination. The information defined in the pre-Rel-20 specifications can be a good basis for the real time exchanges between application and the 5G system, e.g. network information (i.e. congestion information, available data rate) and traffic characteristics (i.e. periodicity).

However, it could be easier for the coordination between application and 5G system via the UE in certain scenarios. At UE side, thanks to the developer-friendly development ecosystem, the application can coordinate with the network via the well-defined APIs exposed by the mobile OS. And as potential benefits, the UE/OS provider acting as a bridge between application and the network can acquire more knowledge on upper layer applications and network conditions for further internal intelligence. At the same time, the network can also realize potential network capability monetization reusing the existing OS/UE chipset ecosystem.

Based on the above analysis, the following Work Task is to be proposed:

WT-1: UE-NW in-band interaction for coordination between network and application:

WT-1.1:   Define mechanism to enable bi-directional (e.g. in-band) exchange of information between the network and the UE for the purpose of real time coordination between application onboard the UE and the network.

NOTE:	The information to be exchanged between the network and the UE can include the existing information defined in the pre-Rel-20 specs.

WT-1.2    E2E (i.e. between UE and UPF) network condition monitoring and exposure to the application, e.g. PDU or PDU Set granularity latency, jitter.

2. Low Jitter and stable delivery

Random jitter in RAN and transport network leads to distortion of traffic pattern and also unstable end-to-end latency, which may further degrade the real time user experience for cloud gaming, XR services, etc. For example, the unstable latency may lead to larger length of jitter buffer in the application layer, which causes higher end-to-end latency for the user experiences. Besides, more congestion control mechanisms in the upper layer base on the variances of RTT latency, which is more sensitive to the jitter and random jitter may lead to the degradation of application traffic.

In addition, the CN-PDB for the transport network indicates the upper bound of packet latency over N3/N9, which is based on the operator’s configuration. However, it can be dynamic given the number of routers that the packet travelled, the load status of these routers, etc.

Therefore, it is necessary to stabilize the packet transmission latency within the 5G system to reduce the application layer adjustments, e.g. low jitter/stable in N3/N9 for wide-area network, latency measurement in N3/N9 to assist RAN radio resource scheduling for stable end-to-end latency requirement (i.e. PDB between UE and UPF).

Based on the above analysis, the following Work Task is to be proposed:

WT-2: Study support of low-jitter stable delivery to reduce application layer adjustments, e.g. IEEE 802.1Qch (periodic schedule) in N3/N9 for stable transport network, latency measurement in N3/N9 to assist RAN radio resource scheduling.

3. Analysis and Prediction of DL Traffic Characteristics in UP

In Rel-19, dynamic DL characteristics are introduced to support the RAN scheduling, i.e. data burst size, time to the next burst.

However, these are only available in case the application server can get aware of them via an implementation-specific way and the RAN can get aware of this information only when the data burst is almost arrived.

Therefore, it is beneficial to further study the user plane based analysis and prediction for the DL traffic characteristics and provide the predicted DL traffic characteristics (e.g. predicted bitrate) in advance for RAN scheduling enhancements.

Based on the above analysis, the following Work Task is to be proposed:

WT-3: Study user plane-based analysis and prediction of DL traffic characteristics for efficient RAN radio resource scheduling.

4. Efficient In-Network Media Delivery

User experience of short video and live streaming services is highly affected by end-to-end latency, jitter and video resolution, which means low latency and low jitter can reduce the first screen delay and video frame freezing and high video resolution provides better picture quality. However, the QoS model of the 5G network does not consider the QoS treatment on the N6 side, the existence of unstable N6 delay and jitter has a negative impact on the end-to-end delay and jitter.

In addition, the video resolution selection may be restricted by the server side due to outbound bandwidth cost, for example, the high resolution may be limited during busy hours (with a large number of access users), which restricts the use of the high-resolution video.  In particular for the case of high-volume traffic the traditional architecture using content delivery network or application server, high egress costs on N6 (i.e. between the CDN or application server and the UPF) can occur.  Even in the case the CDN or application server deployed by the operator of mobile network, in practice it still typically operates by a separate entity from core network and egress costs will still be a factor on N6.

Caching and distribution of high-resolution video at UPF has the potential to solve the above issues which can be significant for the high volume, high data-rate applications. The SA1 requirements also include the requirement of content caching in core network, see clause 6.6.1 of TS22.261.

To support high-resolution video caching and distribution at UPF, the definition of UPF capability of media content caching and distribution is needed, and also the study of discovery of such UPF for UE (e.g. based on DNS procedure) is needed.

For the high-resolution video caching and distribution at UPF, SLA is needed between mobile operator and application service provider to determine which video are to be cache and distributed at UPF, and mobile operator is not to be aware of the content of the video, e.g. video content can be encrypted between UE and APP server at application layer. Also, when mobile operator is to provide the service, the user’s subscription needs to be considered.

Some additional benefits of high-resolution video caching and distribution at UPF:

- Low deployment cost. Only needs updates on the existing UPF, no additional device/server is needed.

- No need for customization of N6 protocol for traffic characteristics/information transmission from App server to UPF. Some traffic characteristics/information (e.g. data burst size, TTNB etc) have been defined, which can be used by NG-RAN to assist radio resource management, currently the traffic characteristics/information needs to be provided from App server to 5GC via a predefined N6 protocol, but in some cases, it may not easy for such N6 protocol(s) to be adopted by the OTT server/CDN server.

Based on the above analysis, the following Work Task is to be proposed:

WT-4: Study how to support for efficient in-network media delivery on user plane in 5GC, considering content caching, duplication and distribution at UPF, in order to reduce transmission latency and bandwidth for the use of protocols like HTTP(s), MoQ etc, with subscription information taken into consideration.

# 3	Proposals

The following WTs are proposed:

WT-1: UE-NW in-band interaction for coordination between network and application:

WT-1.1:  Define mechanism to enable bi-directional in-band exchange of information between the network and the UE for the purpose of real time coordination between application onboard the UE and the network.

NOTE:	The information to be exchanged between the network and the UE can include the existing information defined in the pre-Rel-20 specs.

WT-1.2  E2E (i.e. between UE and UPF) network condition monitoring and exposure to the application, e.g. PDU or PDU Set granularity latency, jitter.

WT-2: Study support of low-jitter stable delivery to reduce application layer adjustments, e.g. IEEE 802.1Qch (periodic schedule) in N3/N9 for stable transport network, latency measurement in N3/N9 to assist RAN radio resource scheduling.

WT-3:	Study user plane based analysis and prediction of DL traffic characteristics for efficient RAN radio resource scheduling.

WT-4:	 Study how to support for efficient in-network media delivery on user plane in 5GC, considering content caching, duplication and distribution at UPF, in order to reduce transmission latency and bandwidth for the use of protocols like HTTP(s), MoQ etc, with subscription information taken into consideration.