3GPP TSG-WG SA2 Meeting #168	S2-2503946

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-2503657)

Source:	Huawei, HiSilicon

Title:	[AA] Architecture assumptions

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Architecture Assumptions for 5GSAT_Ph4-ARC based on those used in phase 3.

# 1. Introduction/Discussion

The following architecture assumptions and principles are based on the ones used during the 5GSAT_Ph3-ARC study, and as the scope of the work in Rel-20 for many WTs in the SID is the same, many of the previous assumptions remain true for this phase 4 study.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change (all new) * * * *

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 23.401 [5] and TS 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Serving satellite: a satellite providing the satellite access to a UE (e.g. providing the serving cell(s)). Depending on the orbit, the serving satellite is covering a given geographic area for a limited period of time.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NOTE:	These definitions are different from those in TS 22.261 [6].

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

# * * * * Second change (all new) * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ISL	Inter-Satellite Link

NTN	Non-Terrestrial Network

# * * * * Third change (all new) * * * *

# 4	Architectural Assumptions

The following architecture assumptions are applied to the study:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [2] is used as a baseline.

-	Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard.

-	NR UE-Satellite-UE communication assumes NR NTN NGSO constellation with feeder link always available.

-  NR UE-Satellite-UE communication assumes non-IMS services via NGSO satellite(s) when, for example, a 5G-LAN service with IP and Ethernet PDU Session type is established between two subnetworks (e.g. on board two vessels).

-	NR UE-Satellite-UE communication assumes roaming for only IMS services including MMTEL and mission critical communications are considered.

-	NR UE-Satellite-UE communication roaming is restricted to two UEs served by same VPLMN and HPLMN.

-	NR UE-Satellite-UE communication for IMS assumes at least a gNB and the set of core network entities, as defined in Rel-19, is present onboard of the satellites.

-	NR UE-Satellite-UE communication for non-IMS assumes at least a gNB and a UPF is present onboard of the satellite.

-	NR UE-Satellite-UE communication assumes that the serving satellite will be of MEO or of LEO type.

-	NR UE-Satellite-UE communication service assumes communication between two UEs under the coverage of the same satellite or different satellites.

NOTE 1:	The study of the scenario when two UEs are under the coverage of the same satellite will be prioritized.

NOTE 2 : Whether solutions are able to support non-IMS services over UE-Satellite-UE communication via NGSO between more than two sub networks needs to be verified.

# * * * * End of changes * * * *