3GPP TSG-WG SA2 Meeting #169	S2-2504751

Fukuoka, Japan, 19th – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, New Solution: RAN Assisted UE Data Collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_ALML_CN_Ph2 / Rel-20

Abstract: A new solution is proposed for KI#1. The main idea is RAN can trigger UE selection, UE data measurement and collection either due to CN request or configuration.

# 1. Introduction/Discussion

This solution addresses Key Issue #1: Transfer of data over UP for UE data collection.

We try to solve KI#1 with the option that RAN triggers UE data measurement and collection. The trigger can be caused by RAN internal logic or configuration; it can also be caused by the core network. Thus, our solution is presented for the following two scenarios: 1) RAN determines to collect UE data based on its internal trigger; 2) Core Network informs RAN to initiate UE data collection. RAN can help with UE selection for UE data collection.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-04.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X | x |  |  |  |

# * * * * Second change (All new text)* * * *

## 6.X	Solution #X: RAN assist UE data collection

### 6.X.1	High level principles

The high-level principles of the solution are:

- RAN triggers UE data measurement and collection either by its internal logic/configuration or core network request.

- RAN triggers data measurement and collection with UE.

- RAN participates in UE selection for data collection.

- A dedicated NWDAF (DE-NWDAF) is responsible for UE data collection in the core network.

- UE setup UP connection with DE-NWDAF to transfer data for UE-side model training.

### 6.X.2	Description

For 5G Core assistance in data transfer from UE to OTT server, we assume that it will be a dedicated NWDAF with UE data exposure capability that collect data from UE. In this solution, the NWDAF with UE data exposure capability is called Data-Exposure NWDAF (DE-NWDAF).

It is also assumed that UE gets UP connection information, e.g., DE-NWDAF address, security related parameters, with UE policy parameters in the UE registration procedure.

This solution proposes RAN assists UE data collection in the following two scenarios: 1) RAN determines to collect UE data based on its internal trigger; 2) Core Network informs RAN to initiate UE data collection. RAN send data collection request to the UE and optionally participate in UE selection.

### 6.X.3	Procedures

#### 6.X.3.1 Procedure of RAN triggering UE data collection

6. data measurement and collection

UE

RAN

AMF

DE-NWDAF

1. Decide to trigger UE data collection

5. N2 Message(Updated UE list)

2. Select UE for data collection

3. RAN sends selected UE list to CN

UDM

4. AMF checks user consent for data collection

7. UE collects and reports measurement data

Figure 6.X.3.1-1 Procedure of RAN triggering UE data collection

1. RAN may determine to collect UE data based on its internal logic or configuration.

2. RAN selects UE for data collection based on UE information (e.g. UE status, capability, UE location).

3. Optionally, if RAN needs Core Network’s assistance for user consent checking, RAN sends the selected UE list to AMF.

Alternatively, if RAN doesn’t need Core Network’s assistance for user consent checking (e.g. RAN gets user consent information in the UE context data), then step 3-5 can be skipped.

Editor’s Note: Whether and how RAN can get user consent in UE context data is FFS and need to be discussed with SA WG3, RAN WG3.

Editor’s Note: The details of Message between RAN and AMF are FFS and need to be discussed with RAN WG3.

4. AMF checks user consent of selected UE(s) for data collection. AMF updates the UE list for data collection by removing those UEs that don’t grant user consent for UE data collection.

5. AMF provides the updated UE list to RAN.

6. RAN triggers data measurement and collection with UE.

Editor’s note: Details about how RAN sends data measurement and collection request to UE, e.g., in RRC message is up to RAN WG2.

7. UE collects data and reports to DE-NWDAF over the UP connection. If no UP connection between UE and DE-NWDAF has been establishment, UE will initiate UP connection establishment.

Editor’s Note: The detailed information and parameters exchanged between UE and DE-NWDAF over UP connection are FFS.

UE

RAN

AMF

DE-NWDAF

1. Decide to trigger UE data collection

4. N2 Message(UE Data Collection Request, [O] UE list)

7. UE collects and reports measurement data

2. Select UE for data collection

3. N1N2MsgTransfer (UE Data Collection Request, [O] UE list)

UDM

5. RAN selects UE for data collection

6. data measurement and collection

6.X.3.2 Procedure of CN informing RAN to initiate UE data collection

Figure 6.X.3.2-1 Procedure of CN informing RAN to initiate UE data collection

1. DE-NWDAF determines to collect UE data based on either data request from OTT server, or its internal trigger.

2. If no UE list is provided by OTT server, DE-NWDAF determines the UE list for data collection based on data collection requirement and UE information (e.g. User consent, UE location, PEI). Otherwise, DE-NWDAF checks user consent for the UEs provided by the OTT server.

3. DE-NWDAF sends UE data collection request to AMF. DE-NWDAF selects AMF based on AoI from its own configuration or OTT server. DE-NWDAF provides AoI, Data context ID, time window to AMF in order to inform AMF what UE data should be collected at which area during which time window.

Editor’s Note: what AMF service to be used for serving the UE data collection request from DE-NWDAF is FFS.

4. AMF informs RAN to initiate UE data collection in NGAP message. AMF chooses RAN based on e.g., AoI. AMF informs RAN Data context ID, time window and optionally UE list if available.

Editor’s Note: The details of message between RAN and AMF are FFS and need to be discussed with RAN WG3.

5. Optionally, if no UE list is provided by DE-NWDAF or RAN decides to do secondary UE selection, then RAN determines the UE list for data collection with assistance of Core Network as Step 2-5 in clause 6.X.3.1. In the case that AMF provides UE list, RAN doesn’t need to check user consent.

6 - 7. Same as step 6 – 7 in figure 6.X.3.1-1

### 6.X.4	Impacts on services, entities and interfaces

RAN:

- 	Initiate to collect UE data based on AMF request or internal trigger.

- 	RAN triggers data measurement and collection with UE.

- 	Receive UE data collection request from AMF.

- 	Participate in UE selection for data collection.

AMF:

- 	Receives UE data collection request from DE-NWDAF.

-	Send UE data collection request to RAN.

- 	Support UE selection for data collection: transfer selected UE list from RAN and provides updated UE list based on user consent.

# * * * * End of changes * * * *