3GPP TSG-WG SA2 Meeting #169	S2-2504755

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-250xxxx)

Title:	[DRAFT] LS Reply on Handling of UE Radio Capability for Paging

Response to:	LS (R3-252276) on Handling of UE Radio Capability for Paging from RAN3

Work Item:	NR_newRAT-Core

Source:	[Huawei to be] SA2

To:	RAN3

Cc:	RAN2

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 6289

- Overall Description:

SA2 thanks RAN3 for the LS on Handling of UE Radio Capability for Paging.

SA2 would like to answer RAN3’s questions as following:

Q1: RAN3 would like to ask SA2 that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then will the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs or total NG-RAN RATs in the UE RADIO CAPABILITY INFO INDICATION message?

SA2 Answer:

In clause 5.4.4.3 in TS 23.501, it has been described that “it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN”. SA2 agrees that the NG-RAN node could send to the AMF the missed paging information on the NG-RAN RAT(s), when the NG-RAN identifies that received UE Radio Capability for Paging Information does not contain paging information on the NG-RAN RAT(s) for the serving NG-RAN node. SA2 has added related description for this case as in the attached CR.

Q2: RAN3 would also like to ask if the “that PLMN” in TS 23.501 clause 5.4.4.3 is interpreted as the serving PLMN?

SA2 Answer:

The “that PLMN” in TS 23.501 clause 5.4.4.3 means the PLMN of the RAN and AMF, i.e. serving PLMN. SA2 has updated the description as in the attached CR.

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to take the above into account and update their specifications accordingly if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25th Aug – 29th Aug, 2025		Goteborg, SE

TSG-SA2 Meeting #171		13th Oct – 17th Oct, 2025		TBC, CN