3GPP TSG-SA2 Meeting #169	S2-2504892

Fukuoka, JP, 19-23 May, 2025

Source:	China Mobile

Title:	New solution for KI#2 Service authorization and revocation

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: the KI#2 Solution on Service authorization and revocation are proposed for FS_Sensing_ARC.

1. Introduction

This pCR proposes the KI#2 Solution on Service authorization and revocation.

2. Text Proposal

It is proposed to capture the following new solution of KI#2 in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X |  |  |  |  |

# * * * * Second change (All text new) * * * *

6.X	Solution #X: sensing service authorization and revocation triggered by AF

6.X.1	Description

This solution aims to address key issue#2: Authorization and Revocation to Support Sensing Service triggered by the AF via invoking the sensing service.

Sensing Function splits into Sensing Control Function (SCF) and the Sensing Processing Function (SPF). SCF is responsible for sensing service control, while SPF is responsible for sensing data collection and processing, etc.

Regarding the sensing service authorization, when the AF decides to trigger the sensing service, SCF will perform authorization for that AF to make sure that only authorized AF can make usage of the specific sensing service. With regard to the sensing service revocation, when the AF determines to stop the specific sensing service due to some reason, e.g., saving resource, the AF can send message to request the SCF to cancel the relevant sensing service, which leads to SF-C release the sensing resource, including the sensing entity referring to a Sensing Transmitter or to a Sensing Receiver.

NOTE x: 	In this solution, for trusted domain, the AF can trigger the sensing service authorization by directly sending the request to the SF-C without going via the NEF.

6.X.2	Procedure for sensing service authorization triggered by the AF

Figure 6.X.2-1: procedure for sensing service authorization.

1. The AF may sends Nnef_SensingService request message to the NEF to obtain sensing result from the network, which may include some sensing service parameters, e.g. target sensing service area, one-time report/periodical report/event-triggered report, sensing operation and so on.

2. The NEF authorizes the AF request of the sensing service. If the authorisation is not granted, the NEF replies to the AF indicating authorisation failure of the sensing service and the following steps will be skipped; otherwise, the NEF proceeds with the following steps.

3. The NEF sends the Nscf_ SensingService request message to the SCF. The request includes some sensing service parameters, e.g. target sensing service area, one-time report/periodical report/event-triggered report, and so on, which are carried in step 1.

4. The SCF performs authorization based on the sensing service requirement. If the authorisation is not granted, the following steps will be skipped and the SCF responds to the NEF indicating authorisation failure of the sensing service.

5. The SCF responds to the NEF indicating the success of the sensing service authorization via sending sends Nscf_ SensingService response message. Consequently, the SCF may select the appropriate sensing entity e.g., based on target sensing service area, and initiate the sensing procedure.

6. The NEF sends Nnef_ SensingService response message to the AF.

6.X.3	Procedure for sensing service revocation triggered by the AF

Figure 6.X.3-1: procedure for sensing service revocation

1. When determining to revoke the sensing service, the AF sends Nnef_ SensingService revocation request message to the NEF.

2. The NEF sends the Nscf_ SensingService_ revocation request message to the SCF to stop the sensing result reporting.

3. The SCF performs the sensing revocation procedure to stop the sensing result reporting.

4. The SCF sends Nscf_ SensingService revocation response message to the NEF indicating the service has been revoked.

5. The NEF sends Nnef_ SensingService revocation response message to the AF indicating the sensing service has been revoked.

6.X.4	Impacts on services, entities and interfaces

SCF:

-	Supports to perform sensing service authorization and/or revocation triggered by the AF.

NEF:

-	Supports to perform sensing service authorization and/or revocation triggered by the AF.

AF:

-	Supports to trigger sensing service and/or sensing service revocation.

# * * * * End of changes * * * *