3GPP TSG-SA2 Meeting #169	S2-2505053

# Fukuoka, Japan, May 19 – May 23, 2025	(revision of S2-2503414)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3913 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to S&F mode transition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Now the transition between S&F mode and Non-S&F mode only considers the UE behaviours that follows the existing AS and NAS procedures. This process is more accuracy to describe the UE behaviours in idle mode. For the UE in connected state to S&F mode, when transition to non-S&F mode occurs, there is nothing impacts. But for the UE in connected state to non-S&F mode, when transition to S&F mode occurs, since the network is not aware of whether the UE is S&F enabled (e.g., pre-19 UE), the MME should detach the UE at least in the split MME case, for the full CN onboard, the MME may detach the UE which can be by MME implementation. So for clarification, it would be better to add some descriptions for MME behaviours when transition to no-S&F mode when the UE is in connected state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when transition to S&F mode occurs and UE is in connected state, the MME may detach the UE with a cause non-specific to S&F Mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network behavios are not clear when transition to S&F mode occurs and UE is in connected state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.13.9.2	Transition between S&F mode and non S&F mode

If a UE is attached to a PLMN using satellite operating in S&F mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F mode (either TN or NTN eNodeB) and vice versa.

If a UE is in ECM-CONNECTED state with a satellite operating in non-S&F mode (i.e., the accessing eNB that is not broadcasting the indication of operating in S&F Mode), upon or after losing the feeder link, the MME may detach the UE with a cause non-specific to S&F Mode.

NOTE A:	It is up to implementation how the MME knows the state of the feeder link.

In order to ensure smooth idle mode transition from S&F mode to non S&F mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

* * * * End of Changes * * * *