3GPP TSG-SA WG2#169	S2-2505371

Fukuoka, Japan, 19 – 23 May 2025

Source:	CableLabs

Title:	Discussion on Rel-20 Indirect Network Sharing phase2

Document for:	Discussion

Agenda Item:	30.2

Work Item / Release:	Rel-20

# 1	Discussion

## 1.1	General

This paper intends to justify the addition of new WT for supporting indirect network sharing architectures for non-3GPP access sharing.

# 2	Justification

Existing 3GPP specifications focus on network sharing for 3GPP radio access technology (RAT) only. Extending existing indirect network sharing (INS) architecture to include non-3GPP access allows operators to create a more comprehensive and versatile network footprint. Integrating these disparate access types enables seamless mobility and service continuity, opening new revenue streams through bundled service offerings, while simplifying inter-operator network sharing across RATs. By incorporating non-3GPP access in network sharing, operators can create a more efficient, flexible, and customer-centric telecommunications landscape.

The Rel-19 specified INS architecture enables the communication between the shared RAN and the Participating Operator’s core network being routed through the Hosting RAN Operator’s core network. INS addresses a key challenge for the operators with regards to the maintenance of the interconnections (e.g., number of network interfaces) between the shared RAN and participating operators’ core networks, especially in the case of a large number of the shared base stations. Although the same issue does not exist with the deployments of non-3GPP access, there are other potential challenges.

In 5G, for inter-operator scenario, the untrusted non-3GPP access network connection to a 3GPP core is via the non-3GPP interworking function (N3IWF). Regardless of whether the non-3GPP access is owned and operated by the hosting network (managed Wi-Fi) or the non-3GPP access is from a public Wi-Fi (unmanaged Wi-Fi), it is treated as untrusted non-3GPP access by the participating operator. In case of untrusted non-3GPP access, the 3GPP and non-3GPP access registrations are independent of one another. Once the UE registers with the untrusted non-3GPP access using non-3GPP access credentials, and has internet access, it registers with the N3IWF formulating an FQDN that resolves to a public IP address, where the traffic flows from the non-3GPP access of the hosting operator to the N3IWF of the participating operator over the internet. This is an unmanaged connection where the participating operator has no control over how the traffic traverses from the untrusted non-3GPP access to the N3IWF which may result in potential high latency.

One way to resolve this issue could be with using the INS architecture for non-3GPP access sharing similar to the specified INS architecture for 3GPP access. Figure 1 shows the proposed INS architecture for non-3GPP access sharing. Here if the non-3GPP access is owned and operated by the hosting operator, the host operator can treat the non-3GPP access as a trusted non-3GPP access network. Trusted non-3GPP access can benefit with authenticated access to the Wi-Fi network using the UE’s SIM credentials unlike untrusted where the device has to authenticate twice, once to gain access to the Wi-Fi network and then for the untrusted access to the 5GC via N3IWF. Furthermore, with non-3GPP access network owned and operated by the host operator, the operator can have complete control over how the traffic traverses to participating operator’s core network via the host operator network.

Figure 1: Indirect Network Sharing architecture for non-3GPP access sharing

Additionally, if Operator X also owns and operates a 3GPP access that is shared by the participating operators, the same interfaces between the host operator’s core network and participating operator’s core network specified as part of the INS architecture for 3GPP access sharing can be utilized for sharing both the 3GPP and non-3GPP access of the host operator. Figure 2 shows the host operator network X sharing both 3GPP and non-3GPP access using the specified INS architecture.

Figure 2: Indirect Network Sharing architecture for 3GPP and non-3GPP access sharing

Another architecture that could be envisioned is when two operators are sharing each other’s RAT. Figure 3 shows Operator X who owns and operates a non-3GPP access network that is shared with Operator Y, and Operator Y who owns and operates a 3GPP access network shared with Operator X. So, for non-3GPP access sharing, Operator X is the hosting operator and Operator Y is the participating operator, while for 3GPP access sharing, Operator Y is the hosting operator and Operator X is the participating operator. And the same interfaces shared between the Operator X and Operator Y networks are leveraged for indirect network sharing of different RATs each operator owns.

Figure 3: Indirect Network Sharing of different RATs across operators

# 3	Proposals

The following WTs are proposed:

WT#3: Extend the Indirect Network Sharing mechanisms for sharing of non-3GPP access (trusted/untrusted).