3GPP TSG SA WG2 Meeting #169	S2-2505415

Fukuoka, Japan, 19-23 May, 2025	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	new solution for KI#2 Revocation of an ongoing Sensing Service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This paper proposes the solution for KI#2 to provide revocation of an ongoing Sensing Service in TR 23.700-04.

* * * Start of Changes (ALL NEW TEXT) * * *

6.X	Solution #X: < Revocation of an ongoing Sensing Service >

6.X.0	High-level solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

In this solution, Sensing Function is responsible for performing the revocation of the Sensing Service.

6.X.1	Description

This solution is aiming to resolve the revocation part of Key Issue#2 “Authorization and Revocation to Support Sensing Service”

It assumes that Sensing Function is responsible for receiving the request(s) or trigger(s) to revoke the Sensing Service the Sensing Service, and to send the revocation request to Sensing Entity(ies) to terminate or temporarily deactivate the Sensing operations (e.g., transmitting or receiving the sensing signals, etc.).

Sensing Service ID is defined to uniquely identify the ongoing Sensing Service in core network.

6.X.2	Procedures

The following procedure as shown in figure 6.X.2-1, is to provide general information flows for revocation of the Sensing Service.

figure 6.X.2 procedure for the revocation of Sensing Service

Procedure:

0), One Sensing Service is initiated and indicated with Sensing Service ID. The sensing measurement data is collected by Sensing Function, and the Sensing Function sends the Sensing result to Sensing Service Consumer.

1), Sensing Function receives the trigger/request for revocation of the specific Sensing Service, due to specific condition(s) are met, e.g., due to some regulation reasons, the Sensing Service consumer is not authorized to request the Sensing Service, or the sensing target area is not allowed to provide Sensing Service, etc.

2a), Sensing Function sends the Sensing Service revocation request to Sensing Entity/Entities (Sensing transmitter(s), or/and Sensing receiver(s)), including Sensing Service ID

2b), Sensing Function may send the Sensing Service revocation notification to Sensing Service Consumer with proper cause directly or via NEF.

3), Sensing Entities/Entity terminates or temporarily deactivate the Sensing Service, for example, stop transmitting and/or receiving the sensing signals, and optionally remove the configuration/parameters for this Sensing Service that is identified by the Sensing Service ID

4), Sensing Entity/Entities send the Sensing Service revocation response to the Sensing Function with result.

5), Sensing Function may send the Sensing Service revocation notification to Sensing Service consumer with proper cause directly or NEF. If step 2a is performed, step 5 is skip.

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

New SF:

- To receive the revocation request/trigger

- To send the revocation notification to Sensing Service Consumer

* * * End of Changes * * *