3GPP TSG-SA WG2 Meeting #169	S2-2505591

Fukuoka, Japan, May 19 – 23, 2025

Source:	Samsung

Title:	KI#3: New solution on Energy-aware NF discovery/selection mechanism

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on NF selection based on energy related information.

1.	Discussion

One of the key issues for Rel-20 EnergySys Phase2 Study includes the discussion on solutions that can support NF selection based on the energy related information. A new solution is proposed to support NF selection based on the energy related information.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change (all new text) * * * *

6.X	Solution #X: NF selection based on energy related information

6.X.0	High-level solution Principles

The solution enables the consumer NF to select or re-select the target NF based on the energy related information of the candidate NFs and focuses on the procedural aspect, i.e., NF registration/update/discovery/status subscribe procedures, UPF re-selection procedures.

6.X.2	Functional Description

NRF maintains also energy related information as part of NF profile for registered NF instance. Such energy related information includes the example in Table 6.X.2-1 described.

Table 6.X.2-1: Energy related Information in the NF profiles

| Information element name | Description |
| --- | --- |
| Energy consumption level | Indicating amount of energy consumption level by the NF over time window. |

Editor’s Note: It I FFS what can be included in the energy related information in Table 6.X.2-1.

NRF may return energy related information as part of the NF profile in response to a NF discovery request. When receiving a request to discover a NF service, NRF considers also energy aware NF discovery policy/criteria (locally configured or included in the NF discovery request as network operation mode) and provides the NF information accordingly.

The

6.X.3	Procedures

6.X.3.1	General

The NRF service procedure as defined in clause 4.17 of TS 23.502 [3] are reused for energy related NF management and discovery.

The SMF may re-select the UPF based on the energy related information.

6.X.3.1	UPF re-selection considering energy information procedure

Figure 6.X.3.6-1 describes the enhanced procedure of the UPF re-selection by the SMF.

Figure 6.X.3.6-1: Procedure for network function reselection

0.	The PDU Session is established over SMF/UPF1. The SMF subscribed to the NRF with the threshold as described in Figure 6.X.3.5-1.

1.	The NRF detects that the energy efficiency or energy consumption exceeds its threshold that is set to UPF1.

2.	The NRF notifies to the SMF, with the indication of threshold exceeded and optionally, the NF profile(s) including Energy related information of the candidate NFs.

3.	The SMF determines whether to re-select the UPF for the PDU Session.

4.	The UPF discovery and re-selection considering Energy related information is performed as described in Figure 6.X.3.4-1.

5.	The PDU session over UPF1 is released, and the PDU session re-established over UPF2. The SSC mode of the PDU Session should be followed.

6.X.4	Impacts on existing services, entities and interfaces

NRF:

-	Management of NF profile including the energy related information

SMF:

-	SMF re-selects the UPF by considering the energy related information.

* * * End of Change * * * *