SA WG2 Meeting #169	S2-2505006

# Fukuoka, Japan, May 19 – May 23, 2025                        (revision of S2-25xxxxx)

Source: 	CATT

Title: 	Update on Reader Selection

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes update on Reader Selection to TS 23.369.

1	Proposal

It is proposed to update the TS 23.369 as followings.

# * * * * First change* * * *

5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers.

The AIOTF obtains the NG-RAN information (supported Area (as a list of “A-IoT Areas”), RAN reader ID list, and, optionally, the location of each served RAN reader) via OAM. The AIOTF receives an AIoT service request including the Target Area information from the NEF or trusted AF. Based on the received Target Area information and the NG-RAN information configured by OAM, the AIOTF selects the NG-RAN node(s) and optionally RAN reader(s).

NOTE 1:	The Target Area information can span the supported Area of multiple NG-RAN node(s) or can be a subset of the supported Area of a NG-RAN node.

The AIOTF sends the AIoT service request to the selected NG-RAN node(s), optionally including the RAN Area information derived from mapping the Target Area information to the supported Area (as a list of “A-IoT Areas”) and/or RAN reader ID list, if selected, to assist RAN reader selection by NG-RAN node, either directly or through the selected AMF. If multiple NG-RAN nodes are selected, the AIOTF sends the AIoT service request to each selected NG-RAN node along with its corresponding RAN Area information and/or the RAN reader ID list, if selected

If the AIOTF does not provide the RAN Area information or the RAN reader ID list in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN readers.

Editor’s note:	NG-RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of NG-RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how NG-RAN transfers and updates its information to the AIOTF in indirect connectivity.

NOTE 2:	RAN reader ID is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) from the AIoT device context to determine targeted RAN reader(s) directly.

NOTE 3:	From the selected NG-RAN node, the AIOTF receives the Inventory Report including the RAN reader ID that represents the AIoT device’s location at reader ID granularity. The AIOTF uses the RAN reader ID to update the last known serving RAN reader information in the local AIoT device context.

# * * * * End of Changes * * * *