3GPP TSG-SA2 Meeting #169	S2-2505134

Fukuoka, Japan, 19th May 2025 - 23rd May 2025

Source:				CEWiT

Title:			Solution for KI#5: Exposure of Sensing Results

Document for:				Approval

Agenda Item:				20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The contribution proposes a solution for Key Issue #5: Sensing Result Exposure

- Introduction

This paper presents a solution that enables the 5G System (5GS) to support the exposure of the computed Sensing Results.

- Text Proposal

It is proposed to agree the following changes in 3GPP TR 23.700-14.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #x> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |

## 6.X	Solution #X: Sensing Result Exposure

### 6.X.1	Description

* * * * Start of Changes * * * *

The proposed solution addresses Key Issue #5.

For the Sensing feature to be supported in the network, a NF namely “SeMF (Sensing Management Function)” is introduced to control and manage sensing operations.

The SeMF can receive requests and expose the computed Sensing Results to,

-	AF(s) authorized by NEF, allowing the AF(s) to leverage real-time sensing information.

-	AF(s), NF(s) in Core network

The Sensing result exposure can be one time, periodic or event triggered.

### 6.X.1.x	Sensing Result Exposure

The following attributes may be present in the sensing service requests:

-	AF ID

-	Sensing Area in terms of Location Co-ordinates, Supported GAD Shapes, Cell ID, TA

-	Object(s) count

-	Time Stamp of Sensing Result information

-	Object(s) dimensions

-	Object(s) status i.e. Stationary, Moving

-	Velocity of object(s)

-	Type of Event Reporting

-	One time, Periodic, Event Triggered

-	Maximum number of Event Reports

-	Duration of Event Reporting

-	Quality of Sensing Result

-	Sensing Result Response Time

The following attributes may be present in the sensing service response:

-	Location of sensed object(s) in terms of Location Co-ordinates, Supported GAD Shapes, Cell ID, TA

-	Sensed object count

-	Time Stamp of Sensing Result information

-	Object(s) status i.e. Stationary, Moving

-	Velocity of sensed object(s)

-	Dimensions of sensed object(s)

-	Quality Indication of Sensing Result

### 6.X.2	Procedures

The following procedure as in Figure 6.X.2-1 provide architecture support for sensing result exposure.

Figure 6.X.2-1: Procedure to support Sensing Result Exposure

1.	The request for a sensing service may come from an AF authorized by NEF, AF or any other NF.

1a-1.	An AF sends a request to the NEF by invoking the Nnef_SensingExposure_Subscribe service. The request may include the AF ID and other parameters listed in clause 6.X.1.x.

1a-2.	The NEF authorizes the request from the AF and performs SeMF selection and discovery either based on local configuration or by querying the NRF based on the parameters in AF request. To the selected SeMF, the NEF invokes the Nsemf_Sensing request service carrying the AF request.

1b.		A AF/NF performs SeMF selection and discovery either based on local configuration or by querying the NRF based on the parameters. To the selected SeMF, the AF/NF invokes the Nsemf_Sensing request service. The request may include the parameters listed in clause 6.X.1.x.

2.	The requested sensing area is converted and mapped to Cell ID, TAI etc. by the SeMF. Based on the sensing area and other parameters in the request, the SeMF authorizes the sensing request.

After successful authorization, the sensing entity selection is performed by the SeMF and the sensing measurement is collected by interacting with sensing entities. Using the collected data, the sensing result is computed.

3.	Based on the request received in step 1, the SeMF reports the sensing results containing parameters listed in clause 6.X.1.x.

### 6.X.3	Impacts on Services, Entities and Interfaces

The following impacts may be present in the following entities for sensing result exposure,

SeMF:

-	Support for sensing request processing and sensing result exposure to AF/NF.

NEF:

-	Enhancements to support Sensing Result exposure to authorized AF(s).

* * * * End of Changes * * * *