SA WG2 Meeting #168	S2-2505174

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-2503690)

Source:	ZTE

Title:	pCR to TS 23.369: AIOTF and NEF Service Definition Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper updates AIOTF and NEF provided AIoT service definition in clause 7 to address the editor’s notes.

1. Discussion

For the following ENs:

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect path.

The AMF provides services to interact with the AIOTF to report the AIoT information, this EN could be removed.

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

It is proposed to unified command service including different operations (read, write and disable) to simplify the service definition; this EN could be removed.

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Editor’s note:	The Command specific parameters for Read/Write/Disable command is FFS.

The AIoT data for commands and the related specific parameters are clarified in the AIOTF service definition.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

There is result indication for write and permanent disable operations, it is proposed to the approximate message size information also applies for write and permanent disable operations; this EN is removed and some clarification is added for this parameter

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

If no AIoT device is discovered during the service operation, this also could be considered as a result of the operation, and it’s different with the AIOTF or the NG-RAN fails to execute the operation. So it is proposed to take this as a operation result.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

It is proposed to add an indication in the final report to indicate that the report is finished.

For the editor’s notes in clause 7.4 of NEF services, these ENs are very similar with the above ENs in AIOTF services, they will be handled in a similar way.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

7.2	AIOTF services

7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

7.2.2	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permenant Disable.

5)	Command specific parameters:

-	Read: AIoT Device memory location, AIoT memory size.

-	Write: AIoT Device memory location, AIoT data.

-	Permanent Disable: Security material for disable operation.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Command Operation.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.4	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation.

Inputs, Required:

1)	Common report information: Transaction ID, List of AIoT Device ID or Failure Cause in case of Failure.

2)	Read command specific report information: Information obtained from each target AIoT Device.

Inputs, Optional: Last report indication.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * * Next Change * * * *

7.4	NEF services

7.4.1	General

In addition to those defined in clause 7.2.8 of TS 23.501 [3] and clause 5.2.6 of TS 23.502 [4], table 7.4.1-1 illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable).

Command specific parameters:

-	Read: AIoT Device memory location, AIoT memory size.

-	Write: AIoT Device memory location, AIoT data.

-	Permanent Disable: Security material for disable operation.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Command operation.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: AF Transaction ID, a list of AIoT Device ID(s) or Failure Cause in case of Failure, Information obtained from each target AIoT Device.

Input, Optional: Last report indication.

Output, Required: Result indication.

Output, Optional: None.

* * * * End of Changes * * * *