3GPP TSG SA WG2 Meeting #170	S2-2507332

Goteborg, Sweden, 25 – 29 August 2025

Title:	[DRAFT] LS on Triggers for Data Collection

Response to:	-

Release:	Release 20

Work Item:	FS_AIML_CN_Ph2

Source:	Lenovo [to be SA2]

To:	RAN, RAN2,

CC:	RAN1, RAN3

Contact person:	Dimitrios Karampatsis (dkarampatsis@lenovo.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

A first round of solutions for data collection have been submitted within TR 23.700-04 as part of FS_AIML_CN_ph2 WT1 objectives. There have been different flavors of solutions, some suggesting a direct interaction between a data collection entity and the UE while others suggesting involvement/assistance of NG-RAN in aspects such as selection of UEs and data collection (measurement) configuration.

Solutions can be, generally, categorized in two types of solutions

-	Type 1 of solutions: Direct interaction between DCF and UE

-	The Data Collection Function in the core receives a request for data from an AF (UE side server), optionally selects UEs for retrieving the requested data and send a data delivery request to the UE (via NAS or via UP). UE provides the data requested via user plane.

-	Type 2 of solutions: RAN assisted data collection

-	The Data Collection Function in the core receives a request for data from an AF (UE side server). The Data Collection Function sends the request to the AMF that interacts with NG-RAN for data collection configuration (configuration on how the UE measures and logs the data) and data collection transfer (configuration on how to transfer logged data via user plane). Solutions also propose the NG-RAN to be involved for UE selection for configuration to collect the data and delivery. UE provides the data requested via user plane.

SA2 has the following questions for further feedback in order to facilitate discussions in SA2 during Q4 2025.

Trigger for data collection configuration

The most recent feedback from RAN2 in R2-2411152/S2-2500025 (handled in SA2#166 Adhoc-e) states as part of the response to Question 1 that:

- Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements).

- Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

In addition as part of the same response RAN2 stated that there was no decision made on when data collection is initiated:

Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server))

Question 1:

Further feedback is needed to clarify whether the trigger for data collection configuration of UEs by gNB is independent of any data transfer request (requested by UE-side server) or the trigger for data collection configuration is based on the data needed by a UE-side server within a data transfer request.

Examples for trigger for data collection are:

-	Data Collection trigger independent to data transfer request: UE is configured with data collection configuration from gNB. When UE receives a data transfer request the UE provides the data if available and collected. UE does not request from gNB new configurations to retrieve the requested data.

-	Data collection trigger dependent to data transfer request: When UE receives a data transfer request the UE request data collection configuration to collect the data requested by the UE-side server or the gNB configures the UE with data collection configuration based on data transfer request received via core network from a UE-side server.

Question 2:

In addition, feedback is needed to clarify whether the data collection configuration changes in a UE, especially in scenarios of mobility where the UE handovers to different gNBs that e.g., does not support one or more of configurations provided by other gNBs.

UE selection for data collection configuration

Some solutions in TR 23700-04 assume that the gNB is involved in UE selection for data collection configuration and data transfer.

Question 3

SA2 asks RAN2 to clarify whether selection of UEs is feasible by gNB given the varying radio conditions and mobility of UEs.

# 2	Actions

To RAN, RAN2

ACTION: 	SA2 kindly request feedback to the questions raised in this LS in order to facilitate discussions in SA2 during Q4 2025.

# 3	Dates of next TSG SA WG 2 meetings

SA2#171		October 13-17, 2025      		Wuhan, China

SA2#172		November 17-21, 2025		Dallas, US