SA WG2 Meeting #171	S2-2508224

13 – 17 Oct, 2025, Wuhan, China	(revision of S2-2508130)

Source:	vivo

Title:	[WT#3.1 AI] AI for 6G/AI capable 6GC

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document summarises the WT and Key Issue proposals for WT#3 regarding the use of AI for 6G architecture.

1. Proposal

Regarding WT#3.1 AI for 6G architecture, this contribution is to provide WT and KI for WT#3.1.

In addition to vivo own view, we also tried to summarise the KI proposals submitted by companies in SA2#170 in Annex as we think it may be good starting point though companies may further update their view.

Please see Annex for the KI summary.

**** First Change ****

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.1.0:   Study how to support reliable and sustainable AI

WT#3.1.1:	Study how to natively integrate and use AI technologies (e.g. AI Agent, NF embedded - AI, Federated Learning, Reinforcement Learning, etc.) into the 6G CN to support customised services and achieve increased levels of performance, efficiency, scalability , while enabling multi-vendor interoperability, including:

-	understanding user service requests (e.g. Intent based or non-intent based) e.g. from UE APP or AF,

-	acting autonomously and controllably with acquiring contextual information, reasoning, explainability, self-learning, decision-making, generating results, interacting with other network function(s), enabling closed-loop interactions,

-	accessing trusted external capabilities provided by AF and internal network AI capabilities provided by 6G NF,

-	selecting/determining AI based methods or non-AI based methods.

NOTE 2: 	The term Intent refers to expectations including requirements, goals and constraints without specifying how to achieve them. As the SA1 work is still in progress, adapting the definition of Intent from SA1 and the use of the term Intent will be determined as part of the study. Intent does not imply any specific solution.

WT#3.1.2:	Study whether and how to:

-	support AI-related functionalities (e.g. model training, model provisioning, performance monitoring) into 6G CN considering 5G CN AI-related functionalities as a starting point for discussion, which includes model interoperability.

-	coordinate between 5G CN AI and 6G CN AI, if needed, e.g. for purpose of consistent service across 5G and 6G.

NOTE 3 :	For WT#3.1.2, aspects related to SA5 will be coordinated with SA5 during the study, aspects related to RAN will be coordinated with RAN during the study.

**** Next Change(all new text) ****

5.X. Key Issue #X: Support for AI in 6G Core entities

### 5.X.1	Description

The objective of this Key issue is to study how to introduce AI technologies into the 6G CN with considering the follow aspects:

-  Study how to support reliable and sustainable AI

-  Study architecture supports for native AI in 6G CN using AI technologies (e.g. AI Agent, NF-embedded AI, Federated Learning, Reinforcement Learning, etc.)

-  Study whether and how to support Closed-loop AI techniques in 6G Core by leveraging Network AI agent(s) and 6G CN NF(s) with built-in AI capability (6937)

NOTE 1: Closed-loop AI techniques in this context refer to a 6G Core entity that continuously monitors, evaluates, and adjusts its actions based on data and feedback received internally or externally to the 6G entity. This creates a feedback loop that enables the 6G entities to improve its performance over time and adapt dynamically to changing conditions. Some closed loops may be entirely inside one 6G entity and will in this case impose very little or no impact in standardization (6937)

NOTE 2: This KI may place some OAM requirement for the implementation of the closed loop therefore coordination with SA5 may be needed (6937)

-  Study whether and how to support interaction between UE/AF and 6G CN NF(s) (e.g. with built-in AI capability and/or AI Agent) to interpret service request e.g. intent based or non-intent based

-  Study how to support interaction among 6G CN NF(s) (e.g. with built-in AI capability and/or AI Agent) to invoke operator service, in order to fulfil the user service request

-  Study whether and how 6G CN access trusted external capabilities provided AF via exposure framework

-  Study how to support registration, discovery of 6G CN NF(s) e.g. with built-in AI capability and/or AI Agent

-  Study whether and how to support distributed Intelligent Framework via e.g. Federated Learning techniques and how to avoid goal conflicts

- Study mechanisms for operators to control and configure models to select AI based methods or non-AI based methods (6445,7359)

- How to use unified data collection framework to support AI related activities e.g. data collection, data storage, data exposure, data privacy and security management, model sharing/delivery, etc.

NOTE 3: Requirements for the data handling to be addressed in WT#5 if possible, requirements on the computing services to be addressed in WT#6 if possible. The data collection is performed according to the conclusions of WT#5

- Study whether and how to:

-	support AI-related functionalities (e.g. model training, model provisioning, performance monitoring) into 6G CN considering 5G CN AI-related functionalities as a starting point for discussion, which includes model interoperability.

- coordinate between 5G CN AI and 6G CN AI, if needed, e.g. for purpose of consistent service across 5G and 6G.

NOTE 4: Aspects related to SA3/SA5 will be coordinated with SA3/SA5 during the study, aspects related to RAN will be coordinated with RAN during the study.

**** End of Changes ****

Annex: Summarizing the KI proposals submitted by companies in SA2#170

1. Discussion

Based on the Tdoc list provided in the TDoc_ordering folder, the content of the following documents is considered to be merged (partially) in this contribution regarding the use of AI for 6G architecture.

| No. | Tdoc | Title | Source |
| --- | --- | --- | --- |
| 1 | S2-2506362 | [WT#3, AI for Network] Work task and Key issues of Network AI agent for 6G | China Mobile, Boost Mobile Network, KPN, SK Telecom, CATT |
| 2 | S2-2506396 | [WT#3, AI for 6G] Support AI-Native Architecture in 6G | Boost Mobile Network, EchoStar, China Mobile, Cisco Systems |
| 3 | S2-2506418 | [WT#3, Study how to support and enable use of AI in 6G (e.g. AI agent, framework)] Enabling use of AI in 6G | Interdigital |
| 4 | S2-2506422 | [WT#3, AI for Network] AI capable UE and network interaction | OPPO |
| 5 | S2-2506442 | [WT#3] Support and enable use of AI in 6G | CATT |
| 6 | S2-2506445 | [WT#3, 6G AI, AI4NET] Discussion and proposals for 6G native AI architecture | Vivo |
| 7 | S2-2506459 | [WT#3] 6G System Support for AI | Apple |
| 8 | S2-2506510 | [WT#3, Agent for Network] Proposals of the Use of AI Agent in 6G Architecture | China Mobile, SK Telecom, CATT |
| 9 | S2-2506511 | [WT#3, AI native] Key issues and justifications for 6G AI native | China Mobile, CATT |
| 10 | S2-2506571 | [WT#3, AI] AI for Network in 6G | Samsung |
| 11 | S2-2506572 | [WT#3] Work Task and Key Issues of WT#3 - AI | ETRI |
| 12 | S2-2506677 | [WT#3, AI in 6G] Key Issue on AI in 6G | NEC |
| 13 | S2-2506690 | [WT#3, AI framework/Agentic AI] AI framework for 6G | Qualcomm |
| 14 | S2-2506698 | [WT#3] Study how to support and enable use of AI in 6G | Nokia |
| 15 | S2-2506727 | [WT#3, all topics] Study how to support and enable use of AI in 6G | NTT DOCOMO |
| 16 | S2-2506788 | [WT#3, AI in 6G] Scope and Key Issues proposal for WT#3 – AI in 6G | IIT Bombay |
| 17 | S2-2506865 | [WT#3, AI in 6G] Scope and Key Issues proposal for WT#3 – AI in 6G | IIT Bombay |
| 18 | S2-2506866 | [WT#3 AI] New Key Issue for the use of AI in 6G | LG |
| 19 | S2-2506878 | [WT#3] Scope and Key Issues proposal to support Agentic AI in 6G | Google |
| 20 | S2-2506916 | [WT#3] Discussion and proposal for AI | ZTE |
| 21 | S2-2506937 | [WT#3] WT and KIs on how to support and enable use of AI in 6G | Ericsson |
| 22 | S2-2506963 | [WT#3] WT Scope discussion on AI framework and AI Agent | China Telecom |
| 23 | S2-2506970 | [WT#3] Key Issues for AI in 6G | TCL |
| 24 | S2-2506986 | [WT#3, NET4AI] 6G AI service for UE | vivo |
| 25 | S2-2507019 | [WT#3] Study how to support and enable use of AI in 6G (e.g. AI agent, framework) | Rakuten Mobile, China Mobile |
| 26 | S2-2507087 | [WT#3, AI in 6G] Key Issues on AI in 6G | Tencent, Tencent Cloud |
| 27 | S2-2507150 | [WT#3, AI Training Service], Support of AI Training Service in 6GS | Huawei, HiSilicon |
| 28 | S2-2507155 | [WT#3, Network Support for UE AI Agent Communication] UE AI Agent Communication | Huawei, HiSilicon |
| 29 | S2-2507196 | Proposed Key Issue for WT3: Study a Distributed Intelligence Framework in 6G | Lenovo |
| 30 | S2-2507223 | [WT#3, Network AI Agent] Agentic core network for 6G | Huawei, HiSilicon |
| 31 | S2-2507354 | [WT#3] KI on AI in 6G | Xiaomi |
| 32 | S2-2507359 | [WT#3] AI in 6G | Deutsche Telekom |

The following lists the content from the relevant contributions.

Text highlighted in yellow is author’s own interpretation that the content belongs to NET4AI (which could be updated per further online/offline corresponding company comments/suggestions)

2. KIs from the contributions

The following is the KI content related to AI for 6G architecture from the list of contributions mentioned above.

2.1	KI from China Mobile, Boost Mobile Network, KPN, SK Telecom, CATT (S2-2506362)

In 6G, it is essential to support endogenous AI capability, which can support users' time-varying, customized requirements with providing various functionalities and service operations. So that 6G network can dynamically adapt to diverse user demands, which may be expressed in various modalities such as natural language or intents.

AI agents within 6G are designed to interpret these demands, translate them into specific network performance and resource requirements, and match them with available network functionalities. This enables the network to provide services that align with users' QoS expectations and requirements on computing, sensing, etc, ensuring a more personalized and efficient service delivery with 6G network. At the same time, NFs need to be enhanced with AI capabilities for more intelligent actions.

The following aspects need to be studied:

-	Whether and how AI agent functionality can be introduced in 6GC to interpret the user requirements from UE or AF.

-	Whether and how network AI agent functionality can invoke the NF or NF services to fulfil some tasks from UE or AF.

-	How Network AI agent functionality support interactions with NF or NF services.

-	Whether and how to expose network capabilities through AI agent functionalities to UE or AF.

-	How to enhance 6G core network functions with AI capabilities (including AI agent).

-	If there are multiple AI agent functionalities in 6G system, how to support the communication between AI agents.

NOTE 2: Interoperability needs to be considered while AI capability or AI agent is introduced in 6G core network.

-	How to support AI/ML framework (including life cycle management and storage of AI/ML model, related data processing, etc.) inside 6G network for enhanced AI capabilities including AI agents.

NOTE 3: Coordination with data framework is needed.

2.2	KI from Boost Mobile Network, EchoStar, China Mobile, Cisco Systems (S2-2506396)

AI is becoming essential in every technology and every system. Therefore, the support of AI in 6G is required to enable 6G system utilize the potential and what AI can offer for enhancing the 6G system and network. This key issue proposes the detailed study for how to garnish AI capabilities in 6G and make AI as integral part of the decision making of every node of the 6G system.

6G is expected to support AI natively where AI in 6G is used for the 6G System performance enhancement while at the same time 6G system supports the communication between the different entities of AI across 6G system.

This key issue proposes to study the following aspects of supporting AI in 6G

-	Study how 6G system architecture can support AI natively where every node in the 6G system can utilize AI inference as part of its decision making.

-	Study how 6G system can support AI where every node in the 6G system is able to receive AI inference as immediate as possible without delay, something similar to as if the node is making an internal decision.

-	Study how 6G system can provide a data framework where all AI entities, e.g., AI Agents, across 6G system can communicate with each other, collect data, communicate its inference, store training data, and distribute data as needed.

-	Study how 6G system can support AI across the 6G system where AI inference can be provided to the UE, 6G RAN, and 6G Core network, i.e., end-to-end.

-	Study whether 6G system needs to support a centralized AI approach, e.g., using NWDAF, while being able to meet the immediate nature of AI inference without extended latency.

NOTE:	This key issue has a dependency on the data framework WT#5 and possibly require coordination with RAN WGs.

2.3	KI from Interdigital (S2-2506418)

Note: Although no specific KIs are provided, in Justifications parts, it is proposed to take new AI technologies e.g. Agent AI to potentially reduce CAPEX and increase operational efficiency and extend/evolve the 5G NWDAF-centric framework towards the distributed AIML in 6G by empowering the key network functions with built-in AI capabilities

2.4	KI from Oppo (S2-2506422)

In order to support the interaction between an AI capable UE and the 6GC to provide intent interaction friendly user experience, the following aspects need to be studied:

-	Overall architecture to support the efficient and reliable interaction between the AI capable UE and the 6GC.

-	How to discover the AI capable UE and AI capable NF with the different capabilities under the control of the 6G core network.

-	How and what configuration parameters are configured between the AI capable UE and the 6GC.

-	Whether and how the AI capable UE interacts with the 6GC over structured 3GPP intent.

2.5	KI from CATT (S2-2506442)

To support and enable use of AI in 6G, the following aspects will be studied:

-	How AI is used for 6GS:

-	6GS architecture model to support native AI, including e.g. defining new NF(s) and interfaces for 6G AI;

-	How to leverage AI in UE and CN NFs functionalities and procedures to improve system performance and service experiences, e.g. MM+AI, SM+AI, use of AI agent;

-	Coordination between UE, RAN and CN for use of AI in 6GS, e.g. data collection, performance monitoring;

-	How to leverage 6G data framework to support use of AI in 6GS, e.g. AI related data collection.

NOTE:	Coordination with WT#5 is required.

-	How 6GS is used for AI(NET4AI):

-	How to support AI applications and services, e.g. AI Agent applications.

-	How to provide AI services (e.g. AI/ML model inference) to 6G users (e.g. AF, UE).

2.6	KI from Vivo (S2-2506445)

This key issue aims to study how to support and enable native AI for Network (i.e. AI4NET) in 6G, including:

-	AI native architecture for 6G system

-	6G Architecture supports for native AI using, e.g. distributed AI with built-in AI capability 6G entities and/or uniform AI agent.

-	How to support interaction among e.g. built-in AI capability 6G entities and/or uniform AI agent.

-	Whether and how to support connectivity services and/or beyond connectivity services assisted by AI , and how to fall back to legacy mechanisms i.e. procedures without AI.

-	How to use unified data collection framework to support AI related activities e.g. data collection, data storage, data exposure, data privacy and security management, model sharing/delivery, etc.

NOTE: 	UE data collection in 5GA and 5GA data collection Framework for AI/ML will be considered.

-	Whether and how to define unified AI framework across SA/RAN e.g. Unified AI terminology/feature and Unified AI lifecycle management.

-	How to support multi-vendors AI Model sharing among 6G entities and/or UE.

2.7	KI from Apple (S2-2506459)

Note: only provided the updated WT and no specific KIs are provided

2.8	KI from China Mobile, SK Telecom, CATT (S2-2506510)

It is proposed to study the following aspects related to AI agents in 6GS:

-	Study how to define the network functionalities of AI agents, such as intent understanding, intent translation, task planning, network capability coordination, etc.;

-	Study how the network functions supporting AI agents interact with other network functions in 6GC, including the interfaces, the mechanism to ensure the security/reliability for interactions, etc.;

-	Study how to support the registration, discovery and update of network AI Agents;

-	Study how to coordinate the task execution among multiple AI Agents, including the interfaces, the task decomposition mechanism, etc.;

-	Study how the 6G system exposes network capabilities to support trusted third-party AI agents and terminal AI agents, including the interfaces, authorization/authentication mechanism, etc.;

-	Study how the network AI agents invocate the capability provided by trusted third-parties, including the capability registration, discovery and authentication mechanisms, etc.

2.9	KI from China Mobile, CATT (S2-2506511)

This Key Issue is to support and enable use of AI in 6G by enhancing capabilities of NFs with AI, related to WT#3. The main points are presented as follows.

KI#1: AI-native 6G network architecture

-	Whether and how to embed AI capabilities into NF, including which NFs can be embedded with AI capabilities and which kind of AI capabilities can be embedded into NF, considering potential use cases.

-	Whether and how to support collaboration between UE, RAN and NFs embedded AI capabilities.

2.10	KI from Samsung (S2-2506571)

-	What architectural enhancements will be required to support AI-powered core network function (e.g. for AIML model training and/or inference)?

-	What AI related information and how will it be transferred between AI-powered NFs, e.g. AI operation related information, AI training results or AIML inference results?

-	How to support cross-domain interactions between AI-powered NFs, RAN node, UE for AIML operation?

-	How to support data collection and transfer for AI operations by network functions, and what data will be collected?

NOTE:	if data management framework is used for AI related data collection and transfer, alignment between WT#3 and WT#5 will be needed.

-	How to support AI capability exposure from 6G system to 3rd party?

-	How to support AI related information exposure to 3rd party, e.g. exposure of AI operation related information, AI training results or AIML inference results?

2.11	KI from ETRI (S2-2506572)

KI: Network automation and optimization using AI agents

This key issue aims to investigate how AI agents can be leveraged to enable end-to-end network automation and optimization across domains of the 6G system, including the RAN, CN, and UE.

It addresses the “AI for 6G” perspective, where distributed AI agents perform autonomous control of NFs for end-to-end network automation and optimization across domains of the 6G system.

This key issue will investigate the following aspects:-

-	How to support full AI lifecycle frameworks that allow AI agents to coordinate automation and/or optimization tasks across RAN, CN, and UE

-	Whether and how to support integration with network data analytics functions (e.g., enhanced NWDAF) for full AI lifecycle support — from data collection and training to inference, feedback, and continuous performance improvement

KI: Efficient operation and collaboration of distributed AI agents (NET4AI)

This key issue aims to investigate architectural enhancements and functional enablers to support the efficient operation and intelligent collaboration of distributed AI agents in 6G systems.

It addresses the “6G for AI” perspective, where the 6G system provides foundational support to enable autonomous, context-aware, and coordinated AI agent behaviors

This key issue will investigate the following aspects:

-	How to support architectural enablers required for efficient communication, coordination, and orchestration among distributed AI agents.

-	How to support cross-domain policy control to support AI agent collaboration.

-	Whether and how to support AI-aware QoS mechanisms, enabling the network to dynamically prioritize AI traffic based on the inference criticality, latency tolerance, and model lifecycle stage (e.g., training vs. inference).

-	Whether and how to support integration resource allocation strategies (compute, storage, communication bandwidth) optimized for agent workloads and their dynamic needs in edge/cloud environments among AI agents

-	Whether and how to support context-aware traffic control functions, allowing distributed AI agents to adapt their behavior based on situational awareness (e.g., mobility, congestion, energy constraints, user intent, etc.).

2.12	KI from NEC (S2-2506677)

Study on weather and how to support AI Agent as an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

2.13	KI from Qualcomm (S2-2506690)

This key issue will address:

-	The different architectures of ML model function and inference function in 6GC.

NOTE 1:	Whether and how to perform local inference by a 6G network function is an implementation decision and beyond the scope of 3GPP.

-	Whether and how to enhance the ML model provision procedure based on the corresponding procedures defined in TS 23.288 [x].

-	Whether and how to enhance the performance monitoring procedure based on the accuracy monitoring defined in TS 23.288 [x].

-	Whether and how to enhance the multi-vendor interoperability based on the corresponding mechanism defined in TS 23.288 [x].

NOTE 2:	Data collection as defined in TS 23.288 is assumed to be sufficient for model training. Allowing network functions other than NWDAF to act as a consumer for data exposed by other network functions, e.g. for the purpose of model training, is not precluded.

NOTE 3:	Agentic AI/AI Agent is a solution that may be proposed for specific problems and use cases. For this reason, agentic AI is not covered by this WT. Instead, agentic AI-based solutions may be proposed as solutions for other WTs if deemed appropriate.

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

2.14	KI from Nokia (S2-2506698)

Solutions to this key issue will study below aspects on how to support and enable use of AI in 6G:

1. 	Whether and how to support the use and integration of ML models within the NFs in the core network for 6G system. .

a.	Whether and what ML model provisioning and ML model training functionalities, services and procedures are needed.

b.	Whether and what ML model inference functionalities, services and procedures are needed.

c.	Whether and what ML performance monitoring functionalities, services and procedures are needed to ensure the reliability and accuracy of ML model outcomes.

NOTE 1: Services related to ML model provisioning, ML model Training, ML model analytics and ML model monitoring are specified for NWDAF in 5G. Where needed, existing services should be taken as the baseline to support the above functionalities.

2.	Whether and how to evolve NWDAF in 6G, including whether the existing analytics should remain within the NWDAF or be handled by other core NFs.

3.	What AI/ML capabilities should be exposed to 3rd party AFs, beyond the existing NWDAF analytics exposure defined in 5G.

4. 	Whether and how to support the use and integration of AI agents within 6G Core NFs.

5.  Whether and how to support AI agent enabled 3rd party AF to obtain services offered by 6G core network via exposure framework.

NOTE 2: Interaction between AI Agents, e.g., AI Agents as part of a NF or AF, is out of 3GPP scope.

6.	How to support energy-efficient and sustainable usage of AI/ML.

NOTE 3: Coordination with WT#5 should be ensured on AI/ML related data aspects, where applicable. WT#3 identifies the requirements for AI/ML data, while WT#5 specifies the corresponding data handling mechanisms.

NOTE 4: Coordination with SA3 is required for any security and privacy related topics.

NOTE 5: Coordination with SA5 is required for any OAM and Charging related topics.

NOTE 6: Core NFs without AI/ML capabilities must be able to interwork with Core NFs with AI/ML capabilities.

2.15	KI from NTT Docomo (S2-2506727)

KI: ML Model Handling Framework in 6GC

This KI will study the following aspect of the ML Model handling framework in 6GC:

-	Whether and how to support customized ML model provisioning/deployment in 6GC NF functions

-	Whether and how to support ML model evaluation and operation visibility in 6GC NF functions

-	Whether and how to support ML model (re)training management in 6GC NF functions

-	Whether and how to support interoperability for model provisioning, training and monitoring in the multi-vendor deployment scenarios

KI: AI-Native NFs in 6GC

This KI will study the following aspects of AI-native NFs in 6GC:

-	Whether and how to support decision making by ML models in 6GC network functions

-	Whether and how to support evaluating the impact of decision by ML models in 6GC NFs

-	Whether and how to support model (re)training based on the evaluation of network performance

KI: Reliability and Sustainability of AI/ML in 6GC

This KI will study the following aspects of reliability and sustainability of AI in 6GC:

-	Whether and how to assess reliability and sustainability of AI/ML in 6GC

-	Whether and how to improve reliability of AI/ML in 6GC

-	Whether and how to improve sustainability of AI/ML in 6GC

KI: Supporting AI/ML based Applications on 6GC (NET4AI)

This KI will study the following aspects of supporting AI/ML based applications (e.g. AI Agent) on top of 6GC:

-	Identify AI/ML based application requirements (e.g., communication, computation, data) to be deployed on top of 6GC considering the use cases and requirements documented by SA1.

-	Identify architectural enablers to address the AI/ML based application on top of 6GC

-	Whether and how to design architecture to support deployment of AI/ML based application on top of 6GC

-	Whether and how to address privacy, security and charging aspects of services provided to AI/ML based application on top of 6GC

-	Whether and how to address user consent and application layer traffic identification.

2.16	KI from IIT Bombay (S2-2506788)

KI: AI Framework to support “AI for energy”

This key issue aims to study an AI/ML framework to support energy optimization in 6G systems.

KI: Energy information collection for AI/ML framework

This key issue aims to study how an AI/ML framework can collect relevant energy information. It also includes identifying which network functions can provide energy information.

KI: Energy information granularity for AI enabled optimization

This key issue aims to identify the granularity level (user level, service level, etc.)  for energy information to support AI-enabled optimization  in the 6G network.

2.17	KI from IIT Bombay (S2-2506865)

This key issue aims to provide solutions for use of AI/ML methods in 6G. Following aspects are considered:

-	Introduce intelligence in the core network to learn about stationary or low mobility users

-	Use AI/ML predictions to reduce paging area size for UEs in idle mode

E.g., ML model can be trained with information pertaining to UEs who are stationary or less mobile during work weeks. Based on their emerging behavioral pattern, paging can be limited to fewer RAN nodes than the entire Tracking Area/s (TA/s).

-	Use AI/ML predictions to reduce paging area size to support SMS delivery for UEs in idle mode

-	Use AI/ML predictions to reduce frequency of periodic location updates from predominantly stationary UEs

2.18	KI from LG Electronics (S2-2506866)

This study focuses on how to support and enable use of AI in 6G system, considering both ‘AI for 6G system’ and ‘6G system for AI’.

This Key Issue aims to study as follows:

-	How to support the high-level architectural aspects of the unified AI-enabled framework cross WGs (based on end-to-end AI Life Cycle Management)

NOTE 1:	This KI requires coordination with SA5 and RAN WGs from the LCM perspective.

-	How to support AI related logical functions (e.g. inference function, model training function, performance monitoring function, etc.)

-	How to support the high-level procedures including the initiation of AI model training and inference, reporting of AI inference results, AI model change based on performance monitoring, etc.

-	How to support AI-operations awareness in user plane

NOTE 2:	This KI requires consideration of user plane architecture studied in WT#1.2.

-	How to control the unified data related services (e.g. initiation, termination of data transfer, etc.) with a focus on AI aspects.

NOTE 3:	This KI requires consideration of Data framework studied in WT#5.

-	How to support QoS dynamically for AI operations/AI services with policy provision for AI aspects

NOTE 4:	This KI requires consideration of QoS framework and Policy framework studied in WT#1.2.

-	How to enhance AI specific requests from AF and the notification on the requested event and/or service performance monitoring, etc.

NOTE 5:	This KI requires consideration of network exposure framework studied in WT#1.2.

2.19	KI from Google (S2-2506878)

This key issue focuses on how to enable:

-	Intelligent application-network interaction through Agentic AI based architecture that comprehends both application service requirements and underlying network capabilities, facilitating seamless translation between application-level requirements and network service parameters.

-	Autonomous service optimization based on real-time network conditions and predictive analytics that enable dynamic resource allocation

-	AI-driven abstraction layers that encapsulate 3GPP API complexity, providing simplified, intuitive interfaces for application developers while maintaining full network capability access and preserving existing network function interactions

-	Proactive resources management utilizing Agentic AI for network services adaptation, e.g. adaptive QoS provisioning.

Solutions for this key issue will describe, based on the WT scope described in Annex A.X:

-	Whether and how to architect AI-native framework that integrate Agentic AI architecture within 5GS/6GS to provide intelligent mediation between third-party applications and network services for network capability exposure, including the protocol and service interfaces for Agentic AI communication.

-	Whether and how to design Intelligent Abstraction mechanisms that provide AI-driven translation from high-level application requirements to specific network services parameters and generate adaptive service composition that dynamically optimize network resource allocation.

-	Whether and how to establish comprehensive security and privacy framework for Agentic AI based network capability exposure to third parties that ensure secure, scalable, and interoperable Agentic AI operations across diverse 6G deployment scenarios.

2.20	KI from ZTE (S2-2506916)

It is proposed to study the following key issues:

KI: 6GS to support AI Agents in the UE (NET4AI)

1.1 Whether and how to define and identify AI Agent in device, considering potential use cases.

1.2 Whether and how to support the operator-controlled network access for AI agent in device (e.g. registration and authentication of AI Agent devices).

1.3 Whether and how to support communication between AI Agent in device (e.g. session management, QoS control, group communication, AI Agent discovery etc.)

1.4 Whether and how to apply AI technology in the UE for connectivity service.

KI: AI in 6GS

2.1 AI Agent in network, this includes the following aspects:

-	Whether and how to define the functionalities (e.g. perception and reaction) of AI agent as separated Network Functions, considering potential use cases.

-	Whether and how to use service-based interface or define different interfaces for communication between AI agents, between AI agent(s) and other NF(s).

-	Whether and how to support network exposure towards AI Agents in 3rd party.

-	Whether and how to improve output quality of AI Agent using advanced technology (e.g. RAG).

2.2 NF embedded AI capability

-	Whether and how to support AI native, considering potential use cases.

2.3 Enhance ML model management

-	Whether and how to enhance ML model management (e.g. a more complete LCM, transfer&online learning, AI capabilities exposure to 3rd parties, NF and UE).

2.21	KI from Ericsson (S2-2506937)

KI: Support for AI in 6G Core entities (framework)

The objective of this Key issue is to study which 6G features and functionalities, being defined in WT#1 (Define the overall 6G architecture), can be enhanced by using native AI in 6G entities. This KI will study whether it is an issue and how to avoid that conflicts may arise e.g. when different 6G entities optimize towards their goals. Standardized data used for inference is to be studied. If the inference is internal to 6G entities and not exposed externally then the output is not to be specified.

NOTE 1: The data collection is performed according to the conclusions of WT#5 (Study data framework) as such is low priority in this sub-WT.

NOTE 2:	This key issue may require coordination with SA5 for coordination of AI in 6G entities to prevent conflicts.

NOTE 3:	The Model LCM is low priority in this key issue and may depend on SA5.

KI: Vertical Federated Learning between 6G Core entities (framework)

When deploying and training a ML Model into a 6G entities, the existing Federated Learning techniques that are defined in 5GC for model training are also applicable for 6G entities. This key issue studies if and how Vertical Federated learning can be used between 6G entities to perform model training, monitoring and inference using the locally available data for this purpose. This key issue does not need input from WT#5 (Study data framework) as the training and inference is performed with the locally available data at the 6G entities but depends on the outcome of Key issue #x on AI in 6G entities.

NOTE:	The data collection is performed according to the conclusions of WT#5 (Study data framework) as such is low priority in this sub-WT.

KI: Applicability of learning using Closed-loop AI techniques in 6G Core (framework, AI agents)

The objective of this key issue is to study whether and how Closed-loop AI techniques can be applied for 6G features and functionalities, being defined in WT#1 (Define the overall 6G architecture). This key issue will cover reinforcement learning (RL) as one of these closed-loop techniques, where an agent is placed inside an 6G entity and learn to make decisions and maximize a reward within its responsibility. This agent is believed to include AI and work as an AI agent. It is related to the native AI in 6GS studied in Key issue #x.

NOTE 1:	Closed-loop AI techniques in this context refer to a 6G Core entity that continuously monitors, evaluates, and adjusts its actions based on data and feedback received internally or externally to the 6G entity. This creates a feedback loop that enables the 6G entities to improve its performance over time and adapt dynamically to changing conditions. Some closed loops may be entirely inside one 6G entity and will in this case impose very little or no impact in standardization.

NOTE 2:	This KI may place some OAM requirement for the implementation of the closed loop therefore coordination with SA5 may be needed.

2.22	KI from China Telecom (S2-2506963)

KI: Key issue on AI framework

This key issue is proposed to study an AI framework with studying and considering the following aspects:

‐	How to support AI built-in NFs to enable AI-based decision-making, with corresponding input and output.

‐	Define a unified Life Cycle Management (LCM) framework to support model training, inference, performance monitoring, evaluation, testing, etc.

‐	How to support cross-domain AIML, including interaction among UE, RAN, CN and the 3rd party server.

‐	How to support multiple AI algorithms (e.g. transfer learning, reinforcement learning, continual learning, and RAG) applied and multi-vendor interoperability.

KI: Key issue on AI agent (AI Agent for NET + NET for UE AI Agent)

This key issue is proposed to study whether and how to support AI Agent enabled network and AI Agent communication network, the following aspects are studied and considered:

‐	Whether and how to define unified identity of the on-device AI Agents.

‐	Whether and how to support registration, discovery, authentication and authorization of the on-device AI Agents.

‐	Whether and how to define the functionality of network AI Agent(s).

‐	Whether and how to support efficient communication between network AI Agents, between network AI Agent and on-device AI agent, between on-device AI agents.

NOTE X: The aspects on cross-domain AI may require coordination and alignment with RAN and SA5.

NOTE Y: The aspects on security may require coordination with SA3.

2.23	KI from TCL (S2-2506970)

KI: AI Agent Capability Management

Different AI agents have different capabilities and are capable for different tasks. This information shall be exposed to the potential users of the AI agents. The potential users include network operator, network user, user equipment, RAN nodes, core network elements, 3rd party network elements and so on. The following issues shall be solved:

1.	Where and how to store the capability of the AI agents

2.	How to expose the capability of the AI agents to the potential users

KI: AI Task Lifecycle Management

Different from traditional telecom network functions, AI task can be very flexible. There is no fixed method for a user to create a task. The task can come from an intent of a user in the method of natural language. The 6G network need to introduce the ability to translate intent into Agent operable tasks and orchestrate the agents in the network to perform the task. The object and requirement of the task shall be commonly understood among agents with different implementations.

Also the task shall be performed with dedicated quality by dedicated nodes to provide deterministic result to fulfil the high reliability and robustness requirement of the telecom network.

Hence, follow issues shall be solved:

1.	A new service or functionality introduced to the network to translate users’ intent and orchestrate the network to perform the task based on the intent.

2.	A unified task defining method or task assigning method to promise the task can be performed correctly by agents with different implementations.

3.	How to design AIQoS to control the high-level AI task performing quality. The detailed parameters can include latency requirement, resource consumption requirement, and energy consumption requirement.

4.	New procedures for the whole AI task life cycle management including task assign/supervise/update/management.

KI: Network Resource Exposure

To perform the task, AI agent needs to get more resource besides the services provided by the core network functions. The resources required include data/working status/configuration API and so on. In addition, agent may require resources from RAN node, user equipment, or 3rd party network elements. These network elements are not included in the SBI interface in current network. Follow issues shall be solved:

1.	How to introduce a unified mechanism for any network elements to expose their resource to the agent

2.	How to manage the information of the resources exposed in the network. How agents discover these resources when needed?

3.	Security and privacy issue of the network resources. The network resources shall only be exposed to the authorized agent which is allowed to handle dedicated private data and information.

4.	How to design the interface for agent to visit and use the network resources? Is a new interface necessary? How to reduce the complexity of the interfaces on the network elements?

KI: Connection Management for AI Agents (NET4AI)

Agents need more flexible connection across user equipment, RAN nodes, core network functions, and 3rd party network elements since the AI agent can be deployed on anyone of them.

The connection quality shall also be controlled to fulfil the high-level AIQoS corresponding to the task to be performed.

Following issues shall be solved:

1.	How to establish flexible connection between user equipment, RAN nodes, core network functions, and 3rd party network elements? Shall it be long connection or short connection? Shall it be established based on task? Shall these connections be independent ones or be combined with legacy control plane/user plane connections?

2.	How to control the connection quality to fulfil the high-level AI task quality control? Which network function or element shall be responsible for such control? How to design the parameters for the QoS of these new connections?

2.24	KI from Vivo (S2-2506986)

This Key Issue includes:

-	How to support AI as a service (NET4AI)

-	How to support control of AI model with coordination with AF, including AI model training, AI model storage, and etc.;

-	How to support selection of AI model for user AI service;

-	How to meet the service requirement for user AI service；

-	How to support task based service, including：

-	How to interpret the service request for task, including intent-based request and non-intent based request；

-	How to coordinate network NFs to fulfill the service performance requirement of task;

-	How to support AI agent communication and collaboration (NET4AI)

-	How to support to support communication for multiple AI agents, including group communication for AI agents；

-	how to support the management of AI agent attribution, including attribution registration, security identification, discovery of AI agent by authorized entity.

Note: 	Benefits and applicable scenarios of 3GPP based AI agent management need to be studied and evaluated.

-	How to support charging for AI agent group communication

2.25	KI from Rakuten, China Mobile (S2-2507019)

Whether and how to study the architectural enhancements required to enable AI agent-based capability exposure in 6G Core, including interactions with third-party AI agents, intent translation, and any potential impacts on service-based interfaces

2.26	KI from Tencent (S2-2507087)

It is proposed to study the following key issues:

-	Whether and how to support AI Agents in network entities including 6G UE, 6G RAN, 6G CN and interaction with AI agents within authorized 3rd party to support AI agents for the potential customers?

-	Whether to introduce AI agents as standard alone NF or embeded within other NFs?

-	Whether and how to support AI agent identification in UEs and expose to other entities? (NET4AI)

-	Whether and how to support communication among AI agents distributed in UE, 6G RAN, 6G CN and authorized third party?  (NET4AI)

2.27	KI from Huawei (S2-2507150)

This key issue proposes to study how an operator can provide AI training service. The support of AI training service may consist in the operator assisting the AI training by generating and providing training data or in also supporting the actual training of the third party AI model and expose the trained AI model to the user. The first step is to collect data and generate the training data according to a set of agreed criteria that depends on the AI model to be trained. If the operator is performing or assisting the training of the third party AI model, then besides the generation of the data the operator also takes care of exposing the generated training data to the AI model and to expose the trained AI model to the third party.

The solutions for this key issue should address the following aspects

-	How to acquire the data used to generate AI training data

NOTE:	Coordination with WT#5 may be needed.

-	How to generate AI training data that meet the requirements of the AI model to be trained. The non-exhaustive list to take into consideration include:

-	input and output data structure

-	time and location of the data collected

-	quality of the data

-	How to expose the AI training data taking into consideration:

-	location where the AI training data to be exposed

-	time when the AI training data to be exposed.

-	How to enable the provisioning of the AI model and parameters (e.g., AI model training method, target accuracy) from a third party, UE or MNO for AI model training.

-	How to perform and optimize the AI model training based on the service requirement

-	How to expose the trained AI model taking into consideration:

-	location where the trained AI model to be exposed

-	time when the trained AI model to be exposed

-	target accuracy that trained AI model has reached.

2.28	KI from Huawei (S2-2507155)

(NET4AI)

UE AI Agent is expected to effectively perform diverse tasks through self-directed planning and actions, it can directly connect and operate via 3GPP and/or non-3GPP access and get communication service from mobile network. A group of UE AI Agents could collaborate with each other to finish a temporary task. Network is expected to provide efficient communication for UE AI Agent and for which network need to identify each UE AI Agent for authentication and authorization, and also provide UE AI Agent discovery based on UE AI Agent attribute(s).

A UE AI agent can take on various forms, such as embodied intelligent robots, virtual assistants running in UE, drones and even in some scenario the AI agent may be offloaded from UE to Service Hosting Environment.

Study the architecture to support communication of UE AI Agents. Following aspects shall be considered and studied:

-	Study an identity and profile management for UE AI Agent.

-	Study how to support discovery of UE AI Agent for task, e.g., based on intent.

- 	Study how to develop UE-Core Network interaction to support UE AI agent communication.

- 	Study how to support efficient communication for UE AI Agent (e.g. QoS treatment, LAN-based group communication, communication in case of NAT etc.), including the cases such as UE AI Agent communicating with application service, UE AI Agents communication of two or a group of UE AI Agents.

NOTE 1: the security related issues are to be coordinated with SA3.

2.29	KI from Lenovo (S2-2507196)

Introduction

The 5G System introduced the NWDAF framework, which enables data collection and providing historical / predictive analytics to other network functions. The architecture supports distributed deployments (e.g., central, regional, edge) and collaborative model training (using FL), which are critical elements for network optimization.

While the NWDAF provides the necessary analytics, the 6G system requires a deeper integration of intelligence to achieve full, real-time autonomy. The goal is to evolve from a network that is analytics-driven to one that is intent-driven and cognitive. This requires a framework where intelligence is not just a service to be consumed, but a native capability with the power to act autonomously.

As part of this key issue it is proposed to study the evolution from the 5G analytics framework to a 6G Distributed Intelligence Framework (DIF), which enables autonomous, intent-driven operations.

Motivation

The NWDAF framework successfully distributes the analytics capability. However, the decision to act on these analytics still resides with the consuming network functions (NFs) or the OAM system. This separation between "analysis" and "action" can introduce latency and complexity, limiting the network's ability to react in real-time to dynamic conditions.

To meet the 6G goals of a lean, efficient, and fully autonomous network, the architecture must support "autonomous agency". This means embedding intelligent entities (e.g., AI Agents) directly into the core network architecture that can perceive the network environment, reason, and execute actions to achieve a high-level goal (an "intent").

The required evolution from 5G to 6G intelligence can be summarized as shown in the table below.

| Aspect | NWDAF in 5G/5GA | Distributed Intelligence Framework in 6G |
| --- | --- | --- |
| Primary role | Provides Analytics-as-a-Service. Other NFs consume analytics and then decide how to act. | Provides Intelligence-as-a-Native-Function. AI Agents are architectural components with the autonomy to act directly. |
| Operational model | Analytics-driven. The action is separated from the analysis. Intelligence is an "add-on" capability. | Intent-driven. An AI Agent receives a high-level goal and autonomously coordinates NFs to achieve it in a fast, closed loop. |
| Architectural Focus | Focuses on distributing the data analytics capability. | Focuses on distributing the autonomous agency capability. |

As part of this key issue for WT3 the following aspects should be studied:

-	Study how an AI Agent should be defined in the 6G architecture (e.g., as a new, standalone NF, or as an embedded capability within other NFs).

-	Study the functions and responsibilities of an AI Agent.

-	Study the necessary interfaces for an AI Agent to interfact with other NFs for the purpose of coordinating NFs to achieve a desired goal.

-	Study how an AI Agent receives intents (e.g., from authorized third parties).

-	Study support of AI/ML operations for AI agents to achieve a requested intent

-	Study whether and how an ML model used by an AI agent is trained to support a requested intent. As part of this objective study how reinforcement learning and online learning are supported within the 6G core network

-	Study whether and how a trained ML model to support a specific intent is provisioned to an AI agent

NOTE x:	Data collection aspects are covered by WT5

NOTE y:	The AI/ML operations defined for the NWDAF framework will be used as a basis.

-	Study the impact on 6G NFs, e.g., how the SMF or PCF would need to evolve to support interaction with an AI Agent.

2.30	KI from Huawei (S2-2507223)

This key issue will address the Network AI Agent aspects within the 6G system architecture. This key issue will attempt to develop a comprehensive network AI Agent framework for the 6G system:

-	Functionalities of network AI agent to support connectivity and beyond connectivity service:

-  How to define network AI Agents and their functionalities,

-  How to allow network AI Agents to access and support connectivity and beyond connectivity services,

-  How to support the registration and discovery of network AI Agents,

-	How to allow network AI Agents to collaborate to fulfil complex tasks,

-	How to allow network AI Agents interact with Data framework regarding data access, processing and other operations to fulfil the data requirements,

-	How to allow network AI Agents to access computing resources to fulfil the computation requirements,

-	How to allow network AI Agents to invoke sensing processing.

NOTE: the granularity of network AI agent is to be further clarified as part of the solutions to the key issue.

-	Interactions between network AI agents and other entities:

-	other network AI agents,

-	How Network AI agent interact with UE or 3rd party entities, e.g, based on intent.

-	How Network AI agent interact with other network functions.

2.31	KI from Xiaomi (S2-2507354)

The study on AI in 6G should cover how to support legacy 5G AI use cases, and how to support new use cases for 6G AI to evolve with the target of reaching AI-native 6G.

Study on AI in 6G is proposed to be organised with two aspects: AI for 6G System and 6G System for AI

The following questions need to be addressed during the study on AI in 6G:

-	How to support AI for 6G System, including:

-	how to support legacy 5G AI use cases on AI for 6G System;

-	how to support new requirements on AI-enabled network function;

-	how to support new requirements on AI for O&M;

-	how to support network AI agent on AI for 6G System;

-	How to support 6G System for AI, including: (NET4AI)

-	how to support legacy 5G AI use cases on 6G System for AI;

-	how to support AI service exposure and workload offloading on 6G System for AI;

-	how to support AI agent on 6G System for AI;

-	how to support Energy Efficiency on 6G System for AI;

-	how to support new QoS/QoE requirements for AI service.

NOTE 1:	This requirements for legacy use cases (enhancement) or new use cases are all based on the conclusion in SA1.

NOTE 2:	Cooperation with the following WTs with dependency is needed: Common data framework in WT#5 for data handling, Computing in WT#6 for AI service exposure and workload offloading.

2.32	KI from Deutsche Telekom (S2-2507359)

This key issue will study architectural solutions to enable trustworthy, efficient, and controlled integration of AI capabilities across the 6G system. The focus will be on architectural enablers for model transparency, monitoring, reliability, explainability, interoperability, and distributed execution. The aim is to ensure AI can be safely integrated into the system while maintaining operator control, fulfilling performance requirements, and supporting future evolutions such as agent-based intelligence.

Solutions addressing this key issue will cover:

-	Identification of the functional elements, which can be enriched by AI.

-	Defining necessary interfaces, functional elements and functional models to support transparent, controllable, and reliable AI integration in 6G architecture.

-	Define what AI-native and AI-native NFs are.

-	Study how the 6G system can support AI model transparency, including the availability of information related to model behaviour, model data usage, and model lifecycle.

-	Define interfaces supporting AI explainability.

-	Study mechanisms for operators to control and configure models, including fallback to non-AI-native modes for critical functions, e.g. in case the system becomes instable (due to AI).

-	Define and/or enhance AI model confidence assessment, model testing and validation.

-	Study how to enable multi-vendor-interoperability despite NF integrated AI.

-	Study enablement of AI model metadata exchange across NFs.

-	Investigate methods to enable model interoperability with different AI and MLOps platforms.

NOTE w: in coordination with SA5

-	Investigate mechanisms to coordinate and offload AI model execution across various entities (e.g., UE, RAN, core, edge, application servers).

NOTE x: in coordination with RAN and SA5

NOTE y: Dependency on WT#6.

-	For Agentic AI Enablement and Control:

-	Define agentic AI.

-	Study whether and how intelligent autonomous agents can be supported, safely integrated into the 6G system architecture and tested considering operator-defined constraints, interactions with network functions and services and how decisions can be logged and audited.

-	Investigate the integration of Retrieval-Augmented Generation (RAG) techniques as an architectural enabler to provide dynamic, explainable, and knowledge-driven AI behavior across the 6G system.