SA WG2 Meeting #S2-171	S2-2508414

13 - 17 October, 2025, Wuhan, China

Source: 	vivo

Title: 	Interim agreements on KI#1: System Architecture to Support Sensing

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides interim agreements on KI#1: System Architecture to Support Sensing.

1	Discussion

At June 2025 SA#108 meeting, R20 5GA SA2 ISAC (see approved SID SP-250833) was down-scoped into gNB-based sensing to aligned with Rel-20 5G-A RAN ISAC study scope. Based on this, we could have:

Observation#1: Only gNB can be performed as Sensing Entity (SE).

Up to August 2025 SA2#170 meeting, 12 Solutions was agreed for KI#1 and captured in TR 23.700-14 V1.0.0.

After analysing and evaluating the architecture proposals, new NF function definitions, existing NF impacts and end-to-end procedure definitions captured in agreed solutions for KI#1, following common aspects can be identified and could be captured as agreed principle for KI#1:

Observation#2: A new CN function Sensing Function (SF) is introduced to support Sensing service which provides following key functionalities:

- NF profile registration for the discovery of SF instances, e.g., by an NEF.

- Handling Sensing service request from Sensing service consumer.

- Sensing service request authorization and revocation.

- Sensing Entity selection and re-selection.

- Sensing data processing and Sensing result generation/exposure.

- Sensing configuration parameters provision.

Observation#3: The Sensing Entity (i.e. gNB) provides following key functionalities:

- Sensing service operations to collect the Sensing data.

- Sensing data report to the SF.

Observation#4: The NEF, if involved, needs to support following key functionalities:

- Handling Sensing service request from the AF.

- SF discovery and selection.

- Sensing service request authorization.

Observation#5: Both direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF) are considered for 5G system architecture to support sensing.

It proposes to capture above observations as interim agreements on KI#1: System Architecture to Support Sensing.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14 V1.0.0.

# * * * * First change* * * *

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

The following interim agreements on KI#1: System Architecture to Support Sensing, are made:

1.	General

a.	A new core network function Sensing Function (SF) is introduced to support Sensing service in 5G system architecture. The SF performs the following functionalities:

-	The SF registers itself in the NRF with its NF profile, this is to enable the discovery of SF instances e.g. by an NEF.

-	The SF receives a Sensing service request from the AF. If the AF is deployed in an untrusted third part, The Sensing service request will firstly send to the NEF and then send to the SF.

-	The SF performs Sensing service authorization and Sensing service revocation. More details of Sensing service authorization and revocation are captured in the interim agreements on KI#2.

-	The SF discovers and selects gNB as Sensing Entity (SE) and triggers the gNB to perform Sensing service operations towards the environment and/or objects. More details of SE selection are captured in the interim agreements on KI#3.

-	The SF receives Sensing data from the SE and performs Sensing result generation. The SF further exposes the Sensing results to the AF. More details of Sensing data collection, Sensing result generation/exposure are captured in the interim agreements on KI#4 and KI#5.

-	The SF provisions required Sensing configuration parameters to the SE. More details of Sensing configuration parameters provision are captured in the interim agreements on KI#6.

b.	Only gNB can be performed as Sensing Entity (SE). The SE performs the following functionalities:

-	The SE receives a Sensing service request from the SF and then performs Sensing service operations to collect the Sensing data.

-	The SE reports the collected Sensing data to the SF for Sensing result generation.

c.	In case of the AF is deployed in an untrusted third part, the NEF needs to be enhanced to support the following functionalities:

-	The NEF receives a Sensing service request from the AF, discovers and selects the SF for Serving Service request, and then sends the Sensing service request to the selected SF.

-	The NEF performs Sensing service authorization to decide whether the AF is allowed to request the Sensing service.

d.	Both direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF) are considered for 5G system architecture to support sensing.

2.	Direct connection architecture

When the SE and the SF communicate directly for sensing signaling:

-	The SF communicates with the SE via a new direct interface reference point Ny for both sensing signaling and sensing data.

NOTE 1:	The protocol stack used between the SF and the SE over Ny will be concluded by RAN WG3.

-	This reference architecture contains below new service-based interfaces:

Nsf:	Service-based interface exhibited by SF.

-	Figure 7.1.1-1 below shows the aspects related to direct connection architecture in service-based representation.

Figure 7.1.1-1: 5G System Architecture to Support Sensing (Direct Connection)

-	Figure 7.1.1-2 below shows the aspects related to direct connection architecture in reference point representation. A new reference point Nz is defined between the SF and the NEF.

Figure 7.1.1-2: 5G System Architecture to Support Sensing in reference point representation (Direct Connection)

3.	Indirect connection architecture

When the SE and the SF communicate indirectly for sensing signaling:

-	The SF communicates with the SE via the AMF for sensing signaling by invoking the service provided by the AMF.

NOTE 2:	Whether to enhance an existing AMF service or define a new AMF service will be determined in the normative phase.

-	The SF communicates with the SE via a new direct interface reference point Ny for sensing data.

NOTE 3:	The protocol stack used between the SF and the SE over Ny will be concluded by RAN WG3.

-	This reference architecture contains below new service-based interfaces:

Nsf:	Service-based interface exhibited by SF.

-	Figure 7.1.1-3 below shows the aspects related to indirect connection architecture in service-based representation.

Figure 7.1.1-3: 5G System Architecture to Support Sensing (Indirect Connection)

-	Figure 7.1.1-4 below shows the aspects related to indirect connection architecture in reference point representation. A new reference point Nx is defined between the SF and the AMF. A new reference point Nz is defined between the SF and the NEF.

Figure 7.1.1-4: 5G System Architecture to Support Sensing in reference point representation (Indirect Connection)

# * * * * End of change* * * *