SA WG2 Meeting #171	S2-2508518

13 - 17 October, 2025, Wuhan, China

Source:	Samsung

Title:	[WT#5 Data framework] WT scope and KI proposal

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution summarizes all contributions for work task scope and key issues for WT#5 on 6G Data framework.

1. Discussion

Architectural Motivation

In 5G system, the followings are supported: network analytics service provided by NWDAF, AIML-based operation (e.g., LMF-based AI/ML Positioning) and non-connectivity data-driven service (e.g., LCS). Accordingly, the 5G system supports the data management, e.g., including collection of data from network functions, UE and RAN node.

- Handling more various and larger data within the system: In 6G, it is expected that the system would generate, utilize and expose more various data with emergence of various network analytics, use cases for AIML-based operations (as SA WGs and RAN WGs would specify further use cases), and additional non-connectivity data-driven services (such as sensing and computing). Especially, support of sensing data transfer or AIML data collection will require the system to be capable of handling the larger data volume.

- Minimization of adverse impact on conventional operations for connectivity services: As data processing and transmission volumes continues to grow, any potential negative impact or degradation to the conventional connectivity service operation of existing UE/RAN/NFs must be prevented to maintain connectivity service quality and stability. For example, stability and performance for signaling exchange among UE, RAN, and NFs should not be impacted due to data collection and processing operations.

- Regulation-compliant data handling in terms of privacy protection: Recently, Global regulatory trends have significantly emphasized privacy protection of personal data, driven by notable regulations such as the EU’s General Data Protection Regulation (GDPR) [1], the California Consumer Privacy Act (CCPA) [2], Korea Act On Promotion Of Information And Communications Network Utilization And Information Protection [3], and similar authority activities emerging around the world. As more various and larger data would be generated and processed by the 6G system, it is required to provide the technical realization of privacy protection in order to make 3GPP system be more compliant with regulations.

Gap Analysis

- Data collection request mechanism is different per use cases: In 5G system, the data collection for this purpose is standardized with respect to specific use cases, which result in diverse ways of data collection across difference use cases. Even though the data source is the same, different logic or protocol is used to transfer the same data type just because the service or use case is different. This approach can cause potential duplication of configuring the data source entity/function and transferring the duplicated data.

- Data transfer path is various within the system: In 5G, the data transfer path within the system is various, for example, UE to NF (e.g., LPP), RAN to NF (e.g., NRPPa), NF to NF (UPF to NF, CP NF to NF, NF to DCCF to NWDAF). These diverse data transfer paths are due to the fact that use case specific mechanism has been specified under each work item, which leads to lack of common data framework with unnecessarily high complexity in monitoring, control, and deployment planning in 5G.

- Data collection overhead over SBI/N2 (i.e., over control plane): The control plane would be overloaded due to the increase in data volume. Especially, NAS, N2, SBI will suffer from the data collection and the conventional operation for connectivity service can be disrupted if we reuse those CP interfaces and signalings for large data collection. For example, in 5G, it is observed that there is too much burden is imposed to the UPF itself as data source processing and transmitting the collected data over SBI (via Nupf_Event_Exposure service) to the NWDAF. Also, for the use case of LMF-based AI/ML Positioning, as NRPPa over N2 would be reused to collect the data from the RAN, it can cause overloaded situation over N2 and NL1 (RP between AMF and LMF). This analysis claims for the overall investigation on whether the current data collection over N2 and SBI can embrace the data collection.

- Consent enforcement cannot meet the regulatory requirement: Global regulatory trends have significantly emphasized privacy protection of personal data, driven by notable regulations as in [1]-[3]. These regulations establish user consent as a foundational principle: individuals must be informed about how their data will be used and must obtain explicit permission before it can be collected, stored, processed, or exposed. However, the following gaps are identified:

- Lack of standardized user consent/privacy setting mechanism in 5GS: It is not specified in 3GPP on how the 5GC performs setting of user consent and revocation mechanism. Note that LCS privacy profile can be dynamically set by the UE or AF via the standardized procedures as in TS 23.273.

- Naive and coarse user consent purpose and privacy management: The user consent in the UDM only contains the abstracted purpose, e.g., feature level such as network capability exposure. The user consent status/data usage record is invisible to the UE. Even though LCS privacy profile includes further detailed information, but the lack of details on the information about the allowed location information consumer/processor and location information usage record.

- Separated mechanism is employed for user consent and LCS privacy management in 5G: The LCS privacy profile can be dynamically set via NAS signaling or AF-provisioning while user consent in the UDM is statically configured. The management and enforcement of user consent and LCS privacy profile is fundamentally the same from a privacy protection perspective. It is thus more considered more efficient to manage them by means of a common mechanism without employing separate mechanism.

- Privacy protection mechanism such as anonymization and pseudonymization is not well standardized: In 6G study, it is expected to study data exposure service from 6G CN. As the data may contain UE or user related data, the technical realization of privacy protection, e.g., anonymization and pseudonymization, should be discussed. Either of them would be required with respect to the requested service or use case of the data exposure.

How to make a progress to study on SA2 6G system architecture for data framework considering the dependency with other WGs (RAN WGs, SA3, SA5)

Proposals - SA2 starts study on 6G system architecture for data framework (including how to collect and process the data from UE, RAN, NF, and/or AF) considering the scope and practices in 5G as a starting point. - SA2 identifies the target data (set) with its purpose/service and SA2-defined data handling functionalities, which can be used as interim deliverables for coordination and collaboration among the WGs.

Considering the guidance from SA (as in SP-251261: “TSG SA asks SA2 and SA5 to progress and collaborate (including with RAN2 and RAN3 where applicable) on 6G data related work tasks and taking the 5G working scope as the starting point”), the above proposal can be a way forward to make a progress.

In 5G SA2 has specified architecture and procedures, or have on-going study for the followings:

- Data collection from UE for purposes of e.g., LMF-side model training, RAN-side model training, NWDAF-side model training (via AF)

  - Configuration of UE and measurement at UE for AS layer data collection has been in RAN scope

  - Transfer of the data collected at the UE to NF and processing the data within CN has been in the scope of SA2

- Data collection from RAN for purposes of e.g., LMF-side model training, sensing

  - How RAN gets the data over AS layer is in the remit of RAN

  - Transfer of the data collected from RAN to NF has been led by RAN with coordination with SA2, noting that the procedure for data transfer from RAN to NF has been specified in SA2.

- Data collection from NF for purposes of e.g., NWDAF-side model training

- Data collection from AF for purposes of e.g., NWDAF-side model training

Note that each component element of the collected data (set) cannot be assumed to be identical across use cases, since it may differ according to the purpose of data collection or the consumer (requestor) of the data. For example, when radio measurement data is collected from UE or RAN, the actual data set collected from UE can be different with respect to purpose/use case. For the purpose AIML model training the measurement data needs to be associated and collected with assistance information (such as quality indicator information) as a self-contained data set, while it will be linked with other type of assistance information for the purpose of OAM-specific aspect. Also, the granularity of data and reporting requirement would be different per each purpose and use cases (different from domains)

Considering the above aspects, the below table provides an example for categorization of data that can be considered to be generated in 6G system along with the practices in 5G.

| Data source | Data category | Data type /Purpose | Data consumer/ processing entity (expected in 6G) | Practices in 5G / Data collection path |  |
| --- | --- | --- | --- | --- | --- |
| UE | AS layer related data | Radio measurement data for AIML data collection (UE/RAN/NF-side model training at NF or OTT server) + e.g., timestamp, quality info | Data collection NF and/or OTT training server | SA2/RAN LMF-side model (positioning case3b) RAN-side model (positioning case 3a) | Path for AIML data collection (UE-side model training at OTT) in 5G |
|  |  | Radio measurement data for positioning + e.g., timestamp, quality info | LMF | SA2/RAN | Path for AIML data collection (LMF-side model training at LMF/NWDAF) in 5G |
|  |  | Radio measurement data for sensing (3GPP-sensing data) | Sensing NF | - | No practice. UE RAN Sensing NF |
|  |  | Radio measurement data for OAM (e.g., MDT data) + e.g., timestamp, location | OAM (e.g., TCE, MnS producer) | SA5 OAM | UE RAN OAM |
|  |  | Radio measurement data for AIML data collection (for RAN2/3 specified use case, e.g., mobility, SON) | RAN | RAN | UE RAN |
|  | NAS layer data or upper layer (e.g., application layer) data | Application-layer data for NWDAF-side model training EVEX |  | SA2 SA4 |  |
|  |  | Positioning data (not related to radio measurement) for positioning, e.g., SSID | LMF |  |  |
|  |  | Non-3GPP Sensing data |  | - | No practice. UE Sensing NF (transparent to RAN) |
| NF | NF-generated data | SA2 control plane data for training NF-side model (for the purpose of analytics service, e.g., per UE or per PDU Session) | Data collection NF / AI NF | SA2 NWDAF | NF NWDAF |
|  | Context data | Context data for NF service | NF | NF, UDSF, UDM | NF, NFUDSF, NFUDM |
|  | AF-provided data | Data provided by AF via Naf service for purpose of NF-side processing (EVEX data from AF to NWDAF) or provisioning info to UE (e.g., GNSS assistance data) | Data collection NF AI NF | NWDAF LMF | AF (NEF) NWDAF AF (NEF) LMF (UE) |
| RAN | RAN measurement or RAN-generated data | Radio measurement data for sensing (3GPP-sensing data) | Sensing NF | - | RAN Sensing NF |
|  |  | Radio measurement data for AIML data collection (RAN/NF-side model) | LMF/RAN | SA2/RAN LMF-side model (positioning case3b) RAN-side model (positioning case 3a) | Path for AIML data collection (LMF-side model training at LMF/NWDAF) in 5G |
|  |  | Radio measurement data for RAN-specified use case | RAN | RAN | RAN |
|  |  | Radio measurement data for OAM | OAM | OAM | RAN OAM |
| OAM | Management domain data | Management data generated/managed for the purpose of MnS producer | OAM | SA5 OAM | RAN/NF OAM |
|  |  | Management data generated/managed for the purpose of MnS producer | NF requiring node-level management data | SA5-provided MnS API is invoked by SA2 NF | RAN/NF OAM |

※Observation: Green-coloured cells in the above table can be considered as candidates of data to be addressed in this SA2 WT#5 (with coordination with the related WGs).

Considering the above discussion, this pCR proposes to update the text for WT scope in S2-2508132 (postponed in SA2#171 meeting) and to accordingly describe the KI description.

2. proposal

It is proposed to approve the following WT scope in TR23.801.

**** First Change (on top of baseline pCR in SA2#171)****

A.X 	Work Task X: 6G unified data framework

The following is WT#5 in the SA2 6G SID:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

This work task aims to design 6G system architecture for data framework that provide functionalities to handle data generated in the 6G system to support data-driven services via 6G system such as AI and sensing.

The scope of the WT includes the following aspects:

- Identify the target data (set) and its purpose (or use case) for the data framework to be studied within this WT scope and its corresponding functionalities.

NOTE 1:	The 6G data framework is for the data produced from UE, 6G RAN, 6G CN, and AF. Further details on the target data (or data tuple or data set) with its purpose will be identified during the study.

- Design end to end 6G system architecture for data framework, e.g. defining new 6G CN NF(s), new data plane/enhancing CP/UP and interfaces, to support efficient and scalable data handling functionalities, e.g.:

- data collection and transfer (including data source discovery, configuration of the data source, transfer of the collected data)

- data processing at 6G CN NF, e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters

- data storage within 6G CN

- data exposure within 6G system and to outside of 6G system

NOTE 2:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 3:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 4:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

- Study from SA2 architectural and system level enhancement on aspects related to user consent management, privacy protection, security and data governance for data generated in 6G system.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

- Study how to support quality control of data and its handling operation such as forwarding treatment , e.g., based on the requirements from data requester.

**** Next Changes (all new)****

5.X. Key Issue #5.1: Data framework for handling of data generated in the 6G system

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The key issue investigates how to design end to end 6G system architecture for data framework that can be commonly used for handling of data generated in the 6G system to support services deployed over the 6G network such as AI and sensing.

- Target data: Identify the target data (set) and its purpose (or use case) for the data framework to be studied within this WT scope and its corresponding functionalities.

NOTE 1:	The identification of target data may need coordination with other WTs, e.g., WT#4 and WT#6.

NOTE 2:	The expected data volume and traffic characteristic for the data transfer/distribution/exposure will be investigated and considered for identifying architectural requirements.

- Data handling functionalities and procedures: identify and investigate functionalities with associated procedures for data handing, at least including data collection, data transfer/distribution, data processing, and data exposure.

NOTE 3:	During the study, it will be investigated whether and how to separate functions and/or functionalities from CP/UP, allowing independent scalability, evolution and flexible deployments of functions or functionalities for 6G data framework. Also, it will be discussed whether the CP/UP interfaces and functionalities are able to handle the collection/transfer of the data generated from heterogeneous data sources (UE, RAN, and NF) in the 6G system with the consideration of its volume and traffic characteristic.

a.	Data collection and transfer: How to support scalable and efficient data collection and transfer functionalities, including, data source discovery/selection, data source configuration for collection, transfer of the collected data with the consideration of the followings:

-	How to minimize adverse impact on the conventional operations and interfaces for conventional connectivity services over CP or UP

-	Whether and how to support efficient and flexible data transfer involving UE, RAN, CN NFs, and AF

-	Whether and how to support enable data controllability and visibility (e.g., MNO controllability, and MNO visibility)

-	For data collection/transfer from UE, whether and how to differentiate or separate the data from the UE’s regular CP or UP traffic.

-	Whether and how to enable service-differentiated data transfer, e.g., based on serving request for data collection/transfer

b.	Data processing: Identify what kind of processing/formatting of the collected data needs to be supported, and whether and how to enable them

c.	Data Storage in 6G CN: How to support data storage that can be commonly used across the data-driven service in the 6G system for efficient data reuse

d.	Data Exposure (i.e., data transfer to other domain, e.g., AF): How to support data exposure with the secured data access control (e.g., authorization of data collection request, data service access authorization mechanism including the authorization to the service requestor and the target object/environment)

NOTE 3:	In this key issue, the data generated in the 6G system will be the target of the data framework. UE, RAN, and NF (potentially also AF) will be considered as data sources for data-driven services.

NOTE 4:  Coordination/dependency with SA5 is needed for the following aspects: charging aspect, data lifecycle, audit management, whether and how to align with the data handling mechanisms supported in the management domain, whether and how to utilize OAM-generated data.

NOTE 5:	Coordination with WT#1.1 (UE-CN interaction), WT#1 (RAN-CN interaction, Management of context data stored in NF), WT#3 (AI-related aspects), WT#4 (Sensing data), and WT#6 (Computing service related data) is expected.

NOTE 6:	The data framework studied in this key issue Data framework for handling of data generated in the 6G system will comply with data governance and privacy protection (including user consent enforcement), which are studied under Key Issue #5.2.

NOTE 7:	Coordination with RAN WGs is needed for the identified issues during the study phase when the data source is RAN or UE for the collected data defined by RAN WGs (e.g., measurement data by UE or RAN).

5.X. Key Issue #5.2: Policy and quality control aspect on handling data

This key issue investigates aspects on policy and quality control of data and its handling operation such as forwarding treatment, e.g., for ensuring data quality based on the requirements from data requester.

-	Whether and how to support policy control and/or service quality control for the purpose of assurance of data quality

-	Whether and how to support policy control and/or service quality control in order to support service-differentiated data transfer, processing and storage mechanisms

NOTE :	Coordination with WT#1.2 is expected.

5.X. Key Issue #5.3: Privacy protection aspects

This key issue investigates aspects on how to enhance privacy protection for the data handling in the 6G system, including enhancements on system and procedure for the data governance, user consent and/or privacy framework. The followings can be considered

-	Whether and how to support dynamic, granular and revocable user consent for data handling

-	Whether and how to support data privacy protection mechanism, e.g., anonymization and/or pseudonymization, based on sensitivity (e.g., whether it contains UE or user data) and/or purpose of data handling

NOTE 1:	The aspects on data security, privacy protection and user consent need coordination and work split with SA3.

NOTE 2:	The use case (e.g., data exposure service or data storage), which require anonymization or pseudonymization, will be identified during the study.

**** End of Changes ****