SA WG2 Meeting #S2-171	S2-2508907

13 – 17 October, 2025, Wuhan, China	(revision of S2-2508121)

Source:	Nokia, T-Mobile US, [Futurewei, InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO]

Title:	[WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for User Plane Architecture

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the study considerations relating to user plane architecture in 6G under WT1.2 defined in SP-250806.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

SP-250806 indicates 1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

In relation to the “user plane architecture”, this paper proposes to consider the following inputs:

Architectural Motivation: A quintessential part of the system architecture of any generation is the user plane architecture. 5G was pivotal in defining a lean user plane architecture able to handle both the excessive user plane traffic and the required functionalities by the user plane to support 5G services. Essentially, a User Plane Function (UPF) is the gateway between the access network and the Data Network. A UPF instance selected to serve a PDU Session supports at least the functionalities required for the PDU Session. A UPF advertises its profile in the NRF and is properly selected by the Session Management function (SMF) which is the sole controller programming UPFs.

The 5G user plane architecture has successfully defined enablers for Edge Computing, I-UPF change, local PSA relocation, deployment topologies with specific SMF Service Areas, Access Traffic Steering, Switching, Splitting, User Plane Information Exposure via a service-based interface, Service Function Chaining, Handling of Payload Headers, 5G VN group communication, Time Sensitive Communications, Deterministic Networking, Cellular IoT, eXtended Reality (XR) and interactive media services.

Following are the key factors of the successful 5G user plane architecture:

-	Lean paradigm with mainly a single UPF entity supporting functionalities as needed, controlled and dynamically programmed by a single controller (SMF).

-	UPF chaining only applies when necessary, e.g. in roaming architectures, in deployments with I-UPF etc. to avoid impacting the performance of the UPF, which already in 5G is called to handle a lot of traffic that is expected to further grow in 6G.

-	3GPP for generations leverages GTP-U, a protocol able to run over any IP based transport network, decoupling the application and the transport layers, fully extensible, allowing 3GPP fast and easy to define potential extensions required by any new service (e.g. PDU Set Information for XR services), without dependency on other standardization organisations, enabling deployments in which transport owner and operator are separate entities.

Evolution to 6G may consider further optimizing the architecture, without unjustifiable jeopardising its essential parts to avoid interworking issues and market segmentation.

Gap Analysis: While the 5G user plane architecture is considered successful in general it is important to further optimise it and evolve it to be able to handle the user plane needs of the 6G era without unjustifiable jeopardising the pilar of the 5G user plane architecture:

-	By identifying any enhancements to the information exchanged between the application (application client in the UE, AS) and the network for 6G user plane traffic, which may be encrypted, for optimal traffic handling and potentially enabling dynamic adaptations.

-	By identifying any multiple options for the same functionality already specified and properly handling them e.g. deprecating them depending on the use case each option serves and its adoption by the market. This is very important to facilitate interworking issues in the field.

-	By identifying extensions to the 5G user plane architecture required to fulfil 6G requirements.

**** First Change ****

# Annex A.X. WT#1.2 User Plane Architecture for 6G Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance application-network collaboration for 6G user plane traffic, which may be encrypted, to enable the UE/AC, AS and the network to exchange information that allows optimal traffic handling, potentially enabling dynamic adaptations.

NOTE 1: Application-network information exchange should be authorized and validated by the operator.

- Whether and how to enhance CP-UP interaction for better multi-vendor interoperability.

- Whether and how to extend the service-based architecture for the user plane control interface to UPF.

- Whether and how to enhance flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

- Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

**** Second Change ****

# 5.X. Key Issue #X (1.2.X): Support of user plane architecture optimization in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance application-network collaboration for 6G user plane traffic, which may be encrypted, to enable the UE/AC, AS and the network to exchange information that allows optimal traffic handling, potentially enabling dynamic adaptations.

NOTE 1: Application-network information exchange should be authorized and validated by the operator.

- Whether and how to enhance CP-UP interaction for better multi-vendor interoperability.

- Whether and how to extend the service-based architecture for the user plane control interface to UPF.

- Whether and how to enhance flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

- Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

**** End of Changes ****