3GPP TSG-WG SA2 Meeting #170	S2-2509062

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2507252)

Source:	Huawei, HiSilicon

Title:	KI1 Interim Agreement: Topology 2 Interim Agreements

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Rel-19 provided interim conclusions for topology 2 support for RRC option for T2, these are converted to interim agreements in Rel-20.

1. Introduction/Discussion

These Rel-20 interim agreements are copies of the interim conclusion from Rel-19 TR 23.700-13 for topology 2 for the RRC option.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI1

The following principles apply for Topology 2:

-	The architecture and procedures for Ambient IoT defined in TS 23.369 [x] are used as a baseline for topology 2 support, with the difference that a UE Reader performs AIoT Service Operation requests from an AIOTF instead of a RAN Reader.

-	Subscription Aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF and AIOTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then the AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g., taking into account assistance information received from the AIOTF) from the gNB.

NOTE 1:	Assistance information is specified in TS 23.369 [x].

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

-	When a UE Reader performs Registration if the UE provides an indication that it is a UE Reader and the AMF determines that it is authorised to be a UE Reader, the AMF assigns a UE Reader ID to the UE. The UE Reader ID is stored it in the UE context in AMF for the UE and when providing the UE Reader authorisation to NG-RAN, it also provides NG-RAN with the UE Reader ID.

-	UE Reader selection supports External Target Area or specific UE Reader selection by the AF:

-	The AF using the existing procedures can indicate an External Target Area information which the NEF translates to AIoT Area(s) and provides to the AIOTF. The AIOTF can select UE Readers if configured with fixed UE Readers locations or construct a candidate list of UE Readers and use existing procedures, for example those defined in TS 23.273 [x], to determine which UE Readers are provided to NG-RAN. This selection is implementation dependent.

-	The AF can indicate a specific UE Reader. The NEF determines the Permanent identifier of the UE, as described in TS 23.501 [x] clause 5.20, using the procedure described in clause 4.15.3.2.13 of TS 23.502 [3]. and includes the Permanent identifier of the UE in the Naiotf_AIoT_Inventory Request and skips Reader Selection.

-	The AIOTF determines the serving AMF for each of Selected UEs using the UDM. The AIOTF then requests the UE Reader ID for the UE Reader from the AMF. The AIOTF exchanges NGAP AIoT Information with the UEs serving NG-RAN node, via the UEs serving AMF, and includes the UE Reader IDs within the NGAP AIoT Information.

In the indirect connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.y-1.

Figure 7.1.y-1: Protocol Stack for the RRC option

# * * * * Second change (all new) * * * *

7.2	Topics for further consideration

7.2.Z	Topics for further consideration for KI1

The following topics require further consideration:

-	How to revoke authorisation of a UE Reader.

# * * * * End of changes * * * *