3GPP TSG-SA WG2 Meeting #171	S2-2509276

13-17 October, 2025, Wuhan , China 	(Revision of S2-2508873)

Source:	China Mobile

Title:	WT#1: New solution for Supporting dynamic configuration of MDC2 endpoints

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#3 of TR 23.700-56.

1. Discussion

This contribution proposes a solution to enhance DC3 and DC4 to support dynamic configuration of MDC2 endpoints when a P2A or P2A2P DC are established.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #X |  |  | X |

# * * * * Second change (All new texts)* * * *

6.X	Solution #X: Support dynamic configuration of MDC2 endpoints with new DCSF services

6.X.0	High-level solution Principles

This solution addresses KI#3: Supporting dynamic configuration of MDC2 endpoints.

This solution takes the existing Rel-18 procedures of P2A and P2A2P DC establishment as the basis and updates the existing steps between DCSF and DC AS by defining new service of DCSF.

6.X.1	Description

This solution defines a new service of DCSF which is used to request to establish, update or delete a MDC2 connection. This service exchanges the MDC2 media points information on MF and DC AS to each other. The connection ID is allocated by DCSF and is used to correlate the MDC2 connection.

The solution also clarifies that the DCSF selects the correct DC AS and get the address of the selected DC AS URL based on binding information which indicates which DC AS is serving the application. The mapping of the app id and the DC AS URL is locally configured on DCSF or is retrieved from the application data in the HSS.

6.X.2	Procedures

6.X.2.1	P2A DC establishment procedure

This procedure depicts the procedure of UE initiated P2A DC establishment which reuses the P2A DC establishment procedure in clause AC.7.2.2 of TS 23.228 as the basis and updates original step 9 and 10 to clarify the interaction between DCSF and DC AS.

The steps in the procedure are as follows:

Step 0-8 is the same with step 0-8 in clause AC.7.2.2 of TS 23.228;

- The DCSF selects the DC AS serving this session based on the binding information of this P2A DC. The DCSF gets the DC AS URL of the selected DC AS by local configuration or retrieves it from DCSF service specific data stored in HSS.

- The DCSF invokes Ndcsf_MDC2ConnectionRequest_Create service operation as defined in clause 6.X.3.2 to request the selected DC AS to establish MDC2 connection with the MF for the P2A DC. The service operation includes the MDC2 media endpoint information on the MF.

- The DC AS responds to Ndcsf_MDC2ConnectionRequest_Create service operation with its MDC2 media endpoint information for the MDC2 connection.

- 12-28. The same with step 11-27 in clause AC.7.2.2 of TS 23.228.

Editor’s note: The procedure of establishing P2A2P DC is similar to the above procedure and will be added later.

6.X.3	DCSF Services

This solution defines DCSF services to support interaction between DCSF and DC AS for establishing the P2A or P2A2P DC.

Table 6.X.3.1-1: DCSF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_MDC2ConnectionRequest | Create | Request/Response | DC AS |
|  | Update | Request/Response | DC AS |
|  | Delete | Request/Response | DC AS |

Editor's note:	Whether the service operation type is Subscribe/Notify (by DCSF) or Request/Response (by DC AS) is FFS.

6.X.3.1	Ndcsf_MDC2ConnectionRequest

#### 6.X.3.1.1	General

Service description: This service enables the consumer to be requested to establish, update or delete MDC2 connection towards the MDC2 endpoint on the MF.

#### 6.X.3.1.2	Ndcsf_MDC2ConnectionRequest_Create service operation

Service operation name: Ndcsf_MDC2ConnectionRequest_Create

Description: This service operation enables DCSF to request the consumer to establish a MDC2 connection with the MF towards the MDC2 endpoint on the MF and responds with its own MDC2 endpoint information.

Inputs, Required: Session ID, Connection ID, remote MDC2 media endpoint information

Session ID is the identity of the IMS session to which the MDC2 connection belongs.

Connection ID is the identity of the MDC2 connection. It is allocated by the DCSF.

Remote MDC2 media endpoint information contains the IP address and port of the MDC2 media endpoint on MF.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. The Calling ID and Called ID can be used by DC AS to decide whether a P2A2P DC should be further established.

Outputs, Required: Result indication.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

#### 6.X.3.1.3	Ndcsf_MDC2ConnectionRequest_Update service operation

Service operation name: Ndcsf_MDC2ConnectionRequest_Update

Description: This service operation enables DCSF to request the consumer to update an existing MDC2 connection.

Inputs, Required: Session ID, Connection ID, updated remote MDC2 media endpoint information.

Inputs, Optional: None.

Outputs, Required: Result indication,.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

#### 6.X.3.1.4	Ndcsf_MDC2ConnectionRequest_Delete service operation

Service operation name: Ndcsf_MDC2ConnectionRequest_Delete

Description: This service operation enables DCSF to request the consumer to delete an existing MDC2 connection.

Inputs, Required: Session ID, Connection ID.

Inputs, Optional: None.

Outputs, Required: Result indication,.

Outputs, Optional: None.

6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DCSF:

-	Supports to select the DC AS serving the session based on the binding information of this P2A DC.

-	supports Ndcsf_MDC2ConnectionRequest service to interact with DC AS to establish, update or delete MDC2 connection for P2A or P2A2P DC.

DC AS:

-	supports Ndcsf_MDC2ConnectionRequest service to interact with DCSF to establish, update or delete MDC2 connection for P2A or P2A2P DC.

# * * * * End of changes * * *