SA WG2 Meeting #171	S2-2508351

13 – 17 October, 2025, Wuhan, CN	(revision of S2-2506640)

Source:	vivo

Title:	[WT#1.2, Network Exposure] Support for network exposure to UE in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study the WT and KI for network exposure to UE for 6G network.

Introduction:

The approved SA2 6G SID includes the following work task:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

…..

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

……

This paper aims to focus on network exposure to UE in 6G under WT#1.2.

1. Justifications

1.1 Architectural Motivation on network exposure to the UE

The network exposure enables the network to expose capability and information to the third party service providers. However, for the UE side, the network information is also needed from the following aspects:

- UE application client needs the network status information for the optimization of the UL data transmission, e.g., adjustment of the codec algorithms according to the network status information. The information exposed to the UE should be real-time. Since the network status may be changed frequently, so the UE application needs the latest network status information for the UL traffic optimization.

- Improve end-user QoE experience: When the network transmission performance is getting worse, the UE can identify that the problem is due to the communication network and provides the solution from the UE's perspective to improve the service experience

- The UE is a whole system including OS, modem, application clients, etc. The network information exposure to the UE could benefit other layers within the UE, not only the application clients. The benefits include the following aspects:

  - Improvement of the UE’s internal control efficiency;

  - Multi-layer optimization within the UE;

  - Support for real-time optimization of service flows at the UE.

- In 6G, the UE may be enabled with AI capabilities, e.g., QoS evaluation and QoS prediction capability. The information (e.g. per UE or per flow QoE, network resource status) exposed to the UE may help the UE to train the AI capability for QoS prediction. This AI capability enables the UE to get the information on whether the QoS requirements for certain applications can be satisfied in the near future.

  - For example, when the user is on the subway during the morning rush hour, the network is congested and the service experience is bad; however, when the user is at the office, the network status is good and service experience will be much better than on the subway. So the network status information for different time and location varies a lot and is beneficial for the AI model training for QoS prediction within the UE.

1.2 Gap Analysis on network exposure

In 5G network, network capability and information exposure to AF has been specified. However, it is not supported for the 5G network to expose the information to the UE directly. The UE may get the exposed information from the AS/AF via application layer; however, this aspect is not fully specified due to the fact that the application layer between the UE and AS/AF is out of 3GPP SA2 scope. The current mechanisms for the network information exposure to UE via AF/AS has the following issues:

- It relies on the deployment of AF at the application server side. And third party AFs are not commonly deployed in the real business.

The 3rd party application providers need to deploy AF for the interaction with the mobile network. Since the users of the application may belong to different operators, the 3rd party AF needs to interact with multiple operators to get certain network information and apply this information for its applications.

Furthermore, the development and deployment of AF in the 3rd party may require extra capabilities to understand and realize APIs defined in the 3GPP standards. Furthermore, different operators may support different APIs towards the AF. So this may not be an easy work for the 3rd party to support the AF.

Figure 1: Network exposure to the AF

- The information exposure to UE from AF/AS may introduce extra delay.

Currently, the network exposes the information to AF via NEF/PCF/UPF. Then the application server sends the information to the UE via application layer. To the delay sensitive applications, this information may not be useful if the information can’t be received by the UE in time.

- The information exposure to the UE via AF/AS can only be exposed to the UE’s App Client. However, as the motivation stated, the UE’s OS or modem may also need the information. If the UE’s App Client doesn’t expose the info to the UE’s OS or modem, these parts within the UE can’t get this information.

1.3 Technical Impact

- The exposure architecture, including whether the new 6G entities or 5G NEF enhancements are needed for the management of the network capability exposure to the UE?

- Network capability/information exposure from the core network to the UE (e.g., via control plane, or via user plane);

- Network capability/information request from the UE and the authentication of the UE request for network capability/information exposure;

For the Overlap with other WTs, e.g., data plane, sensing:

- Other WTs, e.g., data plane, sensing, may also consider the information exposure. It is proposed to solve the information exposure issue, if any, in the WTs of these specific features first, and then, consider how to merging the related conclusion, if any, into the network exposure framework.

1.4 Proposals based on S2-2508136 (the postponed version from pen holder in SA2#170 meeting)

- Proposal 1: X.1, X.2 and X.3 are fine to be studied in this WT. Some wording changes are proposed for clarification.

- Proposal 2: Regards to X.4, this bullet highly depends on the study of WT#3, so it’s better to have the related discussion in the specific WT about the AI related feature first. And then, we align the progress, if needed, in the network exposure framework WT.

**** First Change ****

Annex A.X. WT#1.2 Network Exposure Framework

One aspect of this work task is to study whether and how to support and/or enhance the network exposure framework in 6G. Aspects that can be studied include:

- X.1 How to support a common framework for the exposure of network capabilities.

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

NOTE 1:	Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters.

  - How to support enhanced privacy protection, user consent, data anonymization, and service authorization mechanisms.

NOTE 2:	This aspect of work task has dependency on Work Task #5 (Data Collection).

  - Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

NOTE 3: 	The scope of this work task is to specify the common exposure framework, including possible impact to NEF. This work task will not address the information being exposed defined by other work tasks. Other work tasks can cover services that have an exposure component (e.g. computing and sensing). The exposure framework that is developed in this work task should be adaptable to and suitable for those other services (e.g. computing and sensing).

NOTE 4:	What capabilities are exposed in-band will be coordinated with the work that is done under other work tasks.

- X.2 How to support network capabilities exposure to UE.

  - Whether and how network capabilities should be exposed to a consumer on the UE, considering their untrusted environment and limiting exposure to the hosting UE’s context.

  - How to easily authorize consumers of network capabilities when the consumer is resident on the UE.

  - How to support and authorize the UE request for network information exposure.

- X.3 How to support In-band Information Reporting.

  - Whether and how to report information in-band between application endpoints and 6G network in an operator-controlled manner.

  - Whether and how the reporting path of an exposed capability can be optimized (e.g. in-band or not in-band).

  - How to discover, authorize and activate in-band reporting offered by the network and whether and how to integrate in-band reporting in the common exposure framework.

NOTE 5:	In-band information reporting will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 6:	This work task will cover how capabilities are reported in-band. What capabilities are reported in-band will be coordinated with the work that is done under the User Plane and QoS aspects of Work Task 1.2.

NOTE 7:	Information exposure is based on operator policies/SLAs.

NOTE 8:	The Intent-Based Exposure has dependency on Work Task #3. The goal of intent-base exposure is to make accessing exposed services more user friendly. In terms of supporting intent, this work task will align with WT#3, if needed, for the further consideration on the southbound interfaces to support intent based APIs.

NOTE 9:	This work task requires coordination with SA3 (e.g. user consent, privacy and anonymization), SA6 (e.g. CAPIF).

NOTE 10:	This work task has dependency on service requirements to be specified by SA WG1.

**** Second Change ****

5.X. Key Issue #X: System architecture design to support network exposure for 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This Key Issue will study:

- How to support the network capability exposure to the UE.

  - How to support the UE request for network capability/information and how to authorize the UE request.

  - What information can be exposed to the UE, e.g., latency, congestion information, available bitrate, etc.;

  - How to expose the network capability/information to the UE, e.g., via user plane, via control plane.

NOTE 11:	   The study needs to be coordinated with SA3.

- How to support In-band information reporting.

  - How to discover, authorize and activate in-band reporting offered by the network;

  - What information can be reported in-band, e.g., latency, congestion information, available bitrate, etc.;

  - How to report information in-band between application endpoints and 6G network in an operator-controlled manner.

**** End of Changes ****