3GPP TSG-SA2 Meeting #171	S2-2508471

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2506697)

Source:	Huawei, HiSilicon

Title:	KI#6, Sol #20: Update to focus on configuration parameters

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Update Solution#20 including the terms alignment, details on configuration parameters and other clean-ups.

1. Introduction

This pCR proposes updating Solution#20 for Configuration Parameter Provisioning of Sensing Entities, especially:

-	Change Sensing Entities to gNB(s) as per the scope of the study;

-	Adding more details regarding the configuration parameters, including the ones from AF to SCF, and from SCF to the gNB;

-	Clean up the residual content to only focus on KI#2.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6.20	Solution #20: Solution for Configuration Parameter Provisioning of gNB(s)

6.20.1	High level principles

Different from sensing service request to trigger sensing service operation, configuration parameter provisioning aims to provision general configuration parameters towards the gNB(s), not for individual sensing service operation.

There may be two cases for configuration parameter provisioning towards gNB(s). The first case may be conducted by sensing operation server, which could belong to the operators or have SLA with the operators. The other case may be performed by the OAM or local configuration within the operator’s domain. This solution focuses on the first case.

The high level principles of the solution are:

1. For the untrusted sensing operation server, it sends configuration parameter provisioning request to the NEF. After authentication/authorization was made, the NEF forwards configuration parameter provisioning request to the SCF(s). For the trusted sensing operation server, it sends configuration parameter provisioning request directly to the SCF(s). SCF determines the Sensing configuration parameters based on the received Sensing Service Requirement.

2. The SCF determines the targeted gNB (s) and performs the configuration parameter provisioning on gNB(s).

NOTE 1:	The SF (Sensing Function) splits into SCF (Sensing Control Function) and the SPF (Sensing Processing Function.

6.20.2	Description

This solution is to resolves Key Issue #6 about the configuration parameter provisioning of  gNB(s).

For the purposes of the present solution the following definition applies:

Sensing Control Function: Indicating the logical function which is involved to support Sensing Service and responsible for selecting the gNB(s) and for providing the related configuration information.

Sensing Processing Function: responsible for receiving the 3GPP Sensing Data and for performing the elaboration for determining the Sensing Result.

6.20.3	Procedures

6.20.3.1	Sensing operation server-initiated Configuration Parameter Provisioning procedure

Figure 6.20.3.1: Configuration Parameter Provisioning procedure

1.	The untrusted sensing operation server AF sends the Nnef_SensingService_Subscribe request message to the NEF or the trusted sensing operation server AF sends the Nscf_SensingService_Subscribe request to the SCF with the following parameters: AF ID, sensing related parameters, Target Sensing Service Area and so on. The messages may further include the Sensing Service type, accuracy, resolution, refresh rate, Sensing Service time duration.

NOTE X:	Detailed parameters of the AF inputs are further clarified by Solutions of KI#2.

NOTE Y:	Sensing operation server is application function.

NOTE Z:	The NEF authenticating and authorizing the untrusted sensing operation server to allow to perform the operation will be clarified by the solutions of KI#2.

If NEF is involved, it converts the external target area information to internal target area information. Then the NEF forwards the request to the SCF.

2.	Authorisation of Sensing service as described by solution to KI#2 and discovery and selection of gNB as described by solution to KI#3

NOTE A: The NEF obtaining the SCFs, Then the NEF forwards the request to the SCF. and the SCF determining the targeted gNB(s) will be further clarified by the solutions of KI#3.

3. The SCF provisions the sensing configuration parameters provisioning to gNB(s) via Sensing Request message based on the gNB(s)'s capability(ies), parameters received in step 1 and the possibly sensing service requirements from other AF(s). The sensing configuration parameters contains at least one of the following parameters: accuracy, resolution, refresh rate, Sensing Service time duration.

If there is existing configuration to the related gNB(s), the SCF determines whether the existing configuration to the gNB(s) can fulfil the requirements from gNBs, if so steps 3 and 4 are skipped, otherwise the SCF updates the configuration parameters to reflect the most demanding requirements among the AF(s)'.

4.	The Sensing Processing Function obtain 3GPP Sensing data and determine the Sensing Result as defined in KI#4 solution, then the SCF obtains the Sensing Result and expose Sensing Result as defined in KI#5 solution.

6.20.4	Impacts on services, entities, and interfaces

This solution impacts the following entities.

NEF：

- 	Supports the selection of the SCF based on internal target area information.

SCF:

-	Supports configuration parameter provisioning procedures

gNB:

- 	Supports sensing configuration parameter provisioning procedures.

# * * * * End of changes * * * *