SA WG2 Meeting #S2-171	S2-2508582

13 - 17 October, 2025, Wuhan, China

Source:	Nokia

Title:	Conclusions for KI#2

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes updates for the interim agreements and conclusions for KI#2.

# Discussion

In TR 23.700-04 V0.2.0 (2025-05), some interim conclusions for KI#2 are captured in clause 7.1.2, but the following issues are marked as for further consideration in clause 7.2.2:

-	Whether the UPF can be the consumer of the user plane traffic pattern and behaviour analysis or not.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

In addition, it was unclear whether separate analytics are required for use case 1 and use case 2.

It is proposed to resolve those issues in the following manner:

- Can the UPF be the consumer of the user plane traffic pattern and behaviour analytics?

Input data for the user plane traffic pattern and behaviour analytics are expected to originate primarily from the UPF, and so the main flow of information would be UPF->NWDAF->UPF, i.e. a closed loop. Further, there is a risk of high signalling load related to input data collection for that analytics.

For those reasons it appears preferably that the UPF applies internal logic (possibly ML) for optimisations it can autonomously perform rather than using the NWDAF. It is expected that in in 6G many NFs will be enhanced with ML capabilities to optimize their own behaviour. So it is proposed to leave the optimisations based on internal UPF logic to implementation in Rel-20 and wait for 6G decisions whether any aspects for ML directly applied in NFs are to be standardised.

Further, the UPF has limited freedom to apply internal logic as its behaviour is mostly controlled by the SMF, which in turn may be controlled by policy control. For instance, the SMF/PCF configures the UPF to handle packet flows in specific ways.

While UPF implementations cannot be prevented to subscribe to user plane traffic pattern and behaviour analytics (if such analytics are standardized), it is suggested not to describe that exceptional case in standards.

- Are new UPF events required?

According to Clause5.2.26.2.1 of TS 23.502, the UPF can already expose the following events:

- QoS Monitoring

- User Data Usage Measures or User Data Usage Trends.

- TSC management information

- UE NAT mapping

- Handling of Payload Headers

None of these events are suitable to expose special observed traffic patterns, Abnormal traffic from a special source, or high rates of dropped packets, which would be useful input for user plane traffic pattern and behaviour analytics, so new UPF events are required.

- Are new SMF events required?

Subscription for UE related events observed at the UPF need to be done via the SMF (indirect subscriptions where the SMF configures the UPF to send related notifications can be applied). As it is expected that user plane traffic pattern and behaviour analytics can target specific UEs, SMF based subscriptions need to be considered.

Events supported by the SMF are listed in Clause 5.2.8.3.1 of TS 23.502:

-	UE IP address / Prefix allocation/change.

-	PDU Session Establishment and/or PDU Session Release.

-	UP path change

-	Simultaneous Connectivity failure.

-	QoS Monitoring.

-	Change of Access Type.

-	Change of RAT Type.

-	PLMN change.

-	Change of Satellite backhaul category.

-	Downlink data delivery status.

-	QFI allocation.

-	QoS Flow change.

-	QFI deallocation.

-	Total number of Session Management transactions.

-	Information on PDU Session for WLAN.

-	User plane status information.

-	Session Management Congestion Control Experience for PDU Session.

-	UE session behaviour trends.

-	UE communications trends.

-	UP with redundant transmission.

-	User Data Usage.

-	User Data Usage Trends.

-	Service Signalling characteristics.

-	Usage data per gNB and (I-)UPF.

None of these events are suitable to expose special observed traffic patterns, abnormal traffic from a special source, or high rates of dropped packets, so new SMF events are required.

- How to reduce the reporting load of input data sources

It is essential to avoid that the UPF reports every received packet to the NWDAF for analysis. The following techniques have been suggested in different solutions and should be used in combination:

- The UPF only reports special observed traffic patterns that it identifies based on its implementation. Traffic pattern IDs and/or Application IDs (to be decided in normative phase) are used in the related subscription and event reporting to identify such traffic patterns.

- The UPF only reports sudden increases of packets complying with a certain pattern.

- The UPF only reports high volumes of packets complying with a certain pattern.

- Consumer actions

The following consumer actions are suggested:

- A PCF as consumer of NWDAF analytics could apply PCC rules to block abnormal traffic (use case 1), or to optimize the user plane performance by adjusting the QoS, for instance to increase capacity for legitimate traffic peaks (use case 2).

- A SMF as consumer of NWDAF analytics could then configure UPFs to block abnormal traffic.

- OAM as consumer of NWDAF analytics could configure SMF and/or UPF to block abnormal traffic or to increase capacity for legitimate traffic peaks.

- An AF as consumer of NWDAF analytics and data source could take action to correct abnormal traffic. For instance, in an enterprise network or IoT deployments, an (IOT) server to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

- Different Analytics IDs for use case 1 and use case 2?

For use case 2, the following possibilities could be considered:

- The NWDAF provides analytics about traffic patterns

- The NWDAF provides analytics with policy recommendations derived from observed traffic patterns.

However, it is not desirable to reopen the debate about policy recommendations in 5G. It is also expected for 6G that NFs such as the PCF can obtain ML capabilities, and that might obsolete policy recommendations.

Also the description of use case 2 in clause 5.1.2 already suggests traffic pattern analytics will be used:

The analytic capability of NWDAF can generate the statistic or prediction of the traffic pattern for 5GC to do user plane performance optimization, e.g. assistance to QoS control management, in advance.

Assuming that some analytics about traffic patterns will also be used for use case 2, it seems that there will be no substantial differences with respect to input and output data, although use case 1 possibly requires more variations in the output analytics.

Further, a single analytics ID about traffic patterns for both use cases would provide synergies for the NWDAF.

Thus, it is suggested that the analytics about traffic patterns apply to use case 1 and use case 2.

# Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** Start of the change ***

### 7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

#### 7.1.2.1	General

No impact on RAN and UE operation.

A new user plane traffic patterns analytics ID is defined. It is applicable for both use case 1 and use case 2.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information that allows to subscribe to specific related input data at the UPF. Filters can target special traffic patterns identified by UPF internal logic to be identified by a special ID known to analytics consumer, NWDAF and UPF (whether the analytics ID or a new traffic pattern ID is used will be determined in the normative phase), special UEs or UE group IDs, special data sources, and/or provide thresholds for a minimum volume of an observer traffic pattern, or for a minimum increase rate in the observation of an observer traffic pattern. The NWDAF can apply similar parameters when subscribing to events at the UPF or SMF.

#### 7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM and AF. The consumer may take the output analytics into account as follows:

-	A PCF as consumer of NWDAF analytics could apply PCC rules to block abnormal traffic.

-	A SMF as consumer of NWDAF analytics could then configure UPFs to block abnormal traffic.

-	OAM as consumer of NWDAF analytics could configure SMF and/or UPF to block abnormal traffic or to increase capacity for legitimate traffic peaks.

-	An AF as consumer of NWDAF analytics and data source could take action to correct abnormal traffic. For instance, in an enterprise network or IoT deployments, an (IOT) server to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

-	To derive the output analytics, the following input data may be collected from the UPF and/or SMF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic. The information can identify an traffic pattern as observed by the UPF based on implementation logic via a special ID (application ID and(or special packet ID), a source from where traffic matching the traffic pattern is received, traffic flow filters matching traffic for which traffic patterns are observed, information about incidence rate and its increase rate for special traffic patterns, and information about observed bandwidth and data volume for related traffic flows.

-	If individual UEs are targeted, the NWDAF subscribes at the SMF to obtain related information from the UPF.

#### 7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF. Based on the analytics, the PCF can generate or update PCC rules to optimize the user plane performance by adjusting the QoS, for instance to increase capacity for legitimate traffic peaks.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics on traffic pattern information.

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying a traffic pattern as observed by the UPF based on implementation logic via a special ID (application ID and(or special packet ID), and the associated application/service data flow(s) and related collected QoS monitoring information (e.g. information about observed bandwidth and data volume for related traffic flows).

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

None

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.2	Conclusions for KI#2

No impact on RAN and UE operation.

A new user plane traffic patterns analytics ID is defined. It is applicable for both use case 1 and use case 2.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information that allows to subscribe to specific related input data at the UPF. Filters can target special traffic patterns identified by UPF internal logic to be identified by a special ID known to analytics consumer, NWDAF and UPF (whether the analytics ID or a new traffic pattern ID is used will be determined in the normative phase), special UEs or UE group IDs, special data sources, and/or provide thresholds for a minimum volume of an observer traffic pattern, or for a minimum increase rate in the observation of an observer traffic pattern. The NWDAF can apply similar parameters when subscribing to events at the UPF or SMF.

To derive the output analytics, the following input data may be collected from the UPF and/or SMF to support the NWDAF-based analytics:

-	Information to identify the abnormal traffic. The information can identify an traffic pattern as observed by the UPF based on implementation logic via a special ID (application ID and(or special packet ID), a source from where traffic matching the traffic pattern is received, traffic flow filters matching traffic for which traffic patterns are observed, information about incidence rate and its increase rate for special traffic patterns, and information about observed bandwidth and data volume for related traffic flows.

-	If individual UEs are targeted, the NWDAF subscribes at the SMF to obtain related information from the UPF.

For use case 1, the service consumer of the analytics service may be SMF, PCF, OAM and AF. The consumer may take the output analytics into account as follows:

-	A PCF as consumer of NWDAF analytics could apply PCC rules to block abnormal traffic.

-	A SMF as consumer of NWDAF analytics could then configure UPFs to block abnormal traffic.

-	OAM as consumer of NWDAF analytics could configure SMF and/or UPF to block abnormal traffic or to increase capacity for legitimate traffic peaks.

-	An AF as consumer of NWDAF analytics and data source could take action to correct abnormal traffic. For instance, in an enterprise network or IoT deployments, an (IOT) server to control abnormal traffic to/from (IoT) devices or to reconfigure some mis-configured devices.

For use case 2, the service consumer of the analytics service is a PCF. Based on the analytics, the PCF can generate or update PCC rules to optimize the user plane performance by adjusting the QoS, for instance to increase capacity for legitimate traffic peaks.