3GPP TSG-SA2 Meeting #171	 S2-2508638

# Wuhan, China, October 13-17	2025															(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5586 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on handling of application ID/predefined PDRs in reporting of usage data volume to EIF - Removal of SDF/Application ID level option |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At SA2#170 the following papers S2-2506407 S2-2506408 S2-2506409 were submitted, proposing to shift from the PFCP based approach on N4 for reporting of usage data, at the granularity requested by the EIF, to the SMF and then onwards to the EIF, to the SBI-based UPF exposure service to EIF. The reason to do this was quoted to be the fact that for the case of predefined PDRs (e.g. based on application identifier) the SMF is not aware of the actual Packet Filters, and only the UPF is. In fact the supporting company believes the SMF can be made aware of Applciation IDs and Predefined PDRs semantics and then apply logic to resolve any conflict with PDRs as part of the SDF level reporting. However if this is not acceptable the supporting company can live with the removal of the SDF/Application ID level granularity of energy consumption exposure in rel-19 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of the SDF/Applciation ID level exposure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specified solution does not support the predefined PDRs/Applcation IDs existence in the deployment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.28.2	Neif_EventExposure service

##### 5.2.28.2.1	General

Service description: The service provides the energy consumption information to the consumer NFs. There are several operations for this service:

-	Notifying the energy consumption information to other NFs.

-	Allow consumer NFs to subscribe and unsubscribe for energy consumption information.

The following events can be subscribed by a NF consumer for Target of Event Reporting (Event ID is defined in clause 4.15.1):

-	Energy Consumption: the event notification may contain energy consumption information for the required granularity as described in clause 5.51 of TS 23.501 [2]

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for EIF exposure.

Table 5.2.28.2.1-1: Example of Event Filters for EIF exposure events

| Event ID for EIF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Energy Consumption | <Parameter Type = UE level, Value = SUPI/GPSI> <Parameter Type = S-NSSAI per UE level, Value = SUPI/GPSI and S-NSSAI > <Parameter Type = PDU session level, Value = SUPI/GPSI and DNN/S-NSSAI > |

The following service operations are defined for the Neif_EventExposure service:

-	Neif_EventExposure_Subscribe.

-	Neif_EventExposure_Unsubscribe.

-	Neif_EventExposure_Notify.

End of changes