3GPP TSG-WG SA2 Meeting #171	S2-2508658

Wuhan, China, October August 13 - 17, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	Discussion on Unsupported Command

Document for:	Agreement

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This Discussion Paper discusses the unsupported Command handling in Ambient IoT in Rel-19.

1. Introduction

In clause 5.2.2.2 of TS 23.369, the Read and Write commands are optional for AIoT devices: “It is optional for an AIoT Device to support of Read and Write Commands”.

[Observation-1] An AIoT Device may not support Read and Write Commands.

Also, in clause 5.2.2.2 of TS 23.369, AIoT devices is required to respond when receiving a Read Request or Write Request command which is not supported by the AIoT device: “If an AIoT Device does not support Read Request or Write Request commands, or the parameters in the commands are invalid the AIoT Device shall reject the command.”

CT1 is still discussing how to address the requirement above in stage-3, i.e. whether the NAS message used for rejection should be a generic NAS message (e.g., AIOT STATUS or GENERIC REJECT) or a specific NAS messages (e.g., READ/WRITE COMMAND REJECT).

[Observation-2] An AIoT Device responds the failure in a NAS message to the AIOTF, if it receives an unsupported command.

It hasn’t been clarified how the AIOTF handles such failure in Command Procedure.

[Proposal-1] It is proposed to clarify in the command procedure:

- If the NAS Command Request is not supported by the AIoT device, the AIoT device sends a NAS message indicating the AIoT device does not support the command.

- After receiving the failure in the NAS message, the AIOTF reports the failure to the AF.

After receiving such failure from the AIOTF, the AF understands the AIoT device does not support such command and may updates its own AIoT device profile database. In future, it can avoid sending the same command to this AIoT device again.

However, other AFs are not aware of such situation. They may send the same command to such incapable AIoT device. It is also possible for the same AF to send the command request again if such incapability information hasn’t been persistent into the AIoT device profile database in the AF side.

If the AIOTF receives the same command which the AIoT device does not support, it does not make sense to further send the command request to the AIoT device. Same failure responses from the AIoT device can be expected. The repetition of such failure procedure wastes not only the compute resources of the involved NFs, but also the radio resource of the AIoT system. Furthermore, it wastes the energy of the AIoT device. Such waste may not only be limited to this specific AIoT device. It may affect other AIoT devices in the area, considering the nature of AIoT communication.

The AIOTF can store such unsupported commands into the AIoT device profile data. After receiving the command request, the AIOTF can determine whether the AIoT device can handle the command. If not, the AIOTF can inform the AF directly without further continuing the procedure, to avoid the waste of resources and energy, and to improve the efficiency of the AIoT system.

[Proposal-2] After receiving the NAS failure message, the AIOTF stores such unsupported commands in AIoT Device Profile data in the ADM.

[Proposal-3] For the command request targeting to the individual device, if the AIoT Device Profile data indicates that the AIoT device does not support the command, the AIOTF can respond to the AF directly, without further triggering inventory + command procedure.

[Proposal-4] For the command request targeting to a group of devices, after the AIOTF receives the Inventory Report from the AIoT device, if the AIoT Device Profile data indicates that the AIoT device does not support the command, the AIOTF can inform the AF directly, without further continuing sending command request to this AIoT device.

Additionally, if the AIoT device informs the AIOTF about AIoT device command capabilities, the AIOTF can determine wheter other command types (other than the command type of the sent command request) are not supported by the AIoT device. For example, the AIoT device which does not support read and write would indicate it is incapable of handling read and write, when it receives a read command request. Based on such response, the AIOTF can further avoid sending write command towards this AIoT device. It can further reduce the possible failure of the command procedure to improve efficiency and reduce waste.

[Proposal-5] The [Proposal-2] can be updated to: The AIoT device indicates its device command capabilities (the support of read/write) in the NAS failure message. The AIOTF determines unsupported commands and updates the AIoT Device Profile data in the ADM.

2. Proposal

[Observation-1] An AIoT Device may not support Read and Write Commands.

[Observation-2] An AIoT Device responds the failure in a NAS message to the AIOTF, if it receives an unsupported command.

[Proposal-1] It is proposed to clarify in the command procedure:

- If the NAS Command Request is not supported by the AIoT device, the AIoT device sends a NAS message indicating the AIoT device does not support the command.

- After receiving the failure in the NAS message, the AIOTF reports the failure to the AF.

[Proposal-2] After receiving the NAS failure message, the AIOTF stores such unsupported commands in AIoT Device Profile data in the ADM.

[Proposal-3] For the command request targeting to the individual device, if the AIoT Device Profile data indicates that the AIoT device does not support the command, the AIOTF can respond to the AF directly, without further triggering inventory + command procedure.

[Proposal-4] For the command request targeting to a group of devices, after the AIOTF receives the Inventory Report from the AIoT device, if the AIoT Device Profile data indicates that the AIoT device does not support the command, the AIOTF can inform the AF directly, without further continuing sending command request to this AIoT device.

[Proposal-5] The [Proposal-2] can be updated to: The AIoT device indicates its device command capabilities (the support of read/write) in the NAS failure message. The AIOTF determines unsupported commands and updates the AIoT Device Profile data in the ADM.

It is proposed to agree on TS 23.369 CR#0092 (S2-2508659).