3GPP TSG-WG SA2 Meeting #171	S2-2508699

13th – 17th October 2025, Wuhan, China	  (is revision of XXXX)

Source:	Lenovo

Title:	KI#1 New Solution: AF requesting specific UE reader for AIoT service operation

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes the solution on how to support the AF requesting specific UE(s) to perform the AIoT service operations.

1	Introduction

This paper proposes the solution to address the Key Issue 1 for the Release 20 study, which is depicted below:

## Key Issue #1: support AIoT services under the RRC-based option for UE Reader connectivity

NOTE 1:	Only CM-CONNECTED with RRC_CONNECTED UE Reader is in the scope.

Only the following aspects will be included:

-	How to perform and revoke authorization of UE Readers.

NOTE 2:	Rel-19 interim conclusion in TR 23.700-13 [7] should be used as basis for UE Reader authorization.

-	Potential impact from how the NG-RAN node and UE reader selection can be performed.

NOTE 3:	The aspect of NG-RAN node and UE reader selection requires coordination with RAN WG3.

-	Whether and how the AIOTF can provide the information of the UE readers to the NG-RAN node to assist NG-RAN for UE reader selection.

NOTE 4:	Rel-19 interim conclusion in TR 23.700-13 [7] should be used as basis for this key issue. No Rel-19 AIoT Device impact from this key issue is expected.

NOTE 5:	Conclusions of this KI will be coordinated with RAN WGs.

More specifically, the solution mainly focuses on the use case when the 3rd party AF wants to use specific UE reader (s) for the AIoT service operation and includes the UE reader information (UE ID) in the service operation request towards the core network.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-30.

# * * * * First change * * * *

## 6.X	Solution #X: Support of AF requesting specific UE reader for AIoT service operation

### 6.X.0	High-level Solution Principles

The solution is based on the following general principles:

- AIOTF authorizes the AIoT service request from AF that further includes authorizing the use of specific UE reader (s) for AIoT service operation.

- AIOTF obtains the UE reader related information, that includes the UE reader ID, the serving NG-RAN and serving AMF information of the UE, either via the information received from the serving AMF after UE reader registration procedure and UE reader handover procedure, or via AIOTF actively retrieving the serving AMF information (from UDM) and serving NG-RAN information and UE reader ID from serving AMF. The UE reader related information will be used by the AIOTF to forward the AIoT service operation towards the UE reader correctly.

- The NGAP UE ID is used as the UE reader ID and aligned to both AIOTF and NG-RAN with UE reader authorization indication. AMF sends the NGAP UE ID as the UE reader ID, to the AIOTF (which is associated with SUPI), and it is used between AIOTF and NG-RAN for identifying the UE reader that is being selected/used.

The solution addresses the KI#1, and the use case when AF requesting specific UE reader(s) to perform AIoT service operation.

### 6.X.1	Description

Figure 6.x.1 RRC protocol stack for Topology 2 (Indirect Connectivity)

The protocol stack for the RRC scheme of the topology 2 indirect connectivity, is proposed and shown above in the Figure 6.x.1. Similar to the design of T1 indirect connectivity as described in clause 4.6.2.3 of TS 23.369, for T2 indirect connectivity, we are proposing that the NG-RAN and AIOTF also communicate with each other via the NGAP AIOT information/container. It is assumed that AMF transparently relays the NGAP AIOT information between the NG-RAN and the AIOTF and it is included in the NGAP messages and the AIOT3 messages.

Proposal 1: For Topology 2, the AIOTF and NG-RAN communicate with each other via the NGAP AIOT information/container, and it is transparent to the AMF.

For the UE reader selection, this paper mainly focuses on the scenario when AF is directly requesting specific UE reader (s) for the AIoT service operation by including UE ID (e.g., GPSI). We have identified 2 basic gaps that need to be further addressed, which are illustrated as below:

Gap 1: The authorization of the AF request

For the Release 19 study where NG-RAN is serving as the AIoT reader, the AF will not indicate or request a specific RAN reader to perform the AIoT service operation (e.g., inventory, command), and the authorization for the AF request is based on the AF authorization data stored in the ADM/UDR, which includes the AF ID, allowed area, allowed service operation and allowed target AIoT Devices.

However, for T2, when it comes to the case that a 3rd party AF specifically requests UE reader(s) for an AIoT service operation, the core network (i.e., AIOTF) needs to firstly check whether this AF is authorized to use this UE reader(s) to perform the AIoT service operation. Here, the UE reader can have 2 different options, e.g., the UE reader belongs to/is owned by the operator, or the UE reader belongs to the 3rd party (e.g., Lenovo). For both options, it is necessary for the AIOTF to firstly check whether this AF can ask to use this UE reader before assigning the task, and if not authorized, the AF request shall be rejected, or the AIOTF may recommend other available UE readers to the 3rd party AF.

Proposal 2: The AF authorization needs to be enhanced with allowed UE reader information.

Gap 2: The UE reader ID alignment and information transfer to NG-RAN

As shown in the protocol stack, the NGAP AIOT information between the AIOTF and NG-RAN is transparent to the AMF. Under this assumption, which UE ID should AIOTF use to communicate with the NG-RAN will become a problem. For example, when AF requests a specific UE reader in the form of GPSI, after AIOTF or NEF resolves it into SUPI (via UDM), AIOTF cannot directly use SUPI as the UE reader ID and send it to the NG-RAN since AMF in the between cannot read and let alone translate (e.g., translate SUPI into the ID that NG-RAN knows), and NG-RAN certainly cannot understand the SUPI due to security concern.

On the other hand, if the UE ID is outside the AIOT NGAP information and readable to the AMF, the relay AMF shown in the Figure 6.x.1 between the AIOTF and NG-RAN may not always be the AMF that currently serves the UE (i.e., serving AMF). Following the design in the T1 as described in clause 5.3.4 of TS 23.369, the AMF between the NG-RAN and AIOTF is selected by the AIOTF, based on the local configuration that corresponds to the NG-RAN node(s). Under this case, the middle/relay AMF may not be able to translate the UE ID (SUPI) into the ID that RAN can understand (e.g., NGAP UE ID, 5G-TMSI), and it will also cause the misalignment of the UE ID between NG-RAN and AIOTF.

To solve this problem, the UE reader ID needs to be firstly aligned at both the NG-RAN and AIOTF, understood and used by both sides to exchange the messages related to UE reader. What’s more, in order to forward the message towards the UE from AIOTF, the current serving NG-RAN of the UE reader, needs to be firstly discovered, especially considering the UE mobility. Therefore, it is proposed with the following 2 options:

Option 1: During UE reader registration procedure, the AMF actively sends the authorized UE reader information to the AIOTF, that contains the UE reader ID, the serving NG-RAN information (e.g., NG-RAN ID) and reader allowed/authorization indication.

It is proposed that the NGAP UE ID is reused as the UE reader ID and sent to the AIOTF, which is associated with SUPI, and stored at AIOTF. The NGAP UE ID has already been supported and transmitted on the NGAP interface between AMF and NG-RAN, and on the XN interface during the HO procedure, and it can easily solve the UE reader mobility issue. Compared to the new allocated/generated UE reader ID by AMF or AIOTF, the reuse of NGAP UE ID will have less standard impact. Moreover, the relay AMF between the AIOTF and NG-RAN does not need to understand the NGAP UE ID since it is included within the NGAP AIOT information and is transparent to the relay AMF.

NOTE: The design of the UE reader ID is not limited to the AF requesting specific UE reader (s) scenario.

Based on this, later when a UE reader is determined (either by AIOTF, or indicated by AF), instead of using SUPI, the AIOTF will check the stored information for UE reader, use the NGAP UE ID as the UE reader ID and send it to the serving NG-RAN of the UE, together with the service request.

Option 2: AIOTF obtains the serving AMF of the UE from UDM using UE ID and sends the UE reader related information retrieval request to the serving AMF, when a UE reader is determined (either by AIOTF, or indicated by AF). The serving AMF will send UE reader ID (NGAP UE ID) associated with SUPI, the serving NG-RAN information (e.g., NG-RAN ID) to the AIOTF.

Based on the options listed above, the AIOTF can send the AIOT service request to the UE via the serving NG-RAN, that contains NGAP AIoT information (including UE reader ID (NGAP UE ID)), NG-RAN ID and AIOTF ID.

Proposal 3: NGAP UE ID is reused as UE reader ID and aligned to both AIOTF and NG-RAN by AMF.

Proposal 4: AIOTF receives the UE reader related information that contains the serving NG-RAN information, and UE reader ID, from the serving AMF of the UE.

### 6.X.2	Procedures

#### 6.X.2.1	AF requesting specific UE reader (s) for AIOTF service operation

Figure 6.X.2.1. Procedure of AF requesting specific UE reader (s)

Step 0: UE performs the registration procedure towards the core network and is authorized as AIoT reader by the AMF. It is assumed that the UDM is enhanced with the UE reader allowed/authorization indication, and validity information for the UE reader authorization.

During the UE reader registration and authorization procedure, the AIOTF will receive the UE reader related information from the AMF, that includes the UE reader ID (NGAP UE ID) associated with SUPI, serving NG-RAN information (e.g., NG-RAN ID), the UE reader location and serving area, and UE reader allowed indication (assume AMF conducts the reader authorization). AIOTF can store the UE reader related information locally.

What’s more, during HO procedure, the AMF will always update the UE reader related information to the AIOTF, and AIOTF can subscribe to the AMF about UE reader related information update.

Step 1: The AF sends the AIOT service request towards the core network (to the AIOTF via NEF, or directly). For inventory request, the procedure in clause 6.2.2 of TS 23.369 can be reused, while for command request, the procedure in clause 6.2.3 of TS 23.369 can be reused. Here, the AF will further include the UE ID (s) (e.g., GPSI) as the requested reader to the core network.

Step 2: The AIOTF will check the AF authorization data for the AIoT service request, by using AF ID as the key, towards the ADM. It is assumed that the AF authorization data is further enhanced with the allowed UE reader information (e.g., UE ID).

Step 3: AIOTF sends the AIoT service response message to the AF via NEF (accept or reject, [cause code] and recommended UE reader information). The recommended UE reader information is determined by the AIOTF based on, e.g., UE location, the target area sent by the AF, or the allowed UE reader information that can be requested by the AF.

Step 4: If the AF request is authorized, the AIOTF will translate the UE ID received in the step 1 from GPSI into SUPI, by checking the UDM. Moreover, the AIOTF may further check the serving AMF information of UE in the UDM, then step 6 can be skipped.

Step 5: AIOTF checks locally to see whether the corresponding UE reader related information is available (e.g., provided by AMF and stored locally after UE registration procedure and UE reader handover procedure), and if not, goes to step 6. If the UE reader related information is available, then goes to step 9.

Step 6: AIOTF uses the SUPI as the key towards the UDM to retrieve the serving AMF information (e.g., AMF ID) for the UE.

Step 7: AIOTF sends the UE reader related information retrieval request to the serving AMF based on the AMF ID received in step 6, that includes the SUPI as UE ID for retrieval.

Step 8: AMF sends the UE reader related information to the AIOTF, that includes the UE reader ID (NGAP UE ID), serving NG-RAN information (e.g., NG-RAN ID), optionally the UE reader location, UE reader serving area, and UE reader allowed indication (assume AMF conducts the reader authorization).

Step 9: Based on the UE reader related information, the AIOTF sends the AIoT service request to the NG-RAN via AMF, that includes the NGAP AIoT information, NG-RAN ID, AIOTF ID. The UE reader ID (NGAP UE ID) is included in the NGAP AIoT information.

Step 10: Based on the received AIOTF service request, NG-RAN identifies the UE that is used as the reader, allocates the radio resources, and sends the AIoT services request to the UE, which is under the remit of RAN2.

Step 11: UE reader interacts with the target AIoT devices and sends the responses to the NG-RAN.

Step 12: NG-RAN sends the AIoT service response messages to the AIOTF via AMF, that contains the UE reader ID (NGAP UE ID), correlation ID, NG-RAN ID, AIOTF ID and AIOT NAS messages.

Step 13: AIOTF sends the service response to the AF via NEF, that further contains the UE ID (GPSI) and transaction ID.

6.X.3	Impacts on Services, Entities and Interfaces

AMF:

- Send the UE reader related information to the AIOTF, that includes the serving NG-RAN information, UE reader ID (NGAP UE ID) and UE reader allowed/authorization indication

AIOTF:

- Authorize the AF request that further includes the request for specific UE reader (s) to perform the AIoT service operation.

- Determine the serving AMF information and serving NG-RAN information of UE reader

- Store the mapping of SUPI/GPSI and UE reader ID (NGAP UE ID), and communicate with NG-RAN using the NGAP UE ID as UE reader ID

# * * * * End of changes * * * *