3GPP TSG-SA WG2#171	S2-2508788

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	KI#2: Evaluation and interim agreements

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: Evaluation and interim agreements on KI#2: IMS enhancement for GEO NB-IoT NTN access.

# 1.	Discussion

## 1.1	Roaming scenarios for voice over IMS with local breakout

Although the roaming architecture for voice over IMS with local breakout is specified in clause 4.2.3 of TS 23.228, current VoLTE and VoNR deployments predominantly utilize S8HR/N9HR based IMS roaming architecture.

In the typical case for the deployment of IMS voice over NB-IoT NTN, the VPLMN will be a Satellite Network Operator (SNO) and the HPLMN a terrestrial operator (TNO).

If the GEO NB-IoT NTN access is provided by the PLMN which is different from the UE’s HPLMN, roaming architecture is required to provide voice service over GEO NB-IoT NTN access to the UE. In this scenario, the deployment of interworking capabilities within VPLMN can minimize the impact to IMS network in the HPLMN.

A signaling gateway can be deployed for the interworking with IMS network in the HPLMN via SIP protocol as specified in Mw/Mx/ISC interfaces. The signaling gateway terminates the UNI signaling with UE (e.g., I1 or text-based protocol) and performs as interworking user agent to the IMS in HPLMN.

Based on the roaming agreement, the transcoding between GEO NB-IoT NTN specific voice codec to the existing voice codec (e.g., AMR) may be performed in the VPLMN or HPLMN.

The example of roaming architecture for GEO NB-IoT NTN access is illustrated in Figure 1.1-1.

Figure 1.1-1: IMS local breakout in VPLMN with GEO NB-IoT NTN access

Proposal 1: When GEO NB-IoT NTN access is provided by a PLMN different than the UE’s Home PLMN (HPLMN), a roaming architecture is required to enable voice services over this access. To minimize the impact on the IMS network in the HPLMN, it is recommended to allow the possibility that the Visited PLMN (VPLMN) deploy appropriate interworking capabilities but not mandate them.

## 1.2	IPsec for SIP signaling between the UE and the P-CSCF

According to TS 33.203, IMS mandates the use of IPsec to secure SIP signaling between the UE and the P-CSCF. This ensures:

- Confidentiality (via encryption),

- Integrity (via authentication),

- Replay protection.

The IPsec Header Overhead in IMS signalling includes:

- ESP Header: ~20 bytes,

- ESP Trailer + Padding: Variable (depends on encryption block size),

- ESP Authentication Data: ~12 bytes (e.g., HMAC-SHA1-96).

In summary, the typical total IPsec overhead per packet is about 32–40 bytes.

Proposal 2: The use of IPsec to secure SIP signaling between the UE and the P-CSCF is mandatory in GEO NB-IoT NTN scenarios, in accordance with clause 5.1.3 of TS 33.203 [Y]. An IPsec overhead of approximately 32–40 bytes per packet—due to ESP headers, trailers, and authentication data—should be factored into the estimation of call setup time and signaling latency.

## 1.3	Performance evaluation

In the scenarios of GEO NB-IoT NTN access, the SIP messages exchanged between UE and IMS entities can be minimized by reducing the unnecessary optional SIP headers as specified in IETF RFC 3261 [18]. In addition, SigComp can be utilized by P-CSCF and UE to further reduce the message size.

Table 1.3-1 provides the message size of the necessary SIP messages exchanged for IMS call setup time estimation based on Solution #17.

| SIP Message | Message Size | Message Size After Compression |
| --- | --- | --- |
| INVITE | 491B | 285B (SigComp: Doubango + zlib1.2.3) |
| 100 Trying | 238B | 232B (LZ77) |
| 180 Ringing | 280B | 256B (LZ77) |
| Total | 1009B | 773B |

Table 1.3-1: SIP message size

Transport Layer Overhead and Estimated Call Setup Time (ECST):

The transport layer protocol overhead for IMS signaling—typically comprising UDP/IP/PDCP/RLC/MAC—is estimated to be 52 bytes, as specified in Annex A. When accounting for IPsec overhead (up to 40 bytes), the total estimated transport layer overhead becomes 92 bytes per SIP message.

The combined size of the three key SIP signaling messages, along with IMS-related overhead, is calculated to be 1,049 bytes—comprising 773 bytes for the initial message and 92 bytes for each of the remaining three messages—equivalent to approximately 8392 bits.

Assuming a minimum PHY data rate of 2 kbps for GEO-based voice communication, and applying the formula for Estimated Call Setup Time (ECST) from satellite to terrestrial (as provided in Annex A), the calculation is as follows:

ECST = 3[s] + 8392/2000[s] + 0.285 x (3 x 1 + 2)[s] + 2.5[s] + 2.5[s] = 13.6[s].

Thus, the estimated call setup time is 13.6 seconds.

Proposal 3: By minimizing the use of optional SIP headers as specified in IETF RFC 3261 [18], and leveraging the existing SIP compression (SigComp) capability between the UE and P-CSCF, the SA1 requirement for Call Setup Time (CST) under 30 seconds can be met with sufficient margin.

Further optimization details:

- B2BUA is introduced to support simplified IMS signaling for UEs connected via GEO satellite, while maintaining compatibility with legacy IMS network entities. This enables seamless signaling exchange and protocol adaptation between the satellite-optimized UE and traditional IMS infrastructure.

- QoS Precondition Handling: the SIP precondition mechanism for QoS assurance is not applied. Instead, QoS authorization and resource reservation are either preconfigured or triggered upon receipt of a SIP INVITE to/from the UE. This approach reduces signaling overhead and accelerates call setup.

- Proactive Media Transcoding: to avoid additional SIP interactions for media negotiation, proactive transcoding is applied for low data rate audio codecs, particularly for UEs connected via GEO satellites. This ensures media compatibility without requiring extra signaling.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

### 7.1.X	Agreed principles for KI#2 of IMS enhancement for GEO NB-IoT NTN access

To supports KI#2 of IMS enhancement for GEO NB-IoT NTN access, the following principles are concluded:

Principle 1: When GEO NB-IoT NTN access is provided by a PLMN different from the UE’s Home PLMN (HPLMN), a roaming architecture is required to enable voice services over this access. To minimize the impact on the IMS network in the HPLMN, it is recommended to allow the possibility that the Visited PLMN (VPLMN) deploys appropriate interworking capabilities but not mandate them.

Principle 2: The use of IPsec to secure SIP signaling between the UE and the P-CSCF is mandatory in GEO NB-IoT Non-Terrestrial Network (NTN) scenarios, in accordance with clause 5.1.3 of TS 33.203 [Y]. An IPsec overhead of approximately 32–40 bytes per packet—due to ESP headers, trailers, and authentication data—should be factored into the estimation of call setup time and signaling latency.

Principle 3: By minimizing the use of optional SIP headers as specified in IETF RFC 3261 [18] and by leveraging the existing SIP compression (SigComp) capability between the UE and P-CSCF, the SA1 requirement for Call Setup Time (CST) under 30 seconds can be met with sufficient margin.

Further optimization details:

- B2BUA is introduced to support simplified IMS signaling for UEs connected via GEO satellite, while maintaining compatibility with legacy IMS network entities. This enables seamless signaling exchange and protocol adaptation between the satellite-optimized UE and traditional IMS infrastructure.

- QoS Precondition Handling: the SIP precondition mechanism for QoS assurance is not applied. Instead, QoS authorization and resource reservation are either preconfigured or triggered upon receipt of a SIP INVITE to/from the UE. This approach reduces signaling overhead and accelerates call setup.

- Proactive Media Transcoding: to avoid additional SIP interactions for media negotiation, proactive transcoding is applied for low data rate audio codecs, particularly for UEs connected via GEO satellites. This ensures media compatibility without requiring extra signaling.

# >>>>NEXT CHANGE<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[21]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[22]	3GPP TS 23.003: "Numbering, addressing and identification".

[23]	3GPP TR 22.887: "Study on satellite access - Phase 4".

[24]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[Y]	3GPP TS 33.203: "Access Security for IP-based services".

# >>>>END OF CHANGES<<<<