3GPP TSG-WG2 Meeting #171	S2-2508854

October 13 – 17, 2025, Wuhan, China		(revision of S2-250xxxx)

Source:	Ericsson

Title:	Interim agreement for KI#1

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides agreed principles and topics related to KI#1 for further consideration

# 1. Discussion

This document proposes interim agreements for KI#1. Solutions #1,3-7,19, 27, 28,31, 34, 38 are proposed for KI#1. 38 is rather authorization paper instead of covering Architectural related considerations.

| # | RAN – Sensing function interface | Distribution of Sensing Functions | NEF or new NF as Sensing Exposure function |
| --- | --- | --- | --- |
| 1 | Direct | One NF (Snf) | Gateway Function |
| 3 | Direct | SCF and SPF split | NEF |
| 4 | Direct | One NF (SemF) | NEF |
| 5 | Via AMF | One NF (SF) | NEF |
| 6 | - | One Sensing NF | Gateway Sensing NF/NEF |
| 7 | Via AMF | One NF (SSMF) | NEF |
| 19 | - | SMF and SPF | Exposure Function |
| 27 | - | SCF + SPF | NEF |
| 28 | Direct | One NF (SnCF) | NEF |
| 31 | - | GSMF + SSMF | NEF |
| 34 | Direct | SF vs SCF +SPF | Gateway Sensing Function |

Observation 1: Most of the solutions propose to have direct connection between RAN and the new Sensing Function. By involving AMF communication services are mixed with sensing service and this increase complexity, and additionally involvement of AMF unnecessarily increases latency and give extra load to AMF to proxy information.

Observation 2:  Approximately half of the solutions propose to define one NF for sensing function. Other solutions propose functional splits between controlling and processing domain which increase the complexity by requiring specify the interface between those.

Observation 3:  Approximately two thirds of solutions propose to extend NEF functionality with sensing exposer service. Other solutions propose a new Gateway function for sensing.

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.1	Agreed Principles for KI#1 on System Architecture to Support Sensing

A new network function is introduced to the 5GC reference architecture specified in clause 4.2.3 of TS 23.501 [x]. This new network function is called Sensing Core Function (SnCF), and the SnCF contains Sensing Controlling and Sensing Processing logic and functionalities. The Sensing Controlling Logic is responsible for handling Sensing Requests, and the Sensing Processing Logic is responsible for receiving the 3GPP Sensing Data and for performing the data processing to provide Sensing Results.

SnCF has a direct P2P interface with NG-RAN nodes. SnCF uses this direct interface to send request to NG-RAN. NG-RAN provides sensing data to SnCF also via direct P2P interface.

NEF is the exposure node for sensing service. NRF is used to discover SnCF by NEF or by other 5GC NFs. Figure 7.1.1-1 illustrates the reference architecture used for sensing.

Figure 7.1.1-1: Reference architecture for sensing.

>>>> End of Changes <<<<