3GPP TSG SA WG2 Meeting #171	S2-2508870

# 13-17 October, 2025, Wuhan , China 		    (Revision of S2-2506997, 5206, 3758, 1738)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1588 | rev | 4 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on HSS Nhss_ImsEventExposure service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The principle of IMS DC event exposure is that HSS receives the service invocation of Nhss_ImsEE_Subscribe from AF and subscribe to the NF serving the subscriber on behalf of the AF. When the subscriber has not yet registered on the IMS network, HSS holds the request and wait until the subscriber register. In this case, the result of the subscription can not be returned in the response of the Nhss_ImsEE_Subscribe. However this result is needed for the AF to know whether the subscription is finally successful. It is proposed to update Nhss_ImsEventExposure service with a new service operation to report the result of the subscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new service operation Nhss_ImsEE_Notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AF can not get the result of the event subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.1.5.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change****

### AA.2.1.5	Nhss_ImsEventExposure (ImsEE) service

#### AA.2.1.5.1	Nhss_ImsEE_Subscribe service operation

Service operation name: Nhss_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific subscriber related IMS event.

Inputs, Required: IMS Public Subscriber identity, Event-ID(s) representing the event of interest, Notification Target address, Event Reporting Information as defined in Table 4.15.1-1 of TS 23.502 [94].

Event-ID represents one or multiple events as defined in clauses AD.2.4, and AD.2.5.

Inputs, Optional: Notification Correlation ID, Expiry time, Event Filters, Subscription Correlation ID (in the case of modification of the event subscription).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1 of TS 23.502 [94]).

#### AA.2.1.5.2	Nhss_ImsEE_Unsubscribe service operation

Service operation name: Nhss_ImsEE_Unsubscribe

Description: The NF consumer unsubscribes for notifications to requested IMS event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

AA.2.1.5.3	Nhss_ImsEE_Notify service operation

Service operation name: Nhss_ImsEE_Notify

Description: This service operation is used by the HSS to report the result of subscription with a subscribed Event.

Inputs, Required: Result of the event subscription.

The Result of the event subscription includes single or a list of the final result(s) of the subscription of the events, each of which includes:

-	Event ID: identifies the event being subscribed.

-	Subscription result: indication of whether the subscription is successful.

Outputs, Required: Operation execution result indication.

**** End of Changes ****