3GPP TSG-WG SA2 Meeting #171	S2-2509011

Wuhan, CN, 13th Oct – 17th Oct, 2025

Title:	[DRAFT] ReplyLS on Clarification on 5G VN group

Response to:	LS (S2-2508180/C4-253576) on Clarification on 5G VN group from CT4

Release:	Rel-19

Work Item:	FS_PAIDC_UPF

Source:	[Huawei to be] SA2

To:	CT4

Cc:	CT3

Contact Person:

Name:	Qianghua Zhu

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks for the LS on 5G VN group clarification.

Q1: Can a UE, that is not member (temporary/permanently) of any 5G VN group, have an SM subscription (in DNN configuration) with an S-NSSAI/DNN that is associated with a 5G VN group?

SA2 Answer: Yes. A S-NSSAI/DNN can be used for 5G VN group communication for some UEs within the network while the same S-NSSAI/DNN can be used for non 5G VN group communication for different UEs.

If the network supports a 5G VN group binding with a specific DNN/S-NSSAI, then the UDM/UDR has the group subscription data for this 5G VN group, such as the Group Identifier translation or Group Data defined in Table 5.2.3.3.1-2 of TS23.502.

For a UE that is a 5G VN group member, the 5G VN group ID is present in the “Internal Group ID-list” of UE’s AM subscription and SM subscription, the 5G VN group associated DNN/S-NSSAI is present in the “Subscribed S-NSSAIs” or “Default S-NSSAIs” and “Subscribed DNN list” UE’s AM subscription and SM subscription.

For a UE that is not 5G VN group member, the 5G VN group ID is not present in the “Internal Group ID-list” of UE’s AM subscription and SM subscription, but this UE can still subscribe to the 5G VN group associated DNN/S-NSSAI, namely, the 5G VN group associated DNN/S-NSSAI is present in the “Subscribed S-NSSAIs” or “Default S-NSSAIs” and “Subscribed DNN list” UE’s AM subscription and SM subscription. For this UE’s PDU Session, the 5G VN group communication is not applied.

Q2: Assuming an enterprise deployment with many UEs subscribed to an enterprise specific S-NSSAI/DNN, is it allowed to provision only a subset of these UEs into one 5G VN group with the same aforementioned S-NSSAI/DNN?

E.g., 100 UEs are subscribed to the enterprise S-NSSAI/DNN and only 10 UEs are required to be provisioned to the S-NSSAI/DNN associated 5G VN group. (still keep 1:1 mapping between the S-NSSAI/DNN and 5G VN group that one and only one 5G VN group is associated with this specific S-NSSAI/DNN)

SA2 Answer: Yes. See answers for Q1.

Q3: If the answer to Q1 and Q2 is no, before the subset of UEs (of Q2) is added to a 5G VN group, this subset of UEs have no S-NSSAI/DNN subscription for the 5G VN group, does it imply that this subset of UEs needs to be re-configured with the S-NSSAI/DNN associated with 5G VN group when they are added to the 5G VN group by the AF/NEF invoking Nudm_pp service(such that these UEs start using the S-NSSAI/DNN associated with the 5G VN Group when they establish PDU sessions)?

Q4: If the answer to Q3 is yes, how are the UEs re-configured?

SA2 Answer: All the UEs (of Q2) are subscribed to the specific S-NSSAI/DNN, so for each UE, the specific DNN/S-NSSAI is present in the “Subscribed S-NSSAIs” or “Default S-NSSAIs” and “Subscribed DNN list” UE’s AM subscription and SM subscription.

For the subset of UEs that is going to be added to the 5G VN group, 5G VN group management procedure is used, e.g., Nudm_pp service, then the 5G VN group ID is added into the “Internal Group ID-list” of UE’s AM subscription and SM subscription, and the Group Identifier translation will be updated to include the SUPI of those UEs.

2. Actions:

To CT4 group.

ACTION: 	SA2 asks CT4 group to take the above answers into account during completion of the relevant CT4 work within Rel-19 timeframe.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US

TSG-SA2 Meeting #173		09th Feb – 13th Feb, 2026		India, IN