3GPP TSG-WG SA2 Meeting #171	S2-2509033

# Wuhan, CN, 13th Oct – 17th Oct, 2025		(revision of S2-2507467)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6410 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of UE serving satellite identifier report for UE-Satellite-UE communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the current call setup procedure for UE-satellite-UE communication in IMS described in TS 23.228, after receiving an INVITE message from the originating side P-CSCF, the terminating side P-CSCF requests access network information from 5GC to obtain the UE's serving satellite identifier. However, the UE may be in IDLE state, which means the IMS PDU session may be deactivated. In this case, the subscription request for access network information from the PCF triggers the SMF to activate the IMS PDU session to obtain the UE's serving satellite identifier from the AMF. This aspect was not currently covered by existing specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that after the P-CSCF subscribes to the access network information report for the IMS PDU session from the SMF via PCF, if the IMS PDU Session is deactivated, the request triggers the SMF to activate the IMS PDU Session to obtain the serving satellite identifier of UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | P-CSCF cannot get a correct satellite identifier for UE-satellite-UE communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.14.2	Setup of UE-Satellite-UE communication with IMS user plane components on-board satellites

The AMF reports the identifier of the satellite serving the UE to the SMF as part of the PDU Session establishment, update and Service Request procedures as described in clauses 4.2.3, 4.3.2 and 4.3.3 of TS 23.502 [3].

NOTE 1:	The AMF can determine that the UE is accessing gNB onboard a LEO or MEO satellite and can also determine the satellite identifier based on local configuration, e.g. Global RAN Node IDs associated with the LEO or MEO satellite.

Upon receiving a request from the P-CSCF for accessing network information report (e.g. as a result of the P-CSCF processing an IMS call setup), the PCF subscribes to Access Network Information report from the SMF for the IMS PDU Session, with Satellite Identifier Report parameter in the request, as described in clause 6.1.3 of TS 23.503 [45]. The Access Network Information report contains the identifier of the satellite serving the UE as reported by the AMF.

Based on the Satellite Identifier Report parameter in the request from PCF for Access Network Information reporting, if the IMS PDU Session is deactivated, the SMF activates the IMS PDU Session by invoking the Network Triggered Service Request procedure described in clause 4.2.3.3 of TS 23.502 [3]. During the Service Request procedure, the AMF reports the serving satellite identifier of UE to the SMF.

NOTE 2:	How the serving satellite identifier is used by the IMS to determine whether the UE-Satellite-UE communication can be activated is specified in TS 23.228 [15].

# * * * * End of changes * * * *