SA WG2 Meeting #171	S2-2509049

13 – 17 Oct, 2025, Wuhan, China

Source:	Samsung, [Futurewei, InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO]

Title:	[WT#1.2, User Plane] Scope and Key Issue for 6G User Plane

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses areas to study for enhanced user plane for 6G.

**** First Change (All text new, changes w.r.t S2-2508121) ****

# Annex A.X. WT#1.2 User Plane Architecture for 6G Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance/improve the interface towards the UPF, including:

  - Whether and how to enhance CP-UP interaction for better interoperability (among multiple vendors).

  - Whether and how to extend the service-based architecture for the user plane control interface to UPF.

NOTE 1.1: <place holder>

- Whether and how to enhance the procedures for (re-)selection of  the path involving user plane entities including:

  - Whether and how to support flexible? user plane (i.e., UPF (re)selection) for different service requirements considering path performance and/or capability between access network and data network.

  - Whether and how to enhance the user plane handling, for service/session continuity and mobility, between access network and data network (i.e., best path selection considering i) PSA-UPF path metrics or ii) I-UPF (in case present) and PSA-UPF path metrics).

NOTE 2.1: Solutions should consider to minimize the impact on re-establishment (i.e. release and creation) of resources/context in various entities, due to change of UP path.

**** Second Change (All text new) ****

# 5.X. Key Issue #X Enhanced User Plane Architecture for 6G: control interface towards UPF

This key issue is focused towards enhancing/improving the interaction of UPF with control plane entities, and include:

- Studying whether and how to improve multivendor interface between the SMF and the UPF.

- Studying whether and how to enhance the SBI interface towards the UPF, taking 5G Nupf service as the starting point.

# 5.Y. Key Issue #Y Enhanced User Plane Architecture for 6G considering reselection of path involving user plane entities

This key issue is related to enhance/optimize user plane procedures for flexible traffic handling, session/service continuity and managed performance for end-to-end path between UE and the AS. This key issues includes the study on:

- Whether and how to perform reselection of user plane paths with less signaling and/or session context.

- whether and how to enhance session continuity and mobility

- what specific metrics/triggers to consider which decide the (re)selection of a user plane path

- whether and how to measure/receive the specific metrics/triggers (as identified) which are considered for reselection of user plane path

NOTE Y: Solutions should minimize the impact on re-establishment (i.e. release and creation) of resources/context in various entities (such as UE, RAN), triggered due to change of UP path.

**** End of Changes ****