3GPP TSG-WG SA2 Meeting #171	S2-2509160

13 – 17 Oct, 2025, Wuhan, China	(Revision of S2-2508113)

Source:	Ofinno

Title:	[WT#1.2, Slicing] Scope and key issue on Network Slicing in 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes scope and key issue to Work task 1.2 of the study on Network Slicing for 6G.

# 0. Background

In SA2#170, SA2 discussed proposals on updating WTs for FS_6G_ARC. S2-2508113 related to Network Slicing aspects was discussed as baseline document and finally was postponed.

Following are text proposals in S2-2508113

# Annex A.X. WT#1.2 Scope on Network Slicing The scope of the network slicing work task includes: 1. Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider: a. Identify and address any areas of possible improvement and simplification; b. Identify and address improvements of the application traffic mapping to network slice(s). NOTE: It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network. 2. Study impacts, if any, of the 6G network slicing on the interworking and migration.

# 1. Discussion

Network Slicing was introduced in 5G as a key feature enabling operators to create differentiated services and business models, while offering users the possibility of premium QoS. At the SA2 level, this was realized by allowing applications to be mapped to network slices, notably via URSP rules that connected App IDs with S-NSSAIs.

While effective in concept, this direct App ID-slice mapping raised serious user privacy concerns, since it implicitly revealed which applications were running on the user device. To mitigate this, Rel-18 adopted requirements from GSMA and introduced the notion of traffic categories, defined both in standardized ranges and operator-specific ranges (as specified in TS 23.503 Table 6.6.2.1-2 and TS 24.526 Table 5.2.1). This allowed operators to map applications to traffic categories, and traffic categories to slices, thereby reducing the exposure of explicit App IDs.

Despite enhancements introduced in Rel-19, limitations remain in the current approach of mapping application traffic to slices:

- Residual Privacy Risks: Even when traffic categories are used as an abstraction layer, it is still possible to infer which applications are in use. This is because the categorization inevitably reflects operator-specific mapping decisions, leaving user privacy exposed.

- Lack of Consistency Across Operators: Different operators may choose to map the same application into different traffic categories. As a result, harmonization across PLMNs cannot be ensured, and effectively this approach is not much different from assigning slices directly on a per-application basis.

- Scalability Issues: The need to configure mappings at the granularity of individual applications or traffic categories remains, which introduces operational complexity and makes the approach difficult to scale in diverse 6G service environments.

In 6G, network slicing is expected to evolve in two complementary directions. First, building on the experience from 5G, many slicing features specified over several releases turned out to be overly complex or only partially deployed. For 6G, it is therefore anticipated that network slicing will be simplified and optimized, aligning with new service requirements and the overall system architecture.

At the same time, it is essential to preserve one of the original purposes of network slicing: to provide QoS differentiation and guarantees for diverse services and applications. This fundamental capability should remain at the center of slicing design in 6G, even as unnecessary complexity is reduced.

Accordingly, the study on 6G network slicing should address these aspects in an integrated manner: simplification and optimization of slicing features for practicality and deployment readiness, together with renewed mechanisms that enable QoS differentiation in a scalable and privacy-preserving way.

Therefore, it is proposed, in WT#1.2 Slicing, to study the fundamental mechanisms for mapping application traffic to slice in 6G, with the objectives of ensuring scalability, preserving user privacy, and maintaining alignment with other network slicing aspects. The requirements identified from this study are to be forwarded to the WT#1.2 Policy study, so that an appropriate policy framework can be designed accordingly.

# 2. Proposal

This contribution proposes to update the TR 23.801-01 to include the scope of WT#1.2 and the related Key Issue. The text for WT#1.2 Scope on Network Slicing is proposed in the first change of this paper, and it is on top of the latest version of baseline S2-2508113 discussed in SA2#170. The related KI text is proposed in the second change of this paper.

**** First Change relative to S2-2508113****

# Annex A.X. WT#1.2 Scope on Network Slicing

The scope of the network slicing work task includes:

1.	Study the overall design and functionalities of the 6G network slicing, which may include existing 5G and new 6G functionality. The overall network slicing design will also consider:

a.	Identify and address any areas of possible improvement and simplification;

b.		Identify and address improvements of the application traffic mapping to network slice(s).

NOTE 1:	It will be studied which of the 5G functionality and potential new 6G functionality are mandatory to support in the UE and in the network.

NOTE 2:	This study will identify potential requirements impacting the design of the 6G policy framework (e.g., URSP design for 6G). The derived requirements will be provided to the WT#1.2 Policy scope for consideration.

2.	Study impacts, if any, of the 6G network slicing on the interworking and migration.

**** Second Change – all text is new ****

# 5.X. Key Issue for WT#1.2 on Network Slicing

This key issue will study how to design the network slicing feature in 6G. The study will cover, but not be limited to, the following aspects:

- Study overall design of the 6G network slicing feature, including at least:

  - How to enable the mapping of application traffic to network slices in a flexible and scalable manner, while ensuring preservation of user privacy at the UE.

NOTE:	For bullet 1a, the potential requirements impacting the 6G policy framework design (e.g. URSP for 6G) should be provided to the WT#1.2 Policy study.

**** End of Changes ****